SA WG2 Meeting #162	S2-2404347

Changsha, China, April 15 – 19, 2024	(revision of S2-2400xxxx)

Source:	ZTE

Title:	KI #3, Evaluation and Conclusion

Document for:	Approval

Agenda Item:	19.2

Work Item / Release:	FS_NG_RTC_Ph2 / Rel-19

Abstract of the contribution: The paper gives primary evaluation and conclusion for KI #3.

1. Discussion

KI #3 addresses IMS data channel interworking with MTSI UE. The contribution evaluates the solutions for KI #3 and proposes some primary conclusion for this key issue.

2. Proposal

It is proposed to capture the following changes into TS 23.700-77.

* * * * Start of Changes * * * *

7	Overall Evaluation

Editor's note:	This clause will provide evaluation of different solutions.

7.X	KI #3 Evaluation

Solution #8 reuses messaging service to send a hyperlink of the file to the MTSI UE; and the user of MTSI UE experiences with the download file which replaces the content transported by IMS data channel.

Solution #9 applies the scenario in which the content can be transported over both RTP channel and application data channel. In the procedure, DCSF instructs MF to transform DC streams to audio/video streams. The solution needs coordination with SA4

8	Conclusions

Editor's note:	This clause will list conclusions that have been agreed during the course of the study item activities.

8.X	KI #3 Conclusion

For the real-time communication between DCMTSI UE and MTSI UE, it is proposed using the procedures in solution #9 to establish bootstrap/application data channels between two UEs for the interworking purpose. It is suggested to coordinate with SA4 to specify instruction of how to transform data channel media to video media.

For the non-real-time communication between DCMTSI UE and MTSI UE, it needs further discussion on whether solution #8 can be used.

* * * * End of Changes * * * *