# SA WG2 Meeting #162		S2-2404669

15 - 19 April, 2024, Changsha, China	(revision of S2-24xxxxx)

Source:	OPPO

Title:	FS_MASSS KI#X New solution for subscription enhancement for DualSteer

Document for:	Approval

Agenda Item: 	19.13

Work Item / Release:	FS_MASSS /Rel-19

Abstract of the contribution: This paper proposes a new solution for new subscription enhancement for the FS_MASSS TR 23.700-54.

1	Discussion

This paper proposes for subscription enhancement for DualSteer addressing KI#1.1.

This solution has the following assumptions:

-	Each DualSteer Device has two USIMs for 3GPP accesses i.e. MUSIM UE..

-	The two USIMs belongs to the same MNO.

-	The UDM stores the association subscription for DualSteer UE(s).

-	A USIM may be associated with several USIMs, a DualSteer Device can use the USIM and one of the associated USIM for DualSteer Service..

The UE subscription in UDM needs to be enhanced.

2	Proposal

It is proposed to include the following changes in TR 23.700-54 V0.2.0.

* * * * Start of Changes (All text new) * * * *

6.1.X	Solution #X: The UE subscription data for DualSteer in the UDM

6.1.X.1	Description

This solution addresses the Key Issue #1.1: Subscription aspects to support DualSteer.

This solution shows how the UDM identifies and associates the two subscriptions/SUPIs for the DualSteer Device in the UE subscription data.

This solution is based on the following assumptions:

-	Each DualSteer Device has two USIMs for 3GPP accesses i.e. MUSIM UE..

-	The two USIMs belongs to the same MNO.

-	The UDM stores the association subscription for DualSteer UE(s).

-	A USIM may be associated with several USIMs, a DualSteer Device can use the USIM and one of the associated USIM for DualSteer Service.

The following table 6.1.x.1-1 shows an example of “DualSteer Subscription data” in the UDM for a UE which can be used as one UE of the DualSteer Service.

Table 6.1.x.1-1 Example of DualSteer Subscription data

| Subscription data type | Field | Description |
| --- | --- | --- |
| DualSteer Subscription data | SUPI | Key. |
|  | DualSteer service allowed indication | Indicates whether DualSteer service is allowed for the UE. |
|  | Associated SUPI(s) | Corresponding SUPI(s) associated with this SUPI. The associated SUPI can be used with the UE for DualSteer Service. |

The above parameters in the table 6.1.x.1-1are described as follows:

-	SUPI: Identifier for the UE which can be used as the Key to query the DualSteer Subscription data.

-	DualSteer service allowed indication: Indicate whether this UE can be used as one UE of DualSteer Device.

-	Associated SUPI(s): Indicate the associated UE identifier which can be used in DualSteer device with this UE.

NOTE:	The DualSteer Subscription data can be a single Subscription data type in the UDM or a part of Access and Mobility Subscription data and/or Session Management Subscription data.

6.1.X.2	Procedures

The procedure of AMF/SMF retrieving the UE subscription data from the UDM as defined in clause 4.2.2.2 and clause of 4.3.2.2 of TS 23.502[4] can be reused.

6.1.X.3	Impacts on services, entities and interfaces

The solution impacts the following network functions:

UDM:

-	Provision DualSteer Subscription data to identify and verify whether the UE can be used for DualSteer Service.

-	Provide the associated SUPI(s) to identify and verify whether the associated UE can be used for DualSteer Service.

* * * * End of Changes * * * *