3GPP TSG-WG SA WG2 Meeting #162	S2-2404687

Changsha, China 15th –19th April, 2024                         (revision of S2-240xxxx)

Source:	CATT

Title:	Update Sol#26 for AF provisioning S&F operation parameters

Document for:	Approval

Agenda Item: 	19.1

Work Item / Release:	FS_5GSAT_ARCH_Ph3 / Rel-19

Abstract of the contribution: This paper proposes an update for sol#26 to provide the S&F Satellite Specific Parameters from AF.

1	Discussion

This paper proposes an update for sol#26 mainly to resolve the following EN.

Editor's note: Whether and what parameters to expose as Store and Forward Satellite Specific Parameters is FFS.

2	Proposal

It is proposed to include the following changes in TR 23.700-29 V0.4.0.

* * * * Start of Changes * * * *

6.26	Solution #26: Provision of Store and Forward Satellite Specific Parameters

6.26.1	Description

This solution considers the following key issue:

-	Key Issue #2: Support of Store and Forward Satellite operation.

Especially, it focuses on the provisioning procedure needed before performing S&F Satellite operation

In the existing 5GS, the provisioning capability allows an external party to provision the information, such as expected UE behaviour and service specific parameters. The service specific information consists of information to support the specific service in 5G system. Provisioned data can be used by the other NFs.

This solution reuses this provisioning capability for Store and Forward Satellite Specific Service, and defines the specific parameters for 5G system.

The Store and Forward Satellite Specific Parameters (e.g., Maximum S&F data storage quota, Maximum S&F data retention period, Data forward priority, etc.) can be the parameters provided from an AF (of the operator or a trusted 3rd party) by invoking the Nnef_ParameterProvision Service as described in clause 4.15.6.2 of TS 23.502 [3].

NOTE:	It’s assumed AF has a SLA agreement with 3rd party (e.g., Satellite Control Server) in advance to determine the Store and Forward Satellite Specific Parameters.

The parameters/events that exposing to AF as S&F Satellite specific parameters can refer to Solution#38.

To minimize the existing system impacts, EPS can include or update these kinds of parameters as the subscription information, based on SLA with a trusted 3rd party.

Provisioned Store and Forward Satellite Specific Parameters can be used by the other NFs such as SMF, PCF, etc. for example, being forwarded as subscription information, and considered for the policy generation and enforcement.

6.26.2	Procedures

Figure 6.26.2-1 shows the overall procedure for Provision of Store and Forward Satellite Specific Parameters.

Figure 6.26.2-1: Overall Procedure for Provision of Store and Forward Satellite Specific Parameters

1.	The AF provides one or more parameter(s) to be created or updated, or deleted in a Nnef_ParameterProvision_Create or Nnef_ParameterProvision_Update or Nnef_ParameterProvision_Delete Request to the NEF. The payload of the Nnef_ParameterProvision_Update Request may include Store and Forward Satellite Specific Parameters (e.g., Maximum S&F data storage quota, Maximum S&F data retention period, Data forward priority, etc.).

2.	If the AF is authorised by the NEF to provision the parameters, the NEF requests to create, update and store, or delete the provisioned parameters as part of the subscriber data via Nudm_ParameterProvision_Create, Nudm_ParameterProvision_Update or Nudm_ParameterProvision_Delete Request message, the message includes the provisioned data and NEF reference ID.

NOTE:	The remaining steps of the procedures are based on Figure 4.15.6.2-1 of TS 23.502 [3].

6.26.3	Impacts on services, entities and interfaces

NEF and UDM service operations:

-	Adding new parameter "Store and Forward Satellite Specific Parameters" as Input parameter of Nnef_ParameterProvision and Nudm_ParameterProvision.

* * * * End of Changes * * * *