SA WG2 Meeting #S2-162	S2-2404388

15 - 19 April, 2024, Changsha, China	(revision of S2-240xxxx)

Source:	ZTE

Title:	KI#2: update the Solution 23

Document for:	Approval

Agenda Item:	19.4

Work Item / Release:	FS_EnergySys / Rel-19

Abstract of the contribution: This paper proposes to update the solution 23 to resolve the EN and complete the impact clause.

1. Proposal

It is proposed to accept the change to 23.700-66.

>>>> Start of Change <<<<

## 6.23	Solution #23: Energy credit Provisioning and Energy credit based policy control for UE level

### 6.23.1	Key Issue mapping

This solution proposes that the energy credit can be provisioned in the UE subscription data via OAM or external AF. The PCF (e.g. AM-PCF, SM-PCF) obtains the energy credit information and make the policy decision.

This solution is related to the Key Issue #2 on Subscription and policy control to support energy efficiency and energy saving as service criteria. Especially to focus the following bulletin in the Key issue 2:

-	Whether and how to enhance the existing subscription and policy control framework to support energy related information as service criteria, including:

-	Whether and what new energy related UE subscription information are to be defined, and whether and how to use the energy related UE subscription information.

-	Whether and what new energy related policies are to be defined, and how to perform energy related policy control, e.g. to determine, provision and enforce energy related policies.

-	At what granularity (e.g. network slice, UE, NF, PDU Session, QoS flow, application ID, etc.) the energy related policy control can be performed.

### 6.23.2	Functional Description

This solution enhances the UE subscription to support energy related information as service criteria. The energy credit defined by TS 22.261 [8] is added to UE subscription. It can be per UE level, or per UE per DNN+S-NSSAI level.

The AF or 3rd party also can provision the energy credit for particular services to 5GC as application data. The existing mechanism can be used by PCF to perform the policy control based on the energy credit and energy consumption.

The figure 6.23.2-1 describes the architecture of this solution.

Figure 6.23.2-1: Architecture for Energy information Provisioning and policy control

On top of existing 5G architecture, a new NF, Energy Efficiency function (EEF) is introduced. In this solution, Energy Efficiency function can be standalone NF or co-located with NEF. The functionality of the EEF includes:

-	collects Energy related information in the 5GS, including: energy consumption related information.

-	calculates Energy usage status.

-	store the Energy usage status to UDR, or notify the consumer.

The high level principle/assumption of this solution can be summarized as follows:

-	There may be energy credit in the UE subscription data. It can be per UE level, or per UE per DNN+S-NSSAI level.

-	The AF may provision the energy credit for particular services to 5GC as application data. This data is stored in the UDR.

-	If the energy credit is per UE level in the subscription data, the AMF request the EEF to collect the UE level energy consumption information. If the energy credit is per PDU session level in the subscription data, the PCF subscribes the energy related information can trigger the EEF to collect the  PDU session level, energy consumption information (can reused the existing solution#1 and #6) and calculates the energy credit status.

-	The EEF store/update the energy credit status (e.g. including the Remaining energy credit, etc) to UDR if needed, e.g. after UE deregistration, PDU session release. And EEF can also notify the consumer (e.g. PCF) the energy credit status.

-	During PDU session establishment, the PCF subscribes the energy related information from EEF.

-	When the PCF make the Qos policy decision, e.g. receiving the AF session request, it can take the energy credit status into consideration.

### 6.23.3	Procedures

Figure 6.23.3-1: Procedure of Subscription and policy control to support Energy efficiency and energy saving as service criteria

Figure 6.23.3-1 shows a procedure of Subscription and policy control to support Energy efficiency and energy saving as service criteria. The AF provisioning part reuses the existing mechanism in clause 4.15.6.2 of TS 23.502 [3].

0.	The UE may have the Energy related subscription (e.g. Energy credit) in the UDM. It can be for the UE, or per UE per DNN+S-NSSAI level.

1.	The AF also may provision the Energy related subscription to the 5GC. It invokes the Nnef_ParameterProvision_Create or Nnef_ParameterProvision_Update to the NEF.

2.	If the AF is authorised by the NEF to provision the parameters, the NEF requests to create, update and store the provisioned parameters as part of the subscriber data via Nudm_ParameterProvision_Create/Update.

3.	UDM store the subscription data to UDR.

4a.	If the energy credit is per UE level in the subscription data, the AMF request the EEF to collect the UE level energy consumption information in initial registration.

4b.	The PCF (e.g. PCF for the PDU session) get the UE subscription data as specified in the TS 23.503 [4] clause 6.2.1.3. If the PCF finds there is energy credit related subscription data, the PCF may subscribe the energy credit related information

with the EEF.

5.	The EEF retrieves the energy credit subscription from UDM/UDR.

6.	The EEF collects the energy consumption data and calculates the energy credit status for the UE level, PDU session level. How the EEC collect the energy consumption data can use the solution #1 and #6. This step may be triggered when the EEF receives subscription to the UE energy status.

7.	The EEF notify the PCF about the latest energy status

8.	The EEF update the latest energy status to UDM, e.g. Remaining Energy credit if needed, e.g. in UE deregistration, PDU session release.

9.	The UDM store the energy status to UDR.

8.	If needed, the UDR notify the PCF about the latest energy status

10/11.	When it is needed (e.g. receiving the AF session request, or for the installed PCC rule), the PCF make the Qos policy decision (e.g. updating/creating PCC rule) taking the energy status into consideration.

### 6.23.4	Impacts on existing services, entities and interfaces

EEF:

-	New NF Functionalities are as description of clause 6.23.2.

NEF:

-	Enhance the Parameter Provision service operation to allow the AF to provision the energy credit to subscription data.

UDM:

-	There may be energy credit in the UE subscription data. It can be per UE level, or per UE per DNN+S-NSSAI level.

PCF:

-	subscribes the energy related information from EEF.

AMF:

-	If the energy credit is per UE level in the subscription data, the AMF request the EEF to collect the UE level energy consumption information in initial registration.

>>>> End of Change <<<<