SA WG2 Meeting #162								     												 S2-2404445

Changsha, China, April 15 – 19, 2024	(revision of S2-24xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0437 | rev | - | Current version: | 18.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for ProSe in NPNs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Philips International B.V. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19_ProSe_NPN |  |  |  |  |  | Date: |  |  | 2024-04-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 5G Proximity Services are currently not supported for NPNs. The ProSe specification TS 23.304 needs to be updated to enable this. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Expand the non-roaming architecture for ProSe to also cover NPN support, excluding support for inter-SNPN operation. - Allow combining a PLMN ID together with a Network Identifier (NID) to enable identification of an SNPN, and update PLMN specific terminology to also cover NPNs, except if the terminology of PLMN and the use of PLMN ID pertains to VPLMNs or Local PLMNs or Announcing PLMN IDs. - Enable discovery and selection of 5G ProSe UE-to-Network Relays that support access to an NPN (e.g. that are registered to a certain SNPN and/or that camp on a CAG cell of a certain PNI-NPN), excluding PNI-NPN support for Layer-3 5G ProSe UE-to-Network relay that require CAG. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5G Proximity Services are not supported for NPNs |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 4.2.X	5G ProSe in NPNs

The proximity based services defined in this specification are applicable not only to PLMN(s), but also to PNI-NPN(s) and SNPN(s), with the following adaptations:

-	the non-roaming architecure in clause 4.2.1 is supported, whereby the term PLMN can be interpreted as NPN.

-	the roaming architecture in clause 4.2.2 is not supported for SNPNs.

-	inter-SNPN operation in clause 4.2.3is not supported

-	for 5G ProSe Direct Discovery the procedures in clauses 5.2 and 6, the service interfaces in clause 7, and the identifiers as specified in clause 5.8.1 are used with the following adaptations:

-	the concept of Local PLMN for direct discovery is not supported for SNPNs.

-	the ProSe Application ID in clause 5.8.1.1 may be extended to allow the value of PLMN ID within the ProSe Application ID to be combined with a Network Identifier (NID) to identify an SNPN.

-	the ProSe Application Code in clause 5.8.1.4 may be extended to allow the value of PLMN ID within the ProSe Application Code to be combined with a Network Identifier (NID) to identify an SNPN.

-	the Announcing PLMN ID in clause 5.8.1.11 is not used in case of SNPNs.

-	the Monitored PLMN ID as defined in TS 32.277 [22] may be combined with a Network Identifier (NID) to identify an SNPN of an announcing UE when non-roaming.

-	in the 5G ProSe Direct Discovery with 5G DDNMF procedures as specified in clause 6.3.1, the term PLMN may also be interpreted as NPN, whereby PLMN IDs may be combined with a Network Identifier (NID) to identify an SNPN, except if it pertains to PLMN IDs of VPLMNs or Local PLMNs or Announcing PLMN IDs. The term HPLMN can be interpreted as SNPN. For PNI-NPN the term remains HPLMN.

-	the 5G ProSe UE-to-Network Relay reference architectures for Layer-2 UE-to-Network relay, Layer-3 UE-to-Network relay and UE-to-UE relay in clauses 4.2.7 and 4.2.8 are supported, except for the non-roaming architecture as specified for Layer-3 UE-to-Network relay in clause 4.2.7.1 which is not supported for SNPNs. The term PLMN can be interpreted as NPN and the term HPLMN can be interpreted as SNPN. For PNI-NPN the term remains HPLMN. The term VPLMN is not applicable in case of SNPNs.

-	for Authorization and Provisioning and for the subscription information of the ProSe service in NPNs, the procedures and functionality as specified in clauses 5.1 and 5.7 and 6.2 are used with the following adaptations:

- 	the term PLMN can be interpreted as NPN. The term HPLMN can be interpreted as SNPN. For PNI-NPN the term remains HPLMN. The term VPLMN is not applicable to SNPNs.

-	the 5G ProSe UE-to-Network Relay Discovery policy/parameters for Layer-2 5G ProSe UE-to-Network Relay and Layer-2 5G ProSe Remote UE in clause 5.1.4.1 are extended with an optional CAG Indicator per RSC. The CAG Indicator indicates a list of CAG identifiers for which the RSC is only allowed to be used if the Layer-2 5G ProSe UE-to-Network Relay is currently served by a CAG cell or can connect to a CAG cell that supports a CAG identifier indicated by the CAG Indicator.

-	for 5G ProSe Layer-2 UE-to-Network relay operation, the procedures as specified in clauses 5.4.2 and 5.4.4.2, and the identifiers as specified in clause 5.8.3 are used with the following adaptations:

-	the term PLMN can be interpreted as NPN.

-	a Layer-2 Remote UE has to operate in SNPN access mode, if it intends to access SNPN for ProSe via Layer-2 UE-to-Network relay.

-	for PLMN/NPN selection and relay selection in the 5G ProSe Layer-2 Remote UE, the 5G ProSe Layer-2 Remote UE may check whether a CAG identifier for a PNI-NPN to which it wants to connect is part of the CAG identifier list configured for an RSC using a CAG Indicator as per clause 5.1.4.1, and if the respective RSC is obtained from the 5G ProSe Layer-2 UE-to-Network Relay(s) during 5G ProSe UE-to-Network Relay Discovery, then the PNI-NPN related to the CAG identifier is considered available for NAS PLMN/NPN selection, otherwise the PNI-NPN may not be considered available (e.g. if communication to PNI-NPN is restricted to CAG cells). The Layer-2 5G ProSe UE-to-Network relay shall only include the respective RSC during discovery if it is currently served by a CAG cell or can connect to a CAG cell that supports a CAG identifier indicated by the CAG Indicator.

-	the Layer-2 UE-to-Network relay can participate in the 5G ProSe UE-to-Network Relay Discovery procedure for emergency service if the serving NG-RAN broadcasts ims-EmergencySupport or imsEmergencySupportForSNPN in system information as specified in TS 38.300 [12] and TS 38.331 [16].

-	the NCGI in clause 5.8.3.3 includes the Network identifier (NID) of the SNPN together with the PLMN ID in case the serving cell is operated by an SNPN.

NOTE 1: To prevent that unauthorized 5G ProSe Remote UEs can gain access to an NPN via 5G ProSe UE-to-Network Relay, the existing User Plane and/or Control Plane authorization verification procedures as specified in clause 6.3.4 of TS 33.503 and restricted 5G ProSe Direct Discovery as specified in clause 6.1.3.2 of TS 33.503 are reused.

NOTE 2: Additionally, in order to prevent that unauthorized 5G ProSe Remote UEs can gain access to a CAG cell operated by a PNI-NPN which a Remote UE is not allowed to access, the existing access control mechanisms as specified in clause 5.30.3.4 of TS 23.501 and clause 16.7.4 of TS 38.300 are reused, given that for Layer-2 relay the same procedures as for registration via Uu are used as it is transparent that the 5G ProSe Remote UE uses a Layer-2 5G ProSe UE-to-Network Relay to connect to the network.

-	for 5G ProSe Layer-3 UE-to-Network relay operation, the procedures as specified in clauses 5.4.1, 5.4.4.3 and 6.5, and the identifiers as specified in clause 5.8.3 apply with the following adaptations:

-	the term PLMN can be interpreted as NPN. The term HPLMN can be interpreted as SNPN. For PNI-NPN the term remains HPLMN. The term VPLMN is not applicable to SNPNs.

-	a Layer-3 UE-to-Network relay has to operate in SNPN access mode in order to relay traffic to/from an SNPN for a Layer-3 Remote UE.

-	the NCGI in clause 5.8.3.2 includes the Network identifier (NID) of the SNPN together with the PLMN ID in case the serving cell is operated by an SNPN.

- 	the TAI in clause 5.8.3.2 includes the Network identifier (NID) of the SNPN together with the PLMN ID in case the serving cell is operated by an SNPN.

NOTE 3: To prevent that unauthorized 5G ProSe Remote UEs can gain access to an NPN via 5G ProSe UE-to-Network Relay, the existing User Plane and/or Control Plane authorization verification procedures as specified in clause 6.3.3 of TS 33.503 and restricted 5G ProSe Direct Discovery as specified in clause 6.1.3.2 of TS 33.503 are reused.

NOTE 4: PNI-NPNs that rely solely on PNI-NPN specific slices or DNN are supported for Layer-3 5G ProSe UE-to-Network Relay. Layer-3 relay support for PNI-NPNs that rely on CAG are not supported.

- 	the support for 5G ProSe for UEs in limited service state as specified in clause 5.9 applies whereby the term PLMN can also denote an NPN.

Furthermore, access to an onboarding network via 5G ProSe UE-to-Network Relay is not supported for SNPNs.

.