3GPP TSG-SA2 Meeting #162	S2-2404616

15 - 19 April, 2024, Changsha, China

Title:	[draft] Clarification related to the RAT type only change impact to AM/UE Policy association

Response to:	LS on Clarification related to the RAT type only change impact to AM/UE Policy association (S2-2403881/C3-241578)

Release:	Rel-18

Work Item:	TEI18

Source:	SA2

To:	CT3

Cc:	CT1, CT4

Contact person:

#### Name:	Hyunsook Kim

#### E-mail Address:	hyuns.kim at lge dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1	Overall description

SA2 would like to thank CT3 regarding the LS on Clarification related to the RAT type only change impact to AM/UE Policy association.

SA2 has discussed the matter during SA2#162 meeting and would like to provide the following

Question 1: As per above requirements, there are multiple RAT types defined within one Access type, and there is possibility of UE mobility between RAT types (Inter RAT) within same Access. In which case, do we need the PCF to distinguish between different RAT types to provide AM/UE policies specific to RAT type?

- UE RAT type is determined as defined in clause 5.3.2.3 of TS 23.501 (as already indicated in C3-241578 from CT3), and the values are defined in TS 29.571 and distinguished by the PCF (e.g. “NR_LEO", “NR_MEO", “NR_GEO", etc.).

- Generally, the PCF may consider the change of RAT type for the policy decision. It’s up to operator policy to provide AM/UE policies specific to RAT type.

- As session related aspects as well as AM/UE policies, it is worth noting the following from clause 6.1.3.6 of TS 23.503.

If SMF indicates that a PDU Session is carried over NR satellite access or satellite backhaul, the PCF may take this information into account for the policy decision, e.g. together with any delay requirements provided by the AF.

2	Actions

To CT3

ACTION: 	SA2 kindly asks SA3 to take above information into consideration.

3	Dates of next TSG SA WG2 meetings

TSG-SA2#163		May 27, 2024 – May 31, 2024		Jeju, Korea

TSG-SA2#164		Aug. 19, 2024 – Aug. 23, 2024		Maastricht, Netherlands