3GPP TSG-WG SA2 Meeting #162	S2-245208

Changsha, China, 15 - 19 April, 2024	(revision of S2-24xxxx)

Title:	[DRAFT] Response LS on handling UEs accessing to GERAN/UTRAN over Gn/Gp

Response to:	S2-2403879/C3-241569

Release:	Rel-18

Work Item:	TEI18, TEI17_NIESGU

Source:	3GPP SA2

To:	3GPP CT3

Cc:

Contact Person:

Name:	Fenqin Zhu

E-mail Address:	zhufenqin@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None.

1. Overall Description:

SA2 thanks CT3 for their LS on handling UEs accessing to GERAN/UTRAN over Gn/Gp in C3-241569 and would like to provide the following feedback.

Q1: Whether the PCF and/or AF should identify the SGSN type change, and revert to normal handling when the UE moves back to a network with dedicated bearer/QoS flow support.

A1: When the SMF+PGW-C is enhanced to support UE accessing the network via GERAN/UTRAN, SGSN can only connect to PGW+SMF via Gn/Gp interface. Hence the PCF is not required to identify the SGSN type change, i.e. a new PCRT is not required, as the Change in Access Type PCRT can already inform about the situation that the UE is accessing to GERAN/UTRAN over Gn/Gp (cf. clause 6.1.3.5 of TS 23.503). The AF should also not need to know this as the PCF can always reject AF requests in this situation.

Furthermore, the deletion of the dedicated QoS flow deletion and rejection of secondary PDP context activation is done at the SMF+PGW-C as following:

- the dedicated QoS flow established in the 5G network can be regarded the same as a QoS flow not supported to be transferred to EPS, e.g. no EBI allocated for such a QoS flow. Hence, the AMF will not transfer the related QoS flow in the mapped EPS UE context to MME. And during the interworking procedures, the SMF+PGW-C deletes the PCC rule(s) associated with those QoS Flows and informs the PCF about the removed PCC rule(s) as described in TS 23.502.

- And when the UE is accessing the network via GERAN/UTRAN over Gn/Gp interface, the SMF+PGW-C can reject the secondary PDP context activation procedure directly.

Hence, there is no need for PCF and/or AF to be aware of an SGSN type change, but PCF can be notified of the RAT type change to GERAN/UTRAN.

2. Actions:

To CT3:

ACTION: 	SA2 kindly requests CT3 group to take the above responses into account.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #163		27th May – 31st May, 2024			Jeju, KR

TSG-SA2 Meeting #164		19th – 23rd August, 2024			Maastricht, NL