SA WG2 Meeting #162	S2-2405277

15 April – 19 April, 2024, Changsha, China

Source:	NTT DOCOMO, Nokia

Title:	Key Issue #4, Interim Conclusions

Document for:	Approval

Agenda Item: 	19.3

Work Item / Release:	FS_XRM_Ph2 / Rel-19

Abstract of the contribution: This paper proposes interim conclusions for KI#4

# 1	Discussion

This paper proposes interim conclusion principles for key issue #4

# 2	Proposal

It is proposed to include the following changes in TR 23.700-70.

* * * * Start of Changes – All New Text * * * *

## 8.X	Key Issue #4 Interim conclusions

The following conclusion principles are agreed for key issue #4.

1.	Multiple media flows may be multiplexed into a flow identified by the legacy packet filter. Either PDU Set based handling or ordinary QoS based handling may be applied to each media flow.

2.	For the multiplexed media flows which cannot be differentiated by using the legacy IP Packet Filter, the AF may provide additional Packet Filter in the "AF session with required QoS" procedure to the NEF/PCF. The additional Packet Filter is used to differentiate the media flow among multiple media flows that share the same IP 5-tuple.

a) 	For the media flows that are transported in RTP or SRTP, the additional Packet Filter may need to consider the Synchronization Source (SSRC) and Payload Type (PT) header fields.

b)	For the media flows that are transported over encrypted transport protocols, solution may use in-band assistance information from application.

Editor’s Note: If and how the additional Packet Filter is used to support in-band assistance information from application is FFS.

Editor's Note: The in-band assistance information from application is FFS.

Editor's Note: the details of additional Packet Filter are FFS. For the additional Packet Filter, whether enhance the legacy packet filter or define additional standalone packet filter is FFS.

3. 	The AF may provide distinct PDU Set QoS parameters, Protocol Description for each media flow.

4.	The AF may provide the QoS requirements for the multiplexed media flows either by:

a) providing the filters for the multiple media flows and their QoS requirements, at the same time in a single "AF session with required QoS" procedure request. In this case, Rel-18 multi-modal procedure is used.

b) using separate "AF session with required QoS" procedure request for each of them.

5. 	The PCC rule and N4 rules are enhanced to support the additional Packet Filter.

6. 	The SMF may provide the received additional Packet Filter and Protocol to the UPF in the N4 rules. The PDI in PDR is extended to support the additional Packet Filter.

Editor's Note: whether and how to support to identify media flow in uplink is FFS.

Editor’s note: 	The above conclusion principles are only tentatively agreed.

* * * * End of Changes * * * *