3GPP TSG-SA2 Meeting #162	S2-2405487

# Changsha, Hunan Province, China, 15th Apr 2024 - 19th Apr 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0358 | rev | 1 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | QMC Support for MBS Multicast and Broadcast |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18, NR_QoE_enh-Core |  |  |  |  |  | Date: |  |  | 2024-04-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to the LS from RAN3 (R3-235913), QMC is supported for MBS multicast and broadcast in Rel-18. Based on this LS, general QMC impacts to 5GS have already been documented in TS 23.501 and TS 23.502, but MBS specific impacts are not yet documented, As specified in Clause 21 of TS 38.300, QMC meassurements by the UE are also possible if the UE receives MBS broadcast in RRC-INACTIVE or RRC-IDLE mode, or MBS multicast in RRC-INACTIVE mode. The per UE-configuration to perform QMC also applies for MBS, i.e. there is no per MBS session configuration. It is beneficial to describe QMC support for MBS in MBS specficiation, from feature completeness perspective. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Allignment with Clause 21 of TS 28.300: QMC is also supported to measure the QoE related to the reception of MBS multicast and broadcast sessions. MBS-related QMC by a UE is also supported while the UE receives an MBS braodacast session in RRC-INACTIVE or RRC-IDLE mode, or an MBS multicast session in RRC-INACTIVE mode. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MBS specific impacts of QMC to 5GS are not documented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New 6.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 6.x	QoE Measurement Collection Support for MBS

Quality of Experience (QoE) measurement collection (QMC) may be supported in 5GS as described in Clause 5.25.3 of TS 23.501 [5].

QMC may also be supported to measure the QoE related to the reception of MBS multicast and broadcast sessions.

NOTE:	The per UE-configuration to perform QMC also applies for MBS, i.e. there is no per MBS session configuration.

As specified in Clause 21 of TS 38.300 [9], QMC for MBS by a UE may also be supported while the UE receives an MBS broadcast session in RRC-INACTIVE or RRC-IDLE mode.