3GPP TSG-SA2 Meeting #166	 S2-2411599

# 18 – 22 November 2024, Orlando, USA	   						                (revision of S2-2411256)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0584 | rev | 5 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on Data Collection by LMF procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, [Lenovo, Huawei] |  |  |  |  |  |  |  |  |  |
| Source to TSG: | 3GPP SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-11-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | CR provides updates to procedure for Data Collection from the UE described in clause 5.18.1 and also addresses the following ENs. Editor's note: Whether the AMF can further select and return SUPIs in step 2 and whether the AMF needs to consider the UE capabilities supporting AI/ML based positioning is FFS. Editor's note: Whether and how LMF is informed about the UE capability to report input data for ML Model training is FFS. For SA2#166: Before subscribing the UE list for model training, LMF need to discover the AMFs that served the specific area fro NRF. It is proposed to add the AMF discovery procedure before the existing step 1. When LMF subscribes the UE list to AMF, LMF may further include the minimum number of report UE in the request message to AMF, if there are not enough available UE in the area of interest, the AMF may further indicate the cause to LMF. Regarding the EN for step 5, UE always reports the measurement data periodically or due to the event trigger based on the data collection request. There is no need to specify the UE reporting condition in this procedure, but we can add a reference about the UE cancellation the location procedure if UE if not be able to report. Editor’s Note: Conditions for the UE to report measurement data are FFS |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following clarifications are made: - LMF may be triggered by NWDAF to collect data for training an AIML model for Direct AIML positioning Rev 1 includes changes from S2-2410673 - Clarify the interaction between AMF and LMF in step 2. - Clarify the procedure between LMF and UE in step 4. For SA2#166: - Add the AMF discovery procedure as new step 1a and 1b. - LMF provides the minimum number of UE to AMF in the Namf_EventExposure_Subscriber_Request in the new step 2. - AMF may provide a cause to LMF if not enough available UE in the area of interest in the new step 3. - Remove one of the EN in step 5 in clause 6.x.2. - Add a new trigger that UE may cance the data collection procedure in clause 6.3.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Procedure for data collection from the UE is not complete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.18.1, 6.X (new), 6.x.1 (new), 6.x.2 (new), 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR should be treated together with CR 0574 CR0595 and CR 0597 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************************************ FIRST CHANGE *****************************

### 5.18.1	Void

# * * * * Next change * * * *

## 6.X	Procedures of data collection for LMF-based AI/ML Positioning

### 6.x.1	General

This clause describes the procedure for data collection by the LMF for input data to train LMF-based AI/ML models for Direct AI/ML positioning.

### 6.x.2	Data Collection at LMF to train the LMF-based AI/ML positioning model to perform positioning based on UE measurements.

The LMF needs to obtain input data for LMF-based AI/ML Positioning model training and may also request the UE to provide the UE location. The LMF-based AI/ML Positioning model is trained to perform UE positioning for UEs located in an area of interest that may expand over multiple TA or cover multiple NG-RAN nodes.

Figure 6.X.2-1: Data collection by LMF to train the LMF-based AI/ML Positioning model using UE measurements

1.	The LMF starts data collection for the purpose to train an AI/ML Model for UE Positioning (this may be based on an internal trigger in the LMF or a request from the NWDAF).

The LMF invokes an Nnrf_NFDiscovery_Request service operation to an NRF to discover AMF(s) that served for the area of interest.

2.	The LMF subscribes to AMF to retrieve the list of SUPIs located in an area of interest and the UE Positioning Capability for each UE using Namf_EventExposure_Subscriber_Request (Target of Event Reporting = "any UE", Event ID = "UEs in/out area of interest" and “UE Positioning Capability”). The LMF may also provide the minimum number of UEs in the Namf_EventExposure_Subscriber_Request message to indicate the required minimum number of UE IDs. If the data collection procedure is triggered by NWDAF and NWDAF provides the number for data samples, the LMF may take into account the NWDAF provided the number for data samples to determine the minimum number of UE.

3.	The AMF send Namf_EventExposure_Subscriber_Response ("list of SUPIs in the area of interest"). If “UE Positioning Capability” is also requested in step 1, AMF includes UE Positinong Capabiliy, UE User Plane Positioning Capabilities if available for each SUPI in the response message sent to LMF. If the available number of the UEs is less than the minimum number of UE ID requested in step 2, the AMF may report the cause along with the “list of SUPI in the area of interest” to indicate “not enough available UE”.

4.	The LMF further determines the UEs from the list of SUPIs that received from AMF in step 2 for data collection to train the AI/ML based positioning model based on UE Positioning Capability, UE User Plane Positioning Capabilities, UE consent check result, the PRU information available in the LMF and operator’s policy.

Editor's note:	Further details on user consent for data collection for model training for LMF-based AI/ML Model positioning will be aligned with SA WG3.

5.	For each UE that provides consent to data collection for model training and can support data collection the LMF initiates a request for input data.

UE may cancel the data collection procedure as described in clause 6.3.2 (e.g. if the UE is powered off or if the UE cancels the location request based on user’s input).

Editor's note:	The procedure to collect the input data for AI/ML based positioning calculation by the LMF from the UE is FFS. The procedure will be coordinated with RAN WGs.

Editor's Note:	The input data used for AI/ML based positioning will be decided by RAN WGs.

Editor's Note:	Conditions for the LMF to trigger data collection is FFS, e.g., whether and how the LMF can consider load to determine when to perform step 4 is FFS.

Editor’s Note:	Data collection for performance monitoring is FFS

At the time the LMF decides that the input data for a UE are not needed, e.g. the user consent for data collection for model training is revoked, the LMF stops to collect the input data from the UE to train the LMF-based AI/ML Positioning model.

# * * * * Next change * * * *

### 6.3.2	Cancellation of Reporting of Location Events by a UE

Figure 6.3.2-1 summarizes a procedure to enable a UE to cancel a deferred 5GC-MT-LR procedure for periodic, or triggered location events (e.g. if the UE is powered off or if the UE cancels the location request based on user's input). It is assumed that a deferred 5GC-MT-LR for periodic or triggered location events has already been initiated in the UE according to steps 1-17 for the procedure in clause 6.3.1. If a network entity (e.g. (H)GMLC, AMF or LMF) cancels a deferred 5GC-MT-LR procedure for periodic or triggered location events, part of the procedure in clause 6.3.3 for cancellation by an AF, an NF or external LCS Client would be used to cancel towards the UE and part of the procedure in this clause would be used to cancel towards the AF, the NF or external LCS Client.

The procedure described in Figure 6.3.2-1 also enables a UE to cancel a data collection by LMF to train the LMF-based AI/ML Positioning model as described in clause 6.X.2 (e.g. if the UE is powered off or if the UE cancels the location request based on user's input).

Figure 6.3.2-1: UE Cancellation of a Deferred 5GC-MT-LR for periodic or triggered location events

1.	The UE performs a UE triggered service request as defined in clause 4.2.3.2 of TS 23.502 [19] if in CM-IDLE state in order to establish a signalling connection with the AMF.

2.	The UE sends a Cancel Location request message to the LMF which is transferred via the serving AMF and is delivered to the LMF using an Namf_Communication_N1MessageNotify service operation. The UE includes the deferred routing identifier originally received in step 16 of the procedure in clause 6.3.1 (or as updated by step 26 in clause 6.3.1 or by the procedure in clause 6.4) in the NAS Transport message used to transfer the cancel location request from the UE to the AMF. The AMF then forwards the cancel location request to either the serving LMF or any suitable LMF based on whether the deferred routing identifier indicates a particular LMF or any (default) LMF. The UE also includes the (H)GMLC contact address and the LDR reference number.

If the procedure is performed to cancel the data collection by LMF to train the LMF-based AI/ML Positioning model as described in clause 6.X.2, step 3 to step 5c is skipped.

3.	In the case of roaming, the LMF selects a VGMLC. The LMF then invokes an Nlmf_Location_EventNotify service operation towards the selected VGMLC or (H)GMLC with an indication of the cancelation of location event reporting, the (H)GMLC contact address and LDR reference number.

NOTE 1:	In the case of roaming, the LMF may select the VGMLC for step 3 using the NRF service or using configuration information in the LMF or may use the same VGMLC as for steps 3-8 of clause 6.3.1 (e.g. if the LMF acts as a serving LMF and received the VGMLC address from the AMF as part of step 14 of clause 6.3.1).

4.	This step is skipped for a non-roaming UE. For a roaming UE, the VGMLC invokes an Ngmlc_Location_EventNotify service operation to forward the cancel location request (including the LDR reference number) to the HGMLC which identifies the periodic and triggered location request from the LDR reference number.

NOTE 2:	As an optional optimization for a roaming UE, instead of performing steps 3 and 4, the LMF may invoke the Nlmf_Location_EventNotify service operation directly towards the HGMLC.

5.	The (H)GMLC uses the LDR reference number received in step 3 or step 4 to identify the periodic and triggered location request received in step 1 of clause 6.3.1 and then forwards the cancel location to the external LCS client, the NF or the AF (via the NEF).

6.	The LMF returns an acknowledgment to the UE via the serving AMF.

************************************ END OF CHANGE *****************************