3GPP TSG-SA WG2 Meeting #166	S2-2411950

Orlando, November 18 - 22, 2024

Source:	vivo

Title:	Discussion on early media during USU establishment

Document for:	Discussion

Agenda Item:	19.1.2

Work Item / Release:	5GSAT_Ph3_ARCH

Abstract: This document discusses the early media continuity during USU establishment procedure.

- Introduction

During SA2#165 meeting, the issue of early media handling is indicated during the USU establishment procedure. During the USU establishment procedure, the originating P-CSCF first selects ground AGW as default and sends satellite ID to the terminating side. When terminating P-CSCF selects onboard AGW and sends satellite ID to the originating P-CSCF, the originating P-CSCF changes the ground AGW to onboard AGW, while the early media may already be on going with the originating UE via the ground AGW before the AGW relocation.

- Discussion

  - 2.1 Deployment possibilities

Following figure illustrates the architecture and corresponding IP domains:

IP satellite-domain is on satellite, the onboard-AGWs in IP satellite-domain are IP routable via ISL. IP ground-domain is on ground, the ground-AGWs and early media functions (i.e., MF) are in IP ground-domain. Whether the entities in the two different IP domains can exchange IP packets depends on operator policy and configuration. In some deployment cases, the two IP domains are isolated.

Observation 1: IP satellite-domain and IP ground-domain may be isolated.

If the two IP domains are isolated, considering the early media function is on ground, the AGW relocation for using onboard-AGW for USU will disable the early media delivery (i.e., Path B is not available).

Observation 2: If IP satellite-domain and IP ground-domain are isolated, the AGW relocation will terminate the early media

If the two IP domains are not isolated, when the early media has been played via the ground-AGW before AGW relocation (i.e., Path A is used initially), the changing from ground-AGW to onboard-AGW will cause the path switch for the early media (i.e., from Path A to Path B). During the path switch, the early media is interrupted.

Observation 3: If IP satellite-domain and IP ground-domain are not isolated, the AGW relocation will interrupt the early media played before AGW relocation.

If the two IP domains are not isolated, when the early media is played after AGW relocation (i.e., Path B is used initially), there will be no issue.

Observation 4: If IP satellite-domain and IP ground-domain are not isolated, if the early media played after AGW relocation, the early media continuity can be sustained.

For CAT service in TN case, normally the IMS AS will instruct to play early media when SIP 180 Ringing is received, which means the early media is played after AGW relocation. But considering the delay of the signalling transmission is significant in satellite access, in order to improve user experience, the operator may determine to start the early media playing when first SDP offer/answer is received by the IMS AS serving the early media service.

Observation 5: The early media may be played before AGW relocation in the case of satellite access.

2.2 Proposal

Considering the P-CSCF is not able to be aware of when the early media is start to play, except that operator only deploys early media services that start to be played when receiving SIP 180 Ringing, it is proposed not to change the AGW for playing early media except operator policy allowed.

Proposal 1: The IMS shall be able to use ground AGW for early media and onboard-AGW for regular media in USU case.

There’re three modes for providing early media: gateway model, forking model, and early-session model. Early-session model is not deployed, so only the other two modes are considered below.

Gateway model is illustrated as following:

In gateway model case, the UE#1 will receive SDP answer with only early media information, then after session is answered, the IMS will update the session and regular media information is negotiated. In order to keep early media continuity when early media is playing, the AGW relocation shall not be performed until the IMS session is answered.

Forking model is illustrated as following:

:

In forking model case, the originating side will receive multiple responses with different dialog IDs, and different dialog with different SDP answer, i.e., originating P-CSCF will receive both SDP answer for early media and SDP answer for regular media. The P-CSCF can use the SDP answer for regular media to instruct onboard-AGW for allocating and configuring the resources for regular media (i.e., AGW relocation for regular media). In order to keep early media continuity when early media is playing, the AGW relocation for early media shall not be performed, i.e., the P-CSCF uses the SDP answer for early media to instruct ground-AGW for configurating the resources for early media. When the IMS session is answered by terminating UE, the early media will be stopped by early media function, then ground-AGW can be released.

Proposal 2: The IMS shall be able to sustain using ground-AGW for early media, and not release ground AGW until IMS session is answered.

- Conclusion

Based on the above discussion, the following are observed:

Observation 1: IP satellite-domain and IP ground-domain may be isolated.

Observation 2: If IP satellite-domain and IP ground-domain are isolated, the AGW relocation will terminate the early media

Observation 3: If IP satellite-domain and IP ground-domain are not isolated, the AGW relocation will interrupt the early media played before AGW relocation.

Observation 4: If IP satellite-domain and IP ground-domain are not isolated, if the early media played after AGW relocation, the early media continuity can be sustained.

Observation 5: The early media may be played before AGW relocation in the case of satellite access.

And the following are proposed:

Proposal 1: The IMS shall be able to use ground-AGW for early media and onboard-AGW for regular media in USU case.

Proposal 2: The IMS shall be able to sustain using ground-AGW for early media, and not release ground AGW until IMS session is answered.