3GPP TSG-WG SA2 Meeting #166	S2-2412007

Orlando, US, 18th Nov – 22nd Nov, 2024	(revision of S2-240xxxx)

# Title:	[DRAFT] LS on reply to LS on OAM requirements to support regenerative payload

# Response to:	LS (S2-2411306/R3-245845) on <LS on OAM requirements to support regenerative payload> from RAN3

# Release:	R19

# Work Item:	5GSAT_Ph3_ARCH

Source:	[Huawei to be] SA2

To:	RAN3, SA5

Cc:

Contact Person:

Name:	Steven Wenham

Tel. Number:

E-mail Address:	steven DOT wenham AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

# 1	Overall description

SA3 thanks RAN3 for the Liaison S2-2411306/R3-245845 on OAM requirements to support regenerative payload.

The LS describes as following:

1. Overall Description: RAN3 discussed the potential issues on support of regenerative payload for NR NTN, RAN3 has agreed OAM based solution, which assumes that the information of supported TAIs for NTN gNB is provided to AMF by OAM. 2. Actions: To SA2 and SA5: ACTION: RAN3 respectfully requests SA2 and SA5 to take above into account.

SA2 has the following considerations:

1. Do TAIs supported by NTN gNB change with the movement of satellite following the same principles of transparent satellite access? If yes, does it mean different TAIs for a NTN gNB will be provided to the AMF by OAM at different times with the movement of satellite, which will guarantee that the AMF always explicitly knows what TAIs are supported at the time by an NTN gNB?

2. Based on CRs (S2-2411251 and R3-245799) agreed by SA2 and RAN3, NG Removal procedure is defined to remove interface between a gNB and an AMF. SA2 believes after receiving a NG Removal message, the AMF will release the information of supported TAIs and it is the gNB to determine the time to initiate the NG Removal procedure based on the AMFs serving area (e.g., the serving PLMN allowed operation area).

SA2 would like to confirm whether above considerations are aligned with RAN3 and SA5.

2. Actions:

To RAN3 and SA5 group.

ACTION: 	SA2 asks RAN3 and SA5 groups to take the above information into account in their further work and provide feedback.

3. Date of Next TSG SA WG2 Meetings:

TSG SA2 Meeting #166-Adhoc-e	20th Jan – 24th Jan, 2025			Online

TSG-SA2 Meeting #167		17th Feb – 21st Feb, 2025			Athens, GR