3GPP TSG-SA WG2 Meeting #167	S2-2501448

Athens, Greece, 17th Feb 2025 – 21st Feb 2025

Agenda item:	19.6.2

Source:	Nokia, Vodafone

Title:	Discussion on the Additional ULI

WI/Release:	VMR_Ph2/Rel-19

Document for:	Discussion and Decision

# 1	Discussion

SA2#166 agreed a 23.501 CR in S2-2501322 (included in the LS to RAN3 in S2-2501324) which is describing the Additional ULI only includes information from the broadcasted PLMN. Specifically, For the case of MWAB broadcasted PLMN/SNPN being different from the BH PLMN/SNPN, MWAB-gNB formulates the Additional ULI based on MWABs geo-location.  If the MWAB broadcasted PLMN/SNPN is the same as the BH PLMN/SNPN, then the TAI/NR CGI of the cell serving the MWAB-UE is included in the Additional ULI.

This rules out that the MWAB-UE can read the BH-PLMN/SNPN TAI and NR CGI if the BH PLMN/SNPN is not the same as the broadcasted PLMN/SNPN despite the MWAB implementation can read the ULI related to the BH PLMN/SNPN cell and place it in the Additional ULI (when this is the same as the broadcasted PLMN), even if this option is preferred by the involved PLMNs.

Upon further reflection Nokia believes that which of the two methods is used should be a choice of the broadcasted PLMN operator (based on the ability to establish agreement with the BH-PLMN operator to obtain a suitable mapping information of their NR CGI/TAI to location) and then the MWAB can be configured accordingly. For instance, operator A and operator B merge and they can easily exchange information mapping TAI/NR CGI to location in respective PLMNs as now they are owned by the same legal entity and are progressively merge operations.

Also, the insertion of the TAI/NR CGI of BH PLMN in the Additional ULI requires no MWAB configuration efforts. By contrast, we consider that the information needed to map geolocation to a TAI/NR CGI in the broadcasted PLMN is rather complex and, unless standardized, will inevitably be per MWAB vendor, which in turn means the PLMN operator will need effort to translate a PLMN operator defined mapping into a vendor specific mapping (which may mean custom integration efforts). At present we see no proposal to standardize the encoding of the mapping of geolocation to NR CGI/TAI in 3GPP.

Considering the discussion above, we propose that the choice of behavior is not hardcoded but subject to configuration by the OAM of the broadcasted PLMN. It also means that if the PLMNs are different it is possible that the TAI/NR CGI in Additional ULI are not those of the broadcasted PLMN.

# 2	Conclusions

It is proposed that the Additional ULI behaviour is configurable in the MWAB by the broadcasted PLMN OAM as per CR in S2-2501422. If agreeable, an updated LS to RAN3 should be prepared.