3GPP TSG-SA2 Meeting #167	S2-2501614

Athens, Greece, February 17 – 21, 2025

Source:	Ericsson

Title:	Considerations and proposal for the S&F mode change behaviour

Document for:	Discussion/Agreement

Agenda Item:	19.1.2

Work Item / Release:	5GSAT_Ph3-ARC / Rel-19

Abstract of the contribution: This paper provides an analysis on the mode change scenarios and proposes considerations for defining mode change behaviours.

# 1.	Introduction

S&F mode change is a topic in the exception request agreed by SA (SP-241918):

(2b) – Functional specification of expected UE and network behaviour when transitioning between operation in S&F mode and operation in normal mode

Several proposals to address this were discussed at SA2#166-adhoc-e meeting, mainly focusing on defining UE or network behaviours when there is a change.

The objectives of the proposals differed: some aim to maintain continuous alignment between UE and network, especially for the connected UE, at the expense of immediate detach or S1 release from the network to trigger UE reattach/TAU, while some aim the alignment when there is a need for UE signalling/data transfer with the network.

This paper tries to analyse what are the scenarios when there is mode change.

# 2.	Analysis

In general, there are two different scenarios:

- UE is accessing a cell, and the satellite broadcasting this cell decides to change between normal and S&F mode, e.g. due to a gain/loss in its feeder link.

- UE moves from one cell to another cell, and each cell has different operation mode.

## Scenario a)

A UE in this scenario can be either in idle or connected mode. However, a question is how long time in total (no matter normal mode or S&F mode) the UE can be served by this satellite. The LEO satellite’s visibility to a UE could be very short, typically a few minutes. After that this satellite leaves the UE and another satellite may cover the UE area. So, if a satellite providing the coverage to this UE experiences gain/loss in feeder link, and sometime later, another satellite providing the coverage to this UE around the same location it is likely that it also experiences gain/loss in feeder link. For example, if a UE is located near a coast and the satellite flies over this area will have a gain/loss of feeder link where the ground station is deployed in the continent, then from the UE point of view, every time it is served by a new satellite, there will be a switch of normal mode and S&F mode. So, pushing for an “mode alignment” between UE/network by triggering signalling (e.g. TAU) without considering whether there is user data to be exchanged is a waste, because after one alignment, there will be another alignment.

Furthermore, in reasonable deployments the satellite may continue operating in S&F mode if it gains feederlink connectivity for a few minutes and then loses it again, to avoid toggling back and forth between different modes. Also, in a quasi-earth-fixed cell scenario, the LEO satellite can avoid scenario a) by doing the mode switch only when it relocates a cell from one area to another area, avoiding impact to the CONNECTED mode UEs in a given area.

Observation 1: It is a waste of resources to trigger signalling to do alignment without considering whether user data exchange is needed. It is also not clear if scenario a) is a main case or rather a corner case in real deployments.

For UE in connected mode and using data radio bearers, if the transition is from normal mode to S&F mode, if a S&F deployment does not support radio bearer UP, then the S1-U will be disconnected, and corresponding signalling can be initiated as per current specification, such as error indication due to transport layer issues, then there will be an alignment following the recovery/cleanup signalling. No new mechanisms would be needed.

For a UE in connected mode and the transition is from S&F mode to normal mode, and if a S&F deployment does not support data radio bearer UP, the use case is not clear why the UE requires immediate data transfer over data radio bearer user plane once the satellite starts normal mode, given the fact that the current satellite can disappear in a short time.

Observation 2: Existing error handling will be able to cope with transition from normal mode to S&F mode, and there is no clear use case for immediate data transfer over radio bearer when transition from S&F mode to normal mode.

## Scenario b)

Since the main assumption for a S&F deployment is NB-IoT, in this paper only idle mode mobility is discussed due to no connected mode mobility support for NB-IoT.

A TAU procedure triggered from the UE in the new cell may not always work, e.g. in a split-MME case when the old cell was in S&F mode, if the old serving MME (on ground) still does not have the up-to-date UE context, this will be a TAU using obsolete UE context in the network which may cause the TAU procedure to fail, and the UE needs to reattach, causing extra delays compared to if the UE had initiated attach already from the start.

Observation 3: From this perspective, it is more appropriate to define a common/single UE behavior, that is robust and is likely to work, to handle the case when UE detects mode change and requires access to a network for data transfer.

# 3.	Proposal

Scenario a) can be handled using existing mechanisms. It is also not clear that immediate signalling to handle the mode change is needed, e.g. if there is no user data.

For scenario b), a common UE behaviour (i.e. attach) triggered when there is a need for data transfer is appropriate approach to handle mode change.