3GPP TSG-WG SA2 Meeting #167	S2-2501637

# Athens, Greece, February 17-21, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6094 | rev | - | Current version: | 19.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | RAN Paging Priority corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Peraton Labs, CISA ECD, AT&T, Verizon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1, TEI19 |  |  |  |  |  | Date: |  |  | 2025-02-7 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There is no service priority in Core Network (CN) assisted RAN paging information, according to TS 38.413. Service priority is sent from the AMF to NG-RAN apart from the CN assisted RAN paging information and is mostly delivered in different messages. Service priority refers to priority handling during RAN Paging. RAN calls this RAN Paging Priority, which is distinct from Paging Priority. This CR proposes changes in TS 23.501 to align with RAN on the handling of service priority. |  |  |  |  |  |  |  |  |
| Summary of change: |  | First change: In clause 5.4.3.3 on Paging Priority changed ‘Core Network Assisted RAN Paging Information’ to ‘service priority’. Clarified in which messages service priority may be included. Clarified that the paging can be to the UE and via Xn to other NG-RAN nodes. Second change: Moved the definition of service priority from clause 5.4.6.3 Core Network assisted RAN paging information into clause 5.4.3.3 on Paging Priority. Third change: In clause 5.22.3 adding - For a UE in RRC_INACTIVE state, NG-RAN will use priority paging, including over X2 and Xn, when it receives an ARP used for MPS from an AMF over N2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Continued misalignment. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.3.3, 5.4.6.3, 5.22.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR … |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 5.4.3.3	Paging Priority

Paging Priority is a feature that allows the AMF to include an indication in the Paging Message sent to NG-RAN that the UE be paged with priority. The decision by the AMF whether to include Paging Priority in the Paging Message is based on the ARP value in the message received from the SMF for an IP packet waiting to be delivered in the UPF. If the ARP value is associated with select priority services (e.g. MPS, MCS), the AMF includes Paging Priority in the Paging Message. When the NG-RAN receives a Paging Message with Paging Priority, it handles the page with priority.

The AMF while waiting for the UE to respond to a page sent without priority receives another message from the SMF with an ARP associated with select priority services (e.g. MPS, MCS), the AMF sends another Paging message to the (R)AN including the Paging Priority. For subsequent messages, the AMF may determine whether to send the Paging message with higher Paging Priority based on local policy.

For a UE in RRC_INACTIVE state, the NG-RAN determines Paging Priority via Xn, as per TS 38.413 [34] based on the ARP associated with the QoS Flow as provisioned by the operator policy and the service priority. Service priority is specified as RAN Paging Priority in TS 38.413 [34]. The AMF may include service priority in the PDU Session Resource Setup, Modify and Release messages, the UE Context Modification message, and the Downlink NAS Transport message. Service priority (values 1 to 256) provides the AN with the importance of the downlink data and signalling. The AMF derives this service priority based on available information as described in clause 5.4.6.3. The method to derive the service priority is implementation dependent and can be controlled by an operator.

# * * * * Second change * * * *

#### 5.4.6.3	Core Network assisted RAN paging information

Core Network assisted RAN paging information aids the RAN to formulate a RAN paging policy and strategy in RRC_INACTIVE state, besides the PPI and QoS information associated to the QoS Flows as indicated in clause 5.4.3.

CN assisted RAN paging information may be derived by the AMF per UE and/or per PDU Session based on collection of UE behaviour statistics, Expected UE Behaviour and/or other available information about the UE (such as subscribed DNN, SUPI ranges, Multimedia priority services) and/or information received from other network functions when downlink signalling is triggered.

The Core Network may provide the CN assisted RAN paging information to RAN in different occasions, e.g. during downlink N1 and N2 message delivery, etc.

# * * * * Third change * * * *

### 5.22.3	Invocation-related Priority Mechanisms

The generic mechanisms used based on invocation-related Priority Mechanisms for prioritised services are based an interaction with an Application Function and between the Application Function and the PCF over Rx/N5 interface.

These mechanisms apply to mobile originated as well as mobile terminated SIP call/sessions (clause 5.21 of TS 23.228 [15]) and Priority PDU connectivity services including MPS for Data Transport Service.

NOTE 1:	Clause 5.21 of TS 23.228 [15] is applicable to 5GS, with the understanding that the term PCRF corresponds to PCF in the 5GS.

Invocation-related mechanisms for Mobile Originations e.g. via SIP/IMS:

-	PCF: When an indication for a session arrives over the Rx/N5 Interface and the UE does not have priority for the signalling QoS Flow, the PCF derives the ARP and 5QI parameters, plus associated QoS characteristics as appropriate, of the QoS Flow for Signalling as per Service Provider policy as specified in clause 6.1.3.11 of TS 23.503 [45].

-	PCF: For sessions such as MPS, when establishing or modifying a QoS Flow for media as part of the session origination procedure, the PCF selects the ARP and 5QI parameters, plus associated QoS characteristics as appropriate, to provide priority treatment to the QoS Flow(s).

-	PCF: When all active sessions to a particular DN are released and the UE is not configured for priority treatment to that particular PDU Session for a DN, the PCF will downgrade the IMS Signalling QoS Flows from appropriate settings of the ARP and 5QI parameters, plus associated QoS characteristics as appropriate, to those entitled by the UE based on subscription.

-	SMF/AMF: Based on the QoS parameters (e.g. ARP, 5QI) of a PDU Session, the SMF may provide a PDU Session Priority to the AMF. The AMF stores the information for further PDU Session related priority handling (i.e. for determination of Message Priority header of subsequent message related to the PDU Session).

Invocation-related mechanisms for Mobile Terminations e.g. via SIP/IMS:

-	PCF: When an indication for a session arrives over the Rx/N5 Interface, mechanisms as described above for Mobile Originations are applied.

-	UPF: If an IP packet arrives at the UPF for a UE that is CM-IDLE, the UPF sends a "Data Notification" including the information to identify the QoS Flow for the DL data packet to the SMF, as specified in clause 4.2.3.3 of TS 23.502 [3].

-	SMF: If a " Data Notification" message arrives at the SMF for a QoS Flow associated with an ARP priority level value that is entitled for priority use, delivery of priority indication during the Paging procedure is provided by inclusion of the ARP in the N11 interface "N1N2MessageTransfer" message, as specified in clause 4.2.3.3 of TS 23.502 [3]. Based on the QoS parameters (e.g. ARP, 5QI) of a PDU Session, the SMF may also provide a PDU Session Priority to the AMF.

-	AMF: If an "N1N2MessageTransfer" message arrives at the AMF:

-	from an SMF containing an ARP priority level value that is entitled for priority use;

-	from the SMSF with an MT-SMS related request and either an indication that MPS for Messaging is enabled for the UE from the UDM, or with a Message Priority header value used for MPS for Messaging;

the AMF:

-	handles the request with priority and includes the "Paging Priority" IE in the N2 "Paging" message set to a value assigned to indicate that there is an IP packet at the UPF or an MT-SMS at the SMSF entitled to priority treatment, as specified in clause 4.2.3.3 or clause 4.13.3.6 of TS 23.502 [3], when the UE is CM-IDLE; or

-	determines the service priority ("RAN Paging Priority" IE as specified in TS 38.413 [34]) and includes the service priority in the N2 Downlink NAS Transport message to the NG-RAN, when the UE is CM-CONNECTED.

If a PDU Session Priority is provided by the SMF, the AMF stores the information for further PDU Session related priority handling (i.e. for determination of Message Priority header of subsequent message related to the PDU Session).

-	SMF: For a UE that is not configured for priority treatment, upon receiving the "N7 Session Management Policy Modification" message from the PCF with an ARP priority level that is entitled for priority use, the SMF sends an "N1N2MessageTransfer" to update the ARP for the Signalling QoS Flows, as specified in clause 4.3.3.2 of TS 23.502 [3]. Based on the QoS parameters (e.g. ARP, 5QI) of a PDU Session, the SMF may also provide a PDU Session Priority to the AMF.

-	AMF: Upon receiving the "N1N2MessageTransfer" message from the SMF with an ARP priority level that is entitled for priority use, the AMF updates the ARP for the Signalling QoS Flows, as specified in clause 4.3.3.2 of TS 23.502 [3].

-	NG-RAN: Inclusion of the "Paging Priority" in the N2 "Paging" message or the "RAN Paging Priority" in the N2 "Downlink NAS Transport" message triggers priority handling of paging in times of congestion at the NG-RAN as specified in clause 4.2.3.3 or clause 4.13.3 of TS 23.502 [3], respectively.

-	NG-RAN: For a UE in RRC_INACTIVE state, receipt of an ARP value associated with priority services (e.g., MPS) from the AMF over N2 (e.g., in the PDU Session Resource Setup or Modify message), triggers priority handling of paging, including over Xn, in times of congestion at the NG-RAN, as specified in clause 4.8 of TS 23.502 [3].

Invocation-related mechanisms for the Priority PDU connectivity services:

-	PCF: If the state of the Priority PDU connectivity services is modified from disabled to enabled, the QoS Flow(s) controlled by the Priority PDU connectivity services are established/modified to have the service appropriate settings of the ARP and 5QI parameters, plus associated QoS characteristics as appropriate, using the PDU Session Modification procedure as specified in clause 4.3.3 of TS 23.502 [3].

-	PCF: If the state of Priority PDU connectivity services is modified from enabled to disabled, the QoS Flow(s) controlled by the Priority PDU connectivity services are modified from Priority PDU connectivity service appropriate settings of the ARP and 5QI parameters, plus associated QoS characteristics as appropriate, to those entitled by the UE as per subscription, using the PDU Session Modification procedure as specified in clause 4.3.3 of TS 23.502 [3].

-	SMF/AMF: Based on the QoS parameters (e.g. ARP, 5QI) of a PDU Session, the SMF may provide a PDU Session Priority to the AMF. The AMF stores the information for further PDU Session related priority handling (i.e. for determination of Message Priority header of subsequent message related to the PDU Session).

Invocation-related mechanisms for MPS for Data Transport Service:

-	MPS for Data Transport Service follows the same steps as those for Priority PDU connectivity services. The QoS Flows that will be subject to MPS for Data Transport Service are based on operator policy and regulations by means of local PCF configuration.

NOTE 2:	If no configuration is provided, MPS for Data Transport Service applies to the QoS Flow associated with the default QoS rule.

# * * * * End of changes * * * *