3GPP TSG-SA2 Meeting #167	S2-2501756

Athens, Greece, February 17 – 21, 2025	(revision of S2-250xxxx)

Source:	LG Electronics

Title:	Discussion on NR Femto cell shared by PLMN and PNI-NPN

Document for:	Discussion

Agenda Item:	19.12.2

Work Item / Release:	5G_Femto / Rel-19

Abstract of the contribution: This paper discusses how to support an NR Femto cell shared by PLMN and PNI-NPN, which is defined in RAN3 BL CR.

# Discussion

According to RAN3 BL CR to TS 38.300 (R3-247936), RAN3 agreed three types of cells as an NR Femto cell.

// From RAN3 BL CR to TS 38.300 (R3-247936) ### 4.x.4 Access Control Cells served by an NR Femto node may be deployed as part of a PNI-NPN (see clause 4.8) in order to restrict access to UEs according to the respective subscription. NOTE: The NR Femto node may use the CAG mechanism for PNI-NPN (see clause 16.7.4) as follows: - The NR Femto node may activate a PLMN cell, which can be accessed by legacy UE without access control of CAG. - The NR Femto may activate a cell shared by both PLMN and PNI-NPN, through broadcasting both the plmn-IdentityInfoList and the npn-IdentityInfoList-r16 in the SIB1, but without the cellReservedForOtherUse. Then, this cell is accessible to UEs which have the allowed CAG list including a CAG-ID broadcasted by the cell. For the legacy UE not supporting CAG, this cell is viewed as a normal PLMN cell. - The NR Femto node may activate an NPN-only cell by broadcasting the cellReservedForOtherUse IE with value “true”, then this cell can only be accessed by the UEs whose allowed CAG list includes a CAG-ID broadcasted by the cell.

The first bullet represents an NR Femto cell that is considered as a normal PLMN cell, which does not apply CAG access control mechanism. The third bullet represents an NR Femto cell that is a CAG cell and only UEs that have CAG capability and allowed by subscription can access the cell. These two types of cells and their access control mechanisms are defined in SA2 specifications.

The second bullet represents an NR Femto cell that is shared by PLMN and PNI-NPN. In this case, the cell is not only a CAG cell but also a normal PLMN cell which allows legacy UE (i.e. a CAG non-supporting UE) to access the cell.

Observation 1: RAN3 agreed an NR Femto cell shared by PLMN and PNI-NPN, i.e. a CAG supporting UE as well as a non-CAG supporting UE can access the NR Femto cell.

However, such cell (i.e., a cell shared by PLMN and PNI-NPN), is NOT supported according to following descriptions in TS 23.501 and TS 38.300.

// From TS 23.501 #### 5.30.3.4 Network and cell (re-)selection and access control The following is assumed for network and cell selection and access control: - The CAG cell shall broadcast information such that only UEs supporting CAG are accessing the cell (see TS 38.300 [27], TS 38.304 [50]); NOTE 1: The above also implies that cells are either CAG cells or normal PLMN cells. For network sharing scenario between SNPN, PNI-NPN and PLMNs, please see clause 5.18.

// From TS 38.300 ## 4.6 Radio Access Network Sharing NG-RAN supports radio access network sharing as defined in TS 23.501 [3]. If NR access is shared, system information broadcast in a shared cell indicates a TAC and a Cell Identity for each subset of PLMNs, PNI-NPNs and SNPNs. NR access provides only one TAC and one Cell Identity per cell per PLMN, SNPN or PNI-NPN. In this version of the specification, a Cell Identity can only belong to one network type among PLMN, PNI-NPN or SNPN as defined in TS 23.501 [3]. Each Cell Identity associated with a subset of PLMNs, SNPNs or PNI-NPNs identifies its serving NG-RAN node.

In addition, according to TS 24.501, when a UE access via CAG cell, the AMF should reject the CAG non-supporting UE.

// From TS 24.501 ##### 5.5.1.2.8 Abnormal cases on the network side The following abnormal cases can be identified: …… j) Based on operator policy, if the initial registration request from a UE not supporting CAG is rejected due to CAG restrictions, the network shall reject the initial registration with a 5GMM cause value other than the 5GMM cause #76 (Not authorized for this CAG or authorized for CAG cells only). ……

Therefore, current specifications do not support a shared cell (i.e., a cell shared by PLMN and PNI-NPN) scenario described in RAN3 BL CR and an AMF rejects registration request if the UE does not support CAG.

Observation 2: Current specifications do not support an NR Femto cell shared by PLMN and PNI-NPN.

Please note that in this paper, a shared cell means a cell shared by PLMN and PNI-NPN.

In order to support a shared cell, an AMF needs to know whether a cell is a shared cell or not. Otherwise, according to existing CAG behaviour, an AMF rejects registration of a CAG non-supporting UE when the UE access a shared cell as a normal PLMN cell. Currently, an AMF does not know whether a cell is a shared cell. Therefore, an NG-RAN may need to indicate whether a cell is a shared cell to an AMF. This requires impact on NG-AP interface and which procedure is used to indicate the information needs to be defined by RAN3.

Observation 3: In order to support an NR Femto cell shared by PLMN and PNI-NPN, an AMF needs to know whether a cell is a shared cell or not. This may require impact on NG-AP interface.

Proposal 1: Send LS to RAN3 to ask how the AMF knows whether an NR Femto cell is a cell shared by PLMN and PNI-NPN.

In EPS, CSG is used for access control of Femto cell (HeNB) and there is a hybrid cell, which is similar with an NR Femto cell shared by PLMN and PNI-NPN. When a UE accesses via hybrid cell, based on subscription of the UE, an MME indicates whether the UE is a CSG member to RAN and S-GW/P-GW/PCRF for differentiated treatment and charging between CSG and non-CSG members. See details in the following.

// From TS 23.401 #### 5.3.2.1 E-UTRAN Initial Attach …… 12. If an ESM container was not included in the Attach Request, steps 12, 13,14,15,16 are skipped. If the attach type is not set to "Emergency" or "RLOS", and the ESM container was included in the Attach Request, and the UE has indicated support for Attach without PDN Connectivity, and the network supports Attach without PDN Connectivity, and the PDN Connection Restriction is set in the subscriber data, then the new MME shall not establish PDN connection, and steps 12, 13, 14, 15 and 16 are skipped. …… If the subscription context does not indicate that the APN is for a PDN connection to an SCEF, the new MME selects a Serving GW as described in clause 4.3.8.2 on Serving GW selection function and allocates an EPS Bearer Identity for the Default Bearer associated with the UE. Then it sends a Create Session Request (IMSI, MSISDN, MME TEID for control plane, PDN GW address, PDN Address, APN, RAT type, LTE-M RAT type reporting to PGW flag, Default EPS Bearer QoS, PDN Type, APN-AMBR, EPS Bearer Identity, Protocol Configuration Options, Handover Indication, ME Identity (IMEISV), User Location Information (ECGI and TAI), UE Time Zone, User CSG Information, MS Info Change Reporting support indication, Selection Mode, Charging Characteristics, Trace Reference, Trace Type, Trigger Id, OMC Identity, Maximum APN Restriction, Dual Address Bearer Flag, the Protocol Type over S5/S8, Serving Network, APN Rate Control Status) message to the selected Serving GW. If Control Plane CIoT EPS Optimisation applies, then the MME shall also indicate S11-U tunnelling of NAS user data and send its own S11-U IP address and MME DL TEID for DL data forwarding by the SGW. User CSG Information includes CSG ID, access mode and CSG membership indication. …… 13. The Serving GW creates a new entry in its EPS Bearer table and sends a Create Session Request (IMSI, MSISDN, APN, Serving GW Address for the user plane, Serving GW TEID of the user plane, Serving GW TEID of the control plane, RAT type, Default EPS Bearer QoS, PDN Type, PDN Address, subscribed APN-AMBR, EPS Bearer Identity, Protocol Configuration Options, Handover Indication, ME Identity, User Location Information (ECGI), UE Time Zone, User CSG Information, MS Info Change Reporting support indication, PDN Charging Pause Support indication, Selection Mode, Charging Characteristics, Trace Reference, Trace Type, Trigger Id, OMC Identity, Maximum APN Restriction, Dual Address Bearer Flag, Serving Network, APN Rate Control Status) message to the PDN GW indicated by the PDN GW address received in the previous step. After this step, the Serving GW buffers any downlink packets it may receive from the PDN GW without sending a Downlink Data Notification message to the MME until it receives the Modify Bearer Request message in step 23 below. The MSISDN is included if received from the MME. …… 14. If dynamic PCC is deployed and the Handover Indication is not present, the PDN GW performs an IP-CAN Session Establishment procedure as defined in TS 23.203 [6], and thereby obtains the default PCC rules for the UE. If the UE is accessing over WB-E-UTRA, this may lead to the establishment of a number of dedicated bearers following the procedures defined in clause 5.4.1 in association with the establishment of the default bearer, which is described in Annex F. …… If the CSG information reporting triggers are received from the PCRF, the PDN GW should set the CSG Information Reporting Action IE accordingly. …… 17. If an APN Restriction is received, then the MME shall store this value for the Bearer Context and the MME shall check this received value with the stored value for the Maximum APN Restriction to ensure there are no conflicts between values. If the Bearer Context is accepted, the MME shall determine a (new) value for the Maximum APN Restriction. If there is no previously stored value for Maximum APN Restriction, then the Maximum APN Restriction shall be set to the value of the received APN Restriction. MME shall not deactivate bearer(s) with emergency ARP, if present, to maintain valid APN restriction combination. …… If the UE initiates the Attach procedure at a hybrid cell, the MME shall check whether the CSG ID is contained in the CSG subscription and is not expired. The MME shall send an indication whether the UE is a CSG member to the RAN along with the S1-MME control message. Based on this information, the RAN may perform differentiated treatment for CSG and non-CSG members.

Observation 4: Similar with an NR Femto cell shared by PLMN and PNI-NPN, a hybrid cell is used to allow access both for a membership UE and a non-membership UE.

Observation 5: For hybrid CSG cell support, an MME indicates whether a UE is a CSG member to a RAN and S-GW/P-GW/PCRF for differentiated treatment and charging between CSG and non-CSG members.

However, whether RAN3 intended to introduce the similar feature as CSG hybrid cell in 5GS is not clear, e.g. whether the NG-RAN and PCF need to know whether a CAG non-supporting UE is accessing a shared cell and perform differentiated handling between a CAG supporting UE and a CAG non-supporting UE, e.g. based on the AMF provided information. We believe that differentiated handling is required between a CAG supporting UE and a CAG non-supporting UE, if a shared cell is used. Otherwise, it seems that there is no difference between a normal PLMN cell and a shared cell.

Observation 6: An NR Femto cell shared by PLMN and PNI-NPN should be able to support differentiated handling between a CAG supporting UE and a CAG non-supporting UE similar to CSG hybrid cell supporting differentiated treatment for CSG and non-CSG members.

Proposal 2: Send LS to RAN3 to ask their opinion about differentiated handling between a CAG supporting UE and a CAG non-supporting UE in an NR Femto cell shared by PLMN and PNI-NPN.

# Proposal

Proposal 1: Send LS to RAN3 to ask how the AMF knows whether an NR Femto cell is a cell shared by PLMN and PNI-NPN.

Proposal 2: Send LS to RAN3 to ask their opinion about differentiated handling between a CAG supporting UE and a CAG non-supporting UE in an NR Femto cell shared by PLMN and PNI-NPN.

It is proposed to discuss draft LS is in S2-2501757.