SA WG2 Meeting #S2-167	S2-2501798

17 - 21 Feb, 2025, Athens, Greece

Title:	Reply LS on emergency call back and paging

Response to:	R2-2411041/ S2-2500044

Release:	Rel-17

Work Item:	NR_newRAT-Core, NR_redcap-Core

Source:	SA2

To:	RAN2

Cc:	CT1, RAN3

Contact Person:

Name:	Zhu Jinguo

E-mail Address:	zhu.jinguo@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	Spec: 23.501 CR6112

1. Overall Description:

SA2 thanks RAN2 for the LS on emergency services and eDRX (S2-2500044/R2-2411041). The following question is raised by RAN2 in that LS:

In RAN2#128, companies discussed a scenario where a UE is configured with a RAN eDRX cycle in RRC_INACTIVE state when the UE has a PDU session associated with emergency services. RAN2 notice the below description in TS 23.501 which states that an Idle eDRX cycle should not be used by the UE when it has a PDU session associated with emergency services. However, it is unclear whether it is possible for such UE to be released to RRC_INACTIVE state with a RAN eDRX cycle and if it is so how UE behavior may be.

#### 5.31.7.2 Extended Discontinuous Reception (DRX) for CM-IDLE and CM-CONNECTED with RRC-INACTIVE ##### 5.31.7.2.1 Overview ##### // skipped irrelevant part When the UE has PDU Session associated with emergency services, the UE and AMF follow regular discontinuous reception as defined in clause 5.4.5 and shall not use the extended idle mode DRX. Extended idle mode DRX parameters may be negotiated while the UE has PDU Session associated with emergency services. When the PDU Session associated with emergency services is released, the UE and AMF shall reuse the negotiated extended idle mode DRX parameters in the last Registration Update procedure. // skipped irrelevant part

Based on current specification, a RAN eDRX cycle can be configured by a RAN node when releasing the UE to RRC_INACTIVE state only if the UE is configured with an Idle eDRX cycle by CN.

RAN2 assumes that RAN node should be aware whether a UE has a PDU session associated with emergency services so that it can avoid releasing such UE to RRC_INACTIVE state with a RAN eDRX cycle, or even if it is so, there should be means for the RAN node and the UE to not use the configured RAN eDRX cycle. Regarding how RAN node can be aware of such PDU session, RAN2 understands that CN can already indicate this (e.g. via ARP value) but there is no requirement for CN to do this, so, there is case that RAN node may not be aware of this.

RAN2 would like SA2 and CT1 to consider the scenario described above, provide feedback regarding how the identified issue can be addressed and whether any standard solution is needed?

SA2 analysed the identified issue and would like to provide the following answer:

- When the PDU session associated with emergency service is established(with user plane activated or deactivated), the AMF remove the eDRX cycle in the RAN node so the RAN node will not use the eDRX cycle when the UE is released to RRC_INACTIVE state. Then the UE, RAN and AMF follow the regular discontinuous reception per TS23.501 clause 5.31.7.2.1.

- When the PDU session associated with emergency service is released, the AMF provides the eDRX cycle to RAN node when the UE is in connected state so the RAN node can use the eDRX cycle when the UE is released to RRC_INACTIVE state.

SA2 agreed the attached CR.

2. Actions:

To RAN2 group:

ACTION:  SA2 kindly requests RAN2 to consider answer above.

3. Date of Next SA2 Meetings:

SA2#168                                    7th – 11st April 2025	 		Goteborg, Sweden

SA2#169                                    19th – 23rd May 2025	 		Fukuoka, Japan