3GPP TSG-WG SA2 Meeting #167	S2-2501893

Athens, GR, 17th Feb – 21st Feb, 2025

Title:	[Draft] Reply LS on UUAA Unsubscribe procedure

Response to:	C4-245436, LS on UUAA Unsubscribe procedure

Release:	Rel-17

Work Item:	ID_UAS

Source:	SA2

To:	CT4

Cc:	CT3

Contact Person:

Name:	Guanglei Li

E-mail Address:	liguanglei3@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: TS 23.256 CR 0188 0189 0190

1.	Overall Description

SA2 thanks CT4 for the LS on UUAA Unsubscribe procedure.

As the subscription from AMF/SMF to USA NF is done when creating an authentication session, SA2 sees no need to define an explicit unsubscribe procedure when UE gets deregistered (clause 5.2.2.2 of TS 23.256) or the PDU session gets released (clause 5.2.3.2 of TS 23.256). AMF or SMF can notify UAS NF the UE deregistration or PDU session release event and the UAS NF can further notify USS. Then, UAS NF and USS can remove UUAA context based on the notification.

The principle is similar with the following description in Clause 4.3.2.3 of TS 23.502 for Secondary authorization/authentication.

During Secondary Re-authentication, if the SMF receives an indication from the AMF that the UE is unreachable then it informs the DN-AAA Server that UE is not reachable for re-authentication. Based on this indication from SMF, the DN-AAA Server may decide to keep the PDU Session or request to release the PDU session.

It is assumed that the subscription to deregistration event is implicitly performed by the USS and UAS NF when responding to the UUAA authentication request.

The subscription to PDU Session Status Event is performed by the USS and UAS NF after responding to the UUAA authentication request as described in step 6 of Figure 5.2.3.2 -1: UUAA during PDU Session Establishment.

The attached CR has clarified the principle mentioned above.

2.	Actions

To CT4:

ACTION: 	SA2 kindly asks CT4 to take above information into consideration.

3.	Date of Next TSG SA WG2 Meetings

TSG-SA2 Meeting #168		7th Apr – 11th Apr, 2025		Goteborg, SE

TSG-SA2 Meeting #169		19th May – 23rd May, 2025		Fukuoka City, Fukuoka, JP