3GPP TSG-SA WG2 Meeting #167	S2-2501895

Athens, Greece, 17-21 February 2025

Agenda item:	19.4.2

Source:	Huawei

Title:	Discussion on QoS Flow vs. Service Data Flow granularity

WI/Release:	EnergySys/Rel-19

Document for:	Discussion and Decision

# 1	Discussion

The current description of energy consumption calculation and exposure lists the QoS Flow granularity as the smallest granularity. This however has several problems which we would like to discuss in the following.

Problem #1: QoS Flow concept and awareness

In principle, a QoS Flow is established by the SMF when a PCC rule with QoS parameters (i.e. primarily 5QI and ARP) is provided or activated and there is no existing QoS Flow to which this PCC rule can be added/bound to (cf. clause 6.1.3.2.4 of TS 23.503). There are a few special cases mention which force the usage of a separate QoS Flow for a PCC rule but in general, it can be expected that PCC rules are often combined onto a QoS Flow when they have the same QoS requirements.

The QoS Flow concept and the logic of combining/separating PCC rules is however only known to the SMF. Neither the EIF nor the AF do know this (and not even the PCF has a full knowledge as the SMF is allowed to also separate PCC rules with the same QoS requirements that could be combined). It is therefore totally unclear how the EIF or the AF could refer to a specific QoS Flow. Those NFs can instead refer to a service data flow, i.e. the traffic belonging to a certain service or application which can be detected via (IP or Ethernet) packet filters or an application detection logic. And there is usually a PCC rule corresponding to every service/application that is of interest to the operator or the AF.

And since the QoS Flow will often transfer more than one service data flow (as described above), it cannot be assumed that the traffic of a service data flow is identical to the traffic of it’s QoS Flow. Only the default QoS Flow could be considered as often transferring all of the non-operator based service traffic (i.e. Internet traffic) but even for this QoS Flow, operator service traffic is not unlikely to happen, e.g. DNS or Background data traffic. Furthermore, non-operator based services may also be transferred with a different QoS than the default QoS and thus no longer run via the default QoS Flow.

Consideration #1: AF and EIF can only refer to a service data flow, i.e. the traffic belonging to a certain service or application which can be detected via (IP or Ethernet) packet filters or an application detection logic

Problem #2: User plane traffic detection and handling

The user plane traffic detection and handling takes place at the UPF and is controlled by the SMF which derives the N4 rules based on the PCC rules that are provided or activated (cf. clause 5.7.1.5 and 5.8.2 of TS 23.501). The N4 rules contain all the necessary information for performing the detection (in the PDR), the QoS enforcement (in the QER), the traffic counting (in the URR) and the traffic forwarding (in the FAR). The N4 rules do therefore ensure that the service data flow traffic is properly detected, policed (e.g. bitrate enforcement), counted (e.g. for charging purposes) and transferred via the right QoS Flow. And once a packet is detected by a PDR, it is then handled and further processed according to the other N4 rules that are related to this PDR.

In order to realize a traffic volume counting (which is required for the calculation of the energy consumption of that traffic), we need to ensure that the UPF is able to detect that service data flow traffic and count it properly (i.e. after the policing has taken place). As a similar requirement exists for collecting information for charging and usage monitoring purposes, the UPF and the N4 rules are already providing the required functionality. However, we have to integrate the traffic volume counting requirements from EIF with the other service data flow handling requirements coming from the PCC rules so that the UPF gets consistent and actionable instructions.

When the SMF receives an EIF request for traffic volume counting of a service data flow it should first check whether there is a PCC rule existing for this service data flow (by comparing the packet filters or application identifiers). If so, the SMF just needs to check whether there is already a URR in place which counts the traffic volume. If there is no such URR, the SMF has to generate one and provide it to the UPF for this PDR. If there is no PCC rule existing for this service data flow (which basically means that the service data flow is so far not of interest to the operator and will be handled by another PCC rule for aggregated traffic, most likely by the match-all PCC rule), the SMF should generate the N4 rules that are necessary for the detection and traffic volume counting of this service data flow. All other parameters (i.e. QoS, policing, forwarding) should be copied from the PCC rule which handled this traffic before so that the UPF treats this traffic in the same way as before.

Detecting and counting the service data flow traffic independently from the N4 rules is problematic and complex as the user plane handling instructions cannot be taken into account easily, e.g. if the UPF is performing a Session-AMBR enforcement of with a rather small value, a lot of the DL packets may be discarded by the UPF and should therefore not be counted. Furthermore, additional/new UPF functionality would be required while existing UPF functionality can already accomplish the traffic volume counting of a service data flow.

Consideration #2:  The traffic volume counting requirements from EIF have to be combined with the other service data flow handling requirements coming from the PCC rules so that the UPF gets consistent and actionable instructions.

# 2	Conclusions

Based on the above discussion it is proposed to replace the QoS Flow granularity with the Service Data Flow granularity for the exposure of the energy consumption information and it’s calculation.