3GPP TSG-WG SA2 Meeting #167	S2-2501999

17th – 21st February 2025, Athens, Greece	(revision of S2-250XXXX)

Source:	Lenovo

Title:	PCR to TS 23.xyz: AIoT Reader selection

Document for:	Approval

Agenda Item: 	19.14.2

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: This paper proposes to add description to clause 5.3 of the TS 23.xyz about AIoT Reader Selection.

# Introduction

This paper is prepared for the normative work of Ambient IoT, to add the descriptions about the AIoT Reader Selection according to the conclusions captured in the TR 23.700-13 V1.3.0 clause 8.3.4:

1).	Performs initial reader selection by either:

-	The AF provide information to identify readers to include in the initial reader selection network. The information can identify multiple or an individual Reader.

NOTE 1:	Which readers are identified by the information from the AF is up to the network deployment, configuration or implementation.

-	If a single UE Reader ID is provided by the AF via the NEF for the operation, then that is used as the selected Reader.

-	If reader selection information or UE Reader ID is not provided, then how the AIOTF determines which readers to use is based on implementation. The AIOTF may be e.g. preconfigured with which readers to use, or take the requested target AIoT Devices last known location into account, etc.

If no readers can be selected then the request is rejected.

# 2. Text Proposal

It is proposed to capture the following changes to 3GPP TS 23.xyz for Ambient IoT.

# * * * * First change * * * *

## 5.3	AIoT RAN node and AIoT Reader Selection

The AIOTF selects AIoT RAN node or AIoT Reader or both for an AIoT service operation. During a request for AIoT service operation, the AF may provide information to the AIOTF which is used by the AIOTF to select the AIoT RAN node(s) or AIoT Readers for the AIoT service operation. The AIOTF uses one or a combination of the following information to perform AIoT RAN node and/or AIoT Readers selection:

-	information received by the AF which may include either a target area for the AIoT service operation, AIoT RAN node(s) or AIoT Reader(s) identities.

-	locally configured information about the AIoT RAN node or AIoT Reader identities and optionally the corresponding serving area; or

-	information received from the AIoT RAN node during the NGAP association establishment over the Nx interface in case of direct interface between AIOTF and AIoT RAN. Such information may include the serving area of the AIoT RAN node.

If one or multiple AIoT RAN or AIoT Reader identities are provided to the AIOTF, the AIOTF uses the provided identities as the selected AIoT RAN node or AIoT Reader. The AF provided AIoT RAN or AIoT Readers identities take precedence over the locally configured information or information received from the AIoT RAN node.

If the AIOT RAN or AIoT Reader identity is not provided by the AF, the AIOTF selects the AIoT RAN based on the target area for the AIoT service operation. The AIOTF maps the target area for the AIoT service operation to the serving area of the RAN node(s) or AIoT Reader(s). The AIOTF is aware about the serving area of the AIoT RAN node from the NGAP association establishment via the Nx interface or it can be locally configured in the AIOTF.

The AIOTF may also consider the last known location of the target AIoT Device(s).

If the AIOTF cannot select any AIoT RAN node or AIoT Reader for AIoT service request, then the AIOTF rejects the AIoT service request.

# * * * * End of changes * * * *