SA WG2 Meeting #S2-167	S2-2502052

17 - 21 February, 2025, Athens, Greece	(revision of S2-250xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0697 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the location service when the MWAB is involved. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR_Ph2 |  |  |  |  |  | Date: |  |  | 2025-02-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The MWAB-gNB shall send the additional ULI if it send the ULI. So the AMF always send both ULI and additional ULI to LMF. It proposes to remove the “may”. Calrifying the additional ULI is specified in the 23.501. The AMF may provide “the serving cell identity belongs to a MWAB indication” to LMF in the Nlmf_Location_DetermineLocation service operation. It proposes to add this to how the LMF determine the serving cell belong to MWAB. The GMLC of the UE may retrieve the location information of the MWAB-UE via the GMLC or NEF in MWAB-UE is in the step 8, not step 9. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarifying the additional ULI. Adding the condition that LMF determines the serving cell belong to MWAB. Clarifying the GMLC of the UE retrieve the location information of the MWAB-UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent description. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.19.1, 6.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

## 5.19	Location Service involving Mobile Wireless Access Backhaul

### 5.19.1	General

A MWAB may have location service capability as specified in TS 38.305 [9] and participate in the location service of a UE. As the MWAB may be moving, the location service procedures need to be enhanced as following for an accurate estimation of the UE position:

-	The UE reports the cell IDs of all the cells the UE performed DL positioning measurements on.

-	The MWAB which performed the location service procedures for the UE includes its cell ID in the reported UL positioning measurement.

-	The AMF serving the UE provides the cell ID of serving cell of the UE and indicates, if possible, that the cell-ID belongs to a MWAB to the LMF in the location request. The AMF serving the UE also provide LMF with the additional ULI Information received from MWAB-gNB. The additional ULI Information is specified in the clause 5.49.4 of TS 23.501 [18].

-	Additionally, the LMF uses the serving cell identity belongs to a MWAB indication or reported cell IDs to derive whether the cell ID(s) corresponds to a MWAB. There can be multiple MWAB cells in the measurement report.

-	The LMF may also decide whether the cell ID(s) corresponds to MWAB(s) based on information received in a TRP information exchange i.e., that the cell-ID belongs to a MWAB and the UE-ID (GPSI) associated with MWAB.

-	To aid the LMF to improve the accuracy of the UE location estimation, the MWAB velocity information and time of obtaining its location measurement data should be obtained by the LMF when available. The LMF uses the received location and velocity of the MWAB(s) when estimating the location of the Target UE.

The reference architectures for MWAB involved location services in different scenarios are specified in Annex S of TS 23.501 [18].

Next CHANGE

### 6.1.5	5GC-MT-LR procedure involving MWAB

Figure 6.1.5-1 illustrates the network positioning requested by the LCS clients when MWAB(s) is involved.

In this scenario, it is assumed that the target UE may be identified using an SUPI or GPSI. The procedure follows the functionality in clause 5.19 and the 5G-MT-LR procedures in clauses 6.1.1 and 6.1.2. It is further assumed that:

-	The MWAB-gNB in figure 6.1.5-1 is a NG-RAN in figure 6.1.1-1 and 6.1.2-1.

-	When the MWAB-gNB is integrated to the 5GS, the OAM triggers the LMF to perform TRP Information Exchange procedure. The LMF learns that a new integrated TRP at a MWAB-gNB is mobile and its UE ID (GPSI) of the MWAB-UE via a TRP information exchange towards the MWAB-gNB with the Cell ID of the TRP. The LMF may also performs NRPPa TRP information exchange procedure before the positioning procedure for a target UE, e.g. to determine the position capability of NG-RAN.

-	The LMF that performs the location estimation of the MWAB (MWAB-UE) can be different (e.g. even in different PLMN) than the LMF that performs the location estimation of the target UE (not shown in figure 6.1.5-1).

Figure 6.1.5-1: 5GC-MT-LR procedure involving MWAB

1.	[Optional] The location services client sends a request to the (H)GMLC for a location for the target UE identified by an GPSI or an SUPI. The UE may or may not be served by a MWAB.

2.	The GMLC in MWAB Broadcasted PLMN invokes the Namf_Location_ProvidePositioningInfo service operation towards the AMF serving the target UE to request the current location of the target UE. The location procedure may be also triggered at the AMF for a 5GC-NI-LR as at step 1 in clause 6.10.1, or an event report sent by the UE for a deferred 5GC-MT-LR for periodic or triggered location as at step 25 in clause 6.3.1.

3.	The 5GC-MT-LR procedure as specified in clause 6.1.1 step 5-8 or clause 6.1.2 step 6-12 are performed with the difference that the LMF selection also consider LMF capability supporting MWAB involvement. If the AMF is aware that the target UE is served by a MWAB, it would select the LMF that can support the MWAB handling.

NOTE 1:	The step 4 to 11 may happen as part of step 3.

4.	LMF derives if any MWAB(s) is involved in the positioning of the target UE based on the cell-ID used for positioning measurements in the step 3. The AMF serving the target UE may indicate that the serving cell is an MWAB (if applicable). When the MWAB-gNB was integrated as a TRP, the LMF may determine from in a TRP information exchange procedure that the cell-ID belongs to the MWAB and/or the UE-ID (GPSI) associated with MWAB-UE. The LMF, when the OAM provides a new Cell Id to the LMF, performs a TRP information exchange as specified in TS 38.455 [15] for the new Cell ID. When the MWAB-gNB is integrated in the 5GS, the Cell ID(s) of the MWAB is (are)provided to the LMF. If the LMF selects any cell for UE positioning measurements, and the LMF determines it needs to perform a TRP information exchange with the cell, it shall perform TRP Information Exchange procedure as specified in TS 38.455 [15].As the MWAB can be mobile the LMF may need to determine an updated location of the MWAB by either performing step 5-7 (option 1) or performing step 8-10 (option 2) if option 1 is not feasible.

NOTE 2:	It is assumed that for the case of MWAB mobility, the OAM can timely update the Cell Id of the MWAB to the LMF so as to enable it to perform a proper TRP information exchange. Until the Cell Id of the MWAB is provided to LMF by OAM, the Cell of the MWAB is not used for location procedures.

5.	[Conditional] The LMF initiates a NRPPa TRP Information Exchange procedure to request the updated location information of the TRP(s) associated with MWAB(s) by directing the TRP information exchange messages to the MWAB-gNB associated with the Cell ID(s), for example, by setting the NRPPa TRP Information Type Item IE to "mobile TRP location info".

6.	[Conditional] The MWAB-UE may initiate a 5GC-MO-LR procedure (as defined in clause 6.2) to get its location information if requested by MWAB-gNB based on implementation. The MWAB-UE's location info is used by the co-located MWAB-gNB to determine the updated location for its TRP.

7.	[Conditional] The MWAB-gNB report its updated TRP location, e.g. TRP's geo-coordinate, velocity and the time for obtaining them, to the LMF.

8.	[Conditional] If the LMF invokes the Ngmlc_Location_ProvideLocation_Request service operation to the GMLC in MWAB Broadcasted PLMN by providing the identifier of MWAB-UE and a scheduled location time. The LMF determines the identifier of MWAB-UE based on the received the GPSI of the MWAB-UE in a TRP information exchange when the MWAB-gNB was integrated as a TRP. LMF may receive the scheduled location time in step 3. Otherwise, LMF may generate the scheduled location time, e.g. based on response time. The GMLC of the UE may further retrieve the location information of the MWAB-UE via the GMLC or NEF in MWAB-UE HPLMN which is responsible to retrieve the location information of the MWAB-UE via the GMLC in BH PLMN, based on the information retrieved from UDM of the MWAB-UE (e.g. serving AMF and GMLC address serving the MWAB-UE in BH PLMN).

9.	[Conditional] The GMLC in BH PLMN determines the AMF serving the MWAB-UE and steps 4-10 in clause 6.1.1 or steps 4-23 in clause 6.1.2 is performed. The privacy check in clause 5.4 is skipped for the MWAB based on the subscription data of the MWAB-UE. The scheduled location time received in step 8 is also provided to the MWAB-UE when sending a location request to the MWAB-UE.

10.	[Conditional] The GMLC in MWAB Broadcasted PLMN sends the location service response including the MWAB-UE's location to the LMF when the location response is received from the GMLC or NEF in MWAB-UE HPLMN.

11.	[Conditional] The LMF performs one of the positioning procedures with the target UE described in clauses 6.11.1 and 6.11.2. To reduce the timing offset of the positioning measurements, the UE positioning may be scheduled with the same scheduled location time as the MWAB-UE positioning in step 9. LMF sends the same time with the scheduled location time in step 8 to the target UE in clause 6.11.1. If Network Assisted procedure is used, the MWAB-gNB may provide the MWAB-UE updated location and velocity information and the time obtained them to the LMF via NRPPa message as defined in TS 38.455 [15].

12.	The LMF uses the received location and velocity of the MWAB(s) when estimating the location of the target UE together with the measurements reports in step 3 or optionally the updated measurement reports in step 11.

13.	The LMF returns the Nlmf_Location_DetermineLocation Response towards the AMF to return the current location of the target UE.

14.	The AMF returns the Namf_Location_ProvidePositioningInfo Response towards the GMLC to return the current location of the target UE.

15.	The (H)GMLC of the target UE sends the location service response to the location services client.

END OF CHANGES