3GPP TSG-SA WG2 Meeting #167	S2-2502117

# 17 – 21 February 2025, Athens, GR	                 (revision of S2-250XXXX)

Source: 	Ofinno

Title: 	PCR to 23.xyz: Assistance Information used by AIoT RAN

Document for: 	Approval

Agenda Item: 	19.14.2

Work Item / Release:	AmbientIoT / Rel-19

Abstract of the contribution: This paper proposes to include additional assistance information, an indication indicating a subsequent AIoT service, used by AIoT RAN.

# 1	Proposal

At the last meeting, following assistance information has been agreed to be provided to the RAN node, which are mainly to assist a resource management for the RAN node:

- AIoT service type (e.g., Inventory, Command)

- Optionally, approximate number of AIoT devices based on AF request.

- Optionally, approximate D2R message size based on AF request.

And, also, it was agreed to discuss other assistance information later if necessary.

For the AIoT services, there will be a scenario in which a set of AIoT services are progressed sequentially. For example, an inventory service may be followed by a command service. Also, a first command service may be followed by a second command service for a same AIoT device(s). For example, an AF requests an inventory and command service (e.g., reading) first and may request another command service (e.g., writing) based on the earlier command result.

Similar to the indication used for CIoT procedure, information indicating whether there is a subsequent procedure or not may help to minimize repeated physical layer procedures (e.g., RACH procedures) and may help for better control (e.g., DRX, resource allocation) of the RAN node and AIoT devices.

Proposal: it is proposed to include an additional assistance information indicating if the received AIoT service is standalone or subsequent AIoT service is followed, to the RAN node.

# * * * * First change (all text new)* * * *

## 5.4	Assistance information provided to AIoT RAN node

The AIOTF provides the assistance information to the AIoT RAN together with the service operation requests. The AIOTF determines the assistance information provided to the AIoT RAN based on the information received from the AF. The Assistance information is used by the AIoT RAN for performing service operations, e.g. radio resource allocation.

The following assistance information may be provided:

-	AIoT service type (e.g. Inventory, Command);

-	Optionally, approximate number of AIoT devices based on AF request;

-	Optionally, approximate D2R message size based on AF request;

-	Optionally, an existence of a subsequent AIoT service based on AF request.

Editor's note:		Other assistance information may be added later if necessary.

Editor's note:		How the AIOTF determines the assistance information (e.g. approximate number of AIoT Devices, approximate D2R message size) without input from the AF is FFS.

# * * * * End of Changes * * * *