3GPP TSG-WG2 Meeting #167	S2-2502120

# 17 - 21 February 2025, Athens, Greece

Source:	Ericsson

Title:	Progressing moderated discussion for “Industrial Network enhancements” features

Agenda item:	30.8

Document for:	Discussion/Agreement

# Introduction

In the moderated discussion for “Industrial Network enhancements” features, a number of comments have been made which call for further discussion. We provide comments in an effort to progress the discussion.

# Discussion

WT#1: Uncertain enqueueing and dejittering

Uncertain enqueueing happens due to packets coming from different ingress ports are enqueued at wrong order in an egress port due to the relatively large and unpredictable delay of the end-to-end 5G bridge delay. The source of this problem is the large range of packet delay variation intrinsic to the 5G system. To solve the problem the idea is trying to compensate for the source of the issue (i.e., the jitter). For instance, in TSN, if we can make the 5G delayMin = 5G delayMax, then the jitter becomes zero. If the hold and forward buffering is used to hold packet until they reach their deadline (i.e., 5G delayMax) then the packets will be set already in the right order. This is true even if the path from one ingress port and the path from another ingress port are defined to have different 5G delayMax. What is important is that the delay is constant within the system (with jitter=0 or almost constant with an extremely low jitter). The pipeline of packets will be kept in order at the egress since packets are forwarded based on their deadline. Therefore, it is necessary a a solution that can address the ordering issue induced by the 5G jitter and will be directly solved by extending the functionality of the hold and forward buffering mechanism.

WT#1: Slow TSN market uptake

Delaying this issue will further slow down the market uptake: 5G needs to deliver on time and/or provide precise periodic delivery. We see this issue limiting the use of 5G when the applications cannot de-jitter themselves, such as in industrial automation applications, and many other uses cases. It is an issue to implement TSN Qbv scheduling due to the relatively large 5G bridge jitters compared to very low jitter in wired bridges. This makes the scheduling not scalable because it would require temporal isolation of every transmission. Otherwise, the ordering issue will ruin a schedule from CNC.

WT#1: CQF study addition

We think CQF could be studied as a complement solution to hold and forward buffering mechanisms, with impacts in DS-TT and NW-TT.

WT#2: On DetNet

The idea is that an application can use TSC services to deliver traffic precisely without jitter, which may include but it is not limited to nor dependent on DetNet. This WT is proposed to allow such solution in general and not only for TSN. If TSC service is enriched with such de-jittering function, then DetNet could in principle use it as it is based on the TSC framework. As an alternative, WT#2 can focus only on TSC, and since DetNet is based on the TSC framework, then for example DetNet may use the feature as needed and if configured by operator.

WT#3: Loops in 5G-LAN. Loops in an Ethernet network cause broadcast storms making the Ethernet network unusable. Therefore, the approach should not be based on the detection of loops. Rather, Ethernet networks must prevent loops, in line with the approach defined in IEEE based on the spanning tree.

3GPP must support Ethernet traffic in such a way that it interworks well with IEEE based Ethernet networks that may be connected at the terminal or network side. Any solution that is not compatible with IEEE Ethernet has no market value. The IEEE solution for preventing loops is based on RSTP/MSTP. Specifically, the solution should be compatible with MSTP as required by IEC/IEEE 60802 (RSTP + traffic engineering). Hence, 3GPP support for loop prevention should be based on these protocols.

The loop prevention problem is a general Ethernet related functionality. Hence, the solution should be applicable for 3GPP Ethernet traffic, no matter if 5G-LAN function is used or not.

WT#4: Replication and elimination for redundancy. It has been raised to provide 3GPP native support for replication and elimination for redundancy, based on IEEE TSN FRER or IETF DetNet PREOF. This was raised in an earlier release, but was not prioritized; nevertheless 3GPP support could provide added value for operators.

While TSN FRER or DetNet PREOF could indeed be deployed as an over-the-top solution without explicit 3GPP support, such deployments are typically too complex for 3GPP operators, making it difficult for operators to offer such services on their own.

There can be different types of deployments, and we agree that a 3GPP operator based deployment for replication and elimination may need to interwork, maybe initially or at some point in the future, with wired deployments in the same domain. We do not exclude such deployments with interworking, and therefore we find it important to base the user plane functionality for replication and elimination on IEEE TSN FRER or IETF DetNet PREOF. There is no point for 3GPP to define a user plane format for replication and elimination that is different from the already standardized IEEE or IETF formats. In this sense, the user plane should not be “3GPP proprietary”.

In the control plane, however, we can leverage the already existing 3GPP control plane mechanisms in order to set up the replication and elimination mechanisms. By using 3GPP signalling to set up the replication and elimination, we make it easier for operators to provide the services on their own.

Note that the proposed mechanisms for replication and elimination are for critical traffic, e.g., in an industrial deployments. This makes the solution different from MASSS of Rel-19 (not concluded), which is not aimed at critical industrial traffic.

The study should address both single UE and multiple UEs per device cases. In industrial networks where redundancy is required, the cost of using and additional UE is not significant and well justified for critical traffic.

WT#6 and WT#7 as proposed by Siemens: New work tasks have been proposed as WT#6: Support of interoperable bridged time-sensitive networks profiled by IEC/IEEE 60802, and WT#7: Enhancements for 5GS bridge in integrated IEEE 802.1 industrial communication networks. However, the scope of these WTs appear to be rather broad and would imply a significant amount of effort, compared to the time budget that would currently be available. We therefore propose not to pursue these WTs further in this release.

WT-Nokia-1: It was proposed to study whether and how to implement different configuration domains to perform the priority regeneration according to IEEE Std 802.1Q-2022. However, the scope of these WT appears to be rather broad (e.g., due to implications on configuration) and would imply a significant amount of effort. The applicability of this work is limited to certain deployments only. We therefore propose not to pursue these WTs further in this release.