3GPP SA WG2#167	S2-2502122

Athens, Greece, February 17 – 21, 2025	(revision of xx-yyxxxx)

{Guidance text shown in curly brackets, in italics. All guidance text is to be deleted before WID submission.

Note that this form is to be used for all types of Work Item, covering both normative work and/or studies. A Work Item covering only studies is also known as "Study Item", as stated in TR 21.900.}

# Source:	Ericsson (Moderator)

# Title:	Study of Industrial Networks enhancements- New WTs

{"Revised" to be used only for WID already approved at plenary. For Revised WIDs, two versions have to be provided in a zip file: a clean one and one with revision marks showing the differences with the previously plenary-approved WID.}

# Document for:	Discussion

# Agenda Item:	30.8

Abstract: This document discusses the additional new WTs proposed during NWM discussion of this feature. This is designated as SI-2 in the moderator report and is for further discussion and decision on the way forward based on feedback from participating companies.

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	TBD

{Free text. It has to be the same as in the "Title:" section above. Studies have to start by "Study on"}

Acronym:	TBD

{Propose an acronym. Final acronym to be confirmed at the plenary. The sign "-" is a level separator between (Feature)-(Building Block)-(Work Task). The sign "_" can be freely used. Studies have to start by "FS_". Each acronym level has to be simple and short, 7 characters max recommended}

Unique identifier:

{A number to be provided by MCC at the plenary}

Potential target Release:	Rel-XX

{ Replace XX by the intended Release, e.g. Rel-19.  Note that this field indicates the proposed Release at the time of submission of the WID to TSG approval. It can later be changed without a need to revise the WID. The updated target Release is indicated in the Work Plan}

# 1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No |  |  |  |  |  |
| Don't know | X | X |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

{Tick one or more box(es). The full structure of all existing Work Items is shown in the 3GPP Work Plan in https://ftp.3gpp.org/Information/WORK_PLAN}

| X | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

{"Parent" Work Item refers to the related, earlier-Stage, Work Item, e.g. the related Stage 1 Work Item shall be indicated here when a Stage 2 normative Work Item or Study Item is presented. "Parent" Work Item can also refer to the related preceding Study Item e.g. the related Study Item and the earlier-stage Work Item shall be indicated here when a normative-work Work Items is started. List here all parent Work Items of which requirements are either fully or partially covered by the proposed Item. }

{This section is mandatory to be filled out by the rapporteur. This section is to be filled with care: it indicates to the companies monitoring the parent Work Item that it will be addressed in this study/work item.}

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

### 2.3	Other related Work Items and dependencies

{List here other Work Items which relate to the proposed one, such as a Work Item in an earlier Release if further enhancing the feature from the previous Release)}

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 820017 | Stage 2 of Vertical_LAN | Vertical_LAN |
| 910059 | Support of Enhanced Industrial IIoT | IIoT |
| 970011 | Stage 2 of Extensions to the TSC Framework to support DetNet | DetNet? |

Dependency on non-3GPP (draft) specification:

{This section is to be typically used to identify the IETF dependencies. Delete the header "Dependency on non-3GPP (draft) specification:" if no such dependency}

# 3	Justification

There are 3 new WTs proposed in this feature via the NWM, the following justifications were provided for them.

For WT# SI2-1 and WT# SI2-2:

The following 5G-ACIA’s identified gaps (as in SP-241819) regarding 5G System as Logical Bridge in

integrated IEEE/3GPP networks should be addressed to enable the deployment of 5GS Bridge with IEEE

802.1 and TSN based industrial communication networks:

IEEE 802.1 integration gaps and features

• support of available configuration functionalities for multiple configuration domains (MSTP as in IEC/IEEE

60802 (CIST, TE-MSTID), Priority Regeneration Table),

• support of industrial automation (IA) traffic types specified in IEC/IEEE 60802, for setup of QoS flows

for streams without PSFP configuration,

• reduced mobility functions when a TSN-enabled transport network (TN) is used in the 5G System,

• extensions for LLDP required by IEC/IEEE 60802,

• support of required IEEE 802.1ASdm functionality as required by IEC/IEEE 60802, and

• investigations on required scalability of number of QoS flows.

Netconf/YANG configuration gaps and features

• Support for VLAN configuration via TSN AF,

• Support for required functionalities of Multiple Spanning Tree Algorithm and Protocol (CIST, TE-MSTID)

as required by IEC/IEEE 60802,

• Support for Priority Regeneration functionality,

• Enhanced support of LLDP, i.e., handling of TLVs required by IEC/IEEE 60802,

• Enhancements to Netconf/YANG information and data model for 5GS exposure functionality if required.

For WT# SI2-3:

Frames that are received from a different configuration domain may use priority values that are reserved for TSN streams. To avoid interference with the traffic (especially TSN streams) in the configuration domain such priorities need to be downgraded as described in IEEE Std 802.1Q-2022, Annex U.2.

# 4	Objective

{Summary of what is intended to be achieved. Free text}

The new WTs require feedback on whether to continue discussion on their inclusion (one or more) or to focus only on the original WTs assigned at the SA plenary for 5G-A Rel-20 for further consideration. These WTs as proposed by the company are as follows:

WT# SI2-1: Support of interoperable bridged time-sensitive networks profiled by IEC/IEEE 60802, including:

- Architectural enhancements of 5GS that supports Netconf/YANG configuration

- How to translate IEC/IEEE 60802 TSN configurations (information and data model) to the TSN-related configurations within 5GS

- Enhancements of 5GS exposure to support Netconf/YANG configuration profiled by IEC/IEEE 60802

WT# SI2-2: Enhancements for 5GS bridge in integrated IEEE 802.1 industrial communication networks, including:

- Support of industrial automation (IA) traffic types specified in IEC/IEEE 60802, for setup of QoS flows

for streams without PSFP configuration

- Support for extensions for LLDP required by IEC/IEEE 60802.

WT# SI2-3:

Study whether and how to implement different configuration domains to perform the priority regeneration according to IEEE Std 802.1Q-2022.

# 5	Expected Output and Time scale

{If this WID covers both stage 2 and stage 3, clearly indicate the different completion dates.}

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| {Possible values: "TS" or "Internal TR" or "External TR". See Note 1} | {e.g. "22.XXX" or actual number if known} | {Title of the specification (as per TR 21.801 §6.1.1), to be aligned as much as possible with the WI/SI title} | {e.g. "TSG#87"} | {e.g. "TSG#89"} | {<FamilyName>, <GivenName>, <Company>, <email address>. See Note 2} |

{Note 1:	Only TSs may contain normative provisions. Study Items shall create or impact only TRs.
"Internal TR" is intended for 3GPP internal use only whereas "External TR" may be transposed by OPs.}

{Note 2:	The first listed Rapporteur is the specification primary Rapporteur. Secondary Rapporteur(s) are possible for particular aspect(s) of the TS/TR. In this case, their responsibility has to be provided as "Remarks".}

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| {e.g. "22.281"} | {Possible values: - either free text (e.g. “CS aspects to be removed") - or “Specification to be withdrawn”} | {e.g. "TSG#89"} | {Free text, e.g. "This TS covers Stage 2" or "This TS covers Stage 3" or "This TS covers both stages 2 and 3"} |

# 6	Work item Rapporteur(s)

{Mandatory: <FamilyName>, <GivenName>, <Company>, <email address>}

{Optional: <FamilyName>, <GivenName>, <Company>, <email address>: Secondary task(s)}

{The first listed Rapporteur is the work item primary Rapporteur. The role of a Rapporteur is further described in www.3gpp.org/specifications-groups/delegates-corner/writing-a-new-spec. By default, the primary Rapporteur shall ensure the production of the post-completion summary. 
Secondary Rapporteur(s) are possible for specific secondary task(s), such as: "Write the post-completion summary"; "In charge of a specific aspect of the work item (specify which)"; "Rapporteur for a secondary responsible WG (specify which)"}

# 7	Work item leadership

{One Working Group, e.g.: "SA4". Exceptionally a TSG}

{Secondary responsible Working Group(s) are possible. In this case, list them here}

# 8	Aspects that involve other WGs

{This information is provided as best effort assumption, at the time of submission of the WID to TSG approval. It can be later changed without a need to revise the WID.

The “aspects” can be provided by topic (e.g. “security”, “multimedia”) and/or by specifying the WG(s) e.g.: "SA2, SA3, SA5, SA6. CT6 for storage, and potentially SA4". If not applicable, indicate "None" or "None identified yet"}

For a Stage 2 WID requiring Stage 3 to be done by another group: on a best-effort basis, indicate which potential WG is expected to specify the Stage 3: {possible values: "Not applicable", " unknown", "CT WGs", etc}

# 9	Supporting Individual Members

{At least 4 supporting Individual Members are needed. There is an expectation that these companies will provide resources to progress the work. Note that having 4 supporting companies is a necessary but not sufficient condition: the usual TSG approval process by consensus is needed for the WID approval}

Supporting IM name