3GPP TSG-SA2 Meeting #167	S2-2502271

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5311 | rev | 2 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Complete the details for KI#2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Align the Nnef_ImsSessionManagement service with Nimsas_ImsSessionManagement service. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Calling and called party IDs are removed from Nimsas_ImsSessionManagement_Update request, since the session is identified by session ID. The MDC1 media endpoint address is added to the Nimsas_ImsSessionManagement_Update and Nimsas_ImsSessionManagement_Create requests. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | KI#2 is not implementable. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.6.40.2, 5.2.6.40.3, 5.2.6.40.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************************** Start of Changes ***************************

##### 5.2.6.40.2	Nnef_ImsSessionManagement_Create operation

Service operation name: Nnef_ImsSessionManagement_Create

Description: This service enables the consumer NF to create an IMS session with the requested media (e.g. standalone data channel media).

NOTE:	Only data channel media is supported in this release.

Inputs, Required: Calling ID, Called ID, Media information set.

Calling ID: It is the public identity of calling IMS Subscriber or the identity of the consumer NF, e.g. DC AS.

Called ID: It is the public identity of called IMS Subscriber.

Media information set: Includes a set of media information indicating how the media used in the created session. Each media information contains:

-	Media Correlation ID: It identifies the media component to be created. It is allocated by the consumer NF.

-	Media Type: It indicates the media type used in the created session, e.g. Data Channel.

-	Media Parameter set: It indicates the related media parameters. For different media, it contains different parameters:

-	When the Media Type is Data Channel, it includes:

-	DC Type: indicates the Data Channel Type (ADC or BDC);

-	When DC Type is ADC;

- 	ADC Type: the ADC type (P2A, P2P, P2A2P);

-	App Binding Information,;

-	Target ID(s): the public identity(ies) of the IMS Subscriber the DC is targeted;

NOTE: For P2P ADC, the Calling ID and Called ID of the session are used as the Target IDs.

-	If PA2 or P2A2P is selected, the MDC2 media endpoint address(es) of the application layer is included.

-	When DC Type is BDC;

- 	Target ID: the public identity of the IMS Subscriber the DC is targeted

-	the MDC1 media endpoint address.

Inputs, Optional: Notification Target Address and Notification Correlation ID.

If the request includes a Notification Target Address, the request creates an implicit subscription to be notified for the events in Table AA.2.4.4.5-1 of TS 23.228 [55].

Outputs, Required: Result indication.

Outputs, Optional: Session ID.

The Session ID is included if available. The Session ID can be reported also in the Nnef_ImsSessionManagement_Notify service operation.

##### 5.2.6.40.3	Nnef_ImsSessionManagement_Update service operation

Service operation name: Nnef_ImsSessionManagement_Update

Description: This service operation enables the consumer NF to modify the data channel media in a specific IMS session, such as adding or removing bootstrap data channel(s) or application data channel(s), modify media parameters for the targeted data channel(s).

Inputs, Required: Session ID, Media operation set.

Session ID: It is the identity of session that this request targeted to.

Media operation set: Includes a set of media operations commands indicating how to modify the media used in the specific session. Each media operation command contains:

-	Media Correlation ID: It identifies the media component to be created.

-	Operation Type: It indicates adding, updating or removing the corresponding media.

-	Media Type: It indicates the type of operated media (e.g. Data Channel).

-	Media Parameter set: It indicates the related media parameters. For different media, it contains different parameters:

-	When the Media Type is Data Channel, it includes:

-	DC Type: indicates the Data Channel Type (ADC or BDC);

-	When DC Type is ADC:

- ADC Type: the ADC type (P2A, P2P, P2A2P);

- 	App Binding Information;

- Target ID: the public identity of the IMS Subscriber the DC is targeted;

-	If P2A or P2A2P is selected, if the Operation Type indicates adding or updating the media, the MDC2 media endpoint address of the application layer is indicated.

-	When DC Type is BDC:

- Target ID: the public identity of the IMS Subscriber the DC is targeted

- 	if the Operation Type indicates adding or updating the media, the MDC1 media endpoint address.

Inputs, Optional: Notification Target Address and Notification Correlation ID.

If the request includes a Notification Target Address, the request creates an implicit subscription to be notified for the events in Table AA.2.4.4.5-1 of TS 23.228 [55].

Outputs, Required: Result indication.

Outputs, Optional: None.

*************************** Next Change ****************************

##### 5.2.6.40.5	Nnef_ImsSessionManagement_Notify service operation

Service operation name: Nnef_ImsSessionManagement_Notify

Description: Provides the subscribed event information to the NF Consumer.

Inputs, Required: Event ID as described in Table AA.2.4.4.5-1 of TS 23.228 [55], Notification Correlation ID.

Inputs, Optional: Session ID, Media resource information

The Session ID is included if available. The Session ID may not be available e.g. when session establishment failure is reported.

edia resource information set includes entries corresponding to each media. Each entry in the set includes the Media correlation ID and DC Media Specification. The DC Media Specification shall exist only with media type "DC" and contains additional media resource information, e.g. remote media address for MDC2 interface.

Outputs, Required: Result indication.

Outputs, Optional: None.

*************************** End of Changes ****************************