3GPP TSG SA WG2 #167	S2-2502438

17-21 February, 2025, Athens, Greece	(revision of S2-2502197

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1537 | rev | 3 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#8: Update to Architecture for avatar communications |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-02-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Lack of clarity on some aspects. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the ways in which MF fetches the Avatar representation, directly from the BAR through redirection from the DC AS or indirectly via the DC AS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inaccurate specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.11.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## AC.11.2	Architecture

Figure AC.11.2-1: Architecture to support Avatar communication

To support Avatar communication, the data channel architecture and functions described in clause AC.2 is enhanced as below:

Base Avatar Repository (BAR):

-	The BAR stores Avatar Representation, its Avatar ID, and the associated information as described in clause AC.11.1. One or more Avatar Representation(s), is stored for a user, and each Avatar Representation is identified with Avatar ID.

-	The BAR may provide Avatar ID List to the DC Application Server, and the DC Application Server may further provide to DCSF.

-	The BAR may provide Avatar Representation and the associated information to the DC AS directly or to the MF directly after URL redirection from the DC AS.

DCSF:

-	If the Avatar ID List is downloaded through bootstrap data channel, the DCSF may fetch the Avatar ID List from the DC AS, and send to MF via MDC1.

MF:

-	The MF enables downloading the Avatar ID List and associated information through bootstrap data channel or application data channel to the UE.

The MF supports the UE downloading the Avatar Representation through bootstrap data channel or application data channel.

-	The MF may retrieve the Avatar Representation and the associated information from BAR directly via MDC2 when redirected by the DC AS. Optionally, the MF may retrieve the Avatar Representation and the associated information from BAR indirectly via the DC AS.

-	The MF may support Avatar media processing and transcoding (e.g. ASR/TTS).

DC AS:

-	The DC AS may provide the DC AS URL(s) to DCSF via DC3/N33 or DC4 for downloading of Avatar ID List through bootstrap data channel.

-	The DC AS retrieves the Avatar ID List from BAR(s) via MDC2.

-	The DC AS sends the Avatar ID List either to DCSF via MDC3 for downloading through bootstrap data channel, or to MF via MDC2 for downloading through application data channel.

-	The DC AS may retrieve the Avatar Representation and the associated information from BAR and sends them to MF via MDC2, or redirect the MF to retrieve the Avatar Representation and the associated information from BAR directly, via a URL.

-	The DC AS supports Avatar media negotiation and provides the instruction for Avatar media processing.

# * * * End of changes * * * *