3GPP TSG-WG SA2 Meeting #167	S2-2502556

17th – 21st February 2025, Athens, Greece	(revision of 2502339)

Source:	Lenovo, Huawei, HiSilicon, CATT, Ericsson

Title:	pCR to TS 23.xyz: AF authorization to the Ambient IoT Services

Document for:	Approval

Agenda Item: 	19.14.2

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: This paper proposes to add the support of AF Authorization to Ambient IoT Services for 3rd party AF.

1	Discussion

This paper aims to capture the support of AF authorization to the Ambient IoT services, based on the latest agreed conclusion principles in the TR 23.700-13.

2. Text Proposal

It is proposed to capture the following changes to TS 23.xyz for Ambient IoT.

# * * * * First change * * * *

5.6	AF authorization

The information needed to support the authorization of the AF for performing the Ambient IoT service is stored as the authorization data for 3rd party AF in the ADM, or locally configured in the NEF.

Table 5.6-1 below describes items stored as AF authorization data for the AIoT.

Table 5.6-1: AF Authorization Data for AIoT

| AF Authorization Data | Description |
| --- | --- |
| AF ID | Identifier used to identify the AF. |
| Allowed area | Indicate the allowed area for the indicated AF to perform the AIoT services operations. |
| Allowed service operations | Indicate the allowed service operation (s) for the indicated AF, e.g., inventory, read, write, permanent disable. |
| Allowed target AIoT devices | Indicate the allowed AIoT Device(s) for the indicated AF. |

The authorization to the AF for the Ambient IoT mainly includes two parts:

- NEF performs AIoT AF request authorization based on the service layer agreement (SLA) between the 3rd party AF and the 5GS, the operator policy and local configuration as in TS 33.501 [X].

- AIOTF may perform authorization of AIoT service requested by the AF, by retrieving the AF authorization data stored in the ADM as described in above Table 5.6-1.

Editor’s Note: Whether to perform AF authorization only in NEF or in both NEF and AIOTF (including which parameter to use in Table 5.6-1), and whether a trusted AF requires authorization is FFS.

# * * * * End of changes * * * *