3GPP TSG-WG2 Meeting #167	S2-2501486

# 2025-02-17 – 2025-02-21, Athens, Greece

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0672 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedure to collect input data for ML Model training at NWDAF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-02-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Proposal 1: The ground truth data (and the quality) are added to the procedure to collect input data from LMF. Removal of Editor´s Note in clause 6.22.4: Editor's note: Whether the NWDAF needs to be aware of PRU information and indicate to obtain the PRU information from LMF is FFS. Whether the NWDAF needs to indicate Data source type (UE or RAN measurements) and/or data type(s) and/or a target ground truth label precision is also FFS. Proposal 2: The NWDAF can indicate that it has a model that needs input data to train it, as such the NWDAF can indicate if the model requires data from RAN or from the UE and in which area the model is valid. Other information such as data type is not known by LMF or NWDAF, so far RAN1 has not informed SA2 that different input data can be requested by the LMF, just NG-RAN or the UE provides input data to LMF. The same happens to the ground truth that is collected by LMF. If the LMF or NWDAF receives input data or ground truth data with low level of accuracy is upto LMF or NWDAF to use it for model training or not. The proposal is that LMF collects the data and provides to the NWDAF; while any filtering of the input data should happen in the NWDAF, that as such will not depend on the LMF capability to do any filtering of the input data that is requested by NWDAF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update clause 6.22.4 based on Proposal 1, i.e., add ground truth data, define the input parameters that NWDAF can provide in Nlmf_DataExposure service. Update clause 6.22.4 based on Proposal 2, i.e. define input data that NWDAF can collect. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | LMF-based AI/ML positioning is not fully specified in Rel19 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.22.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************** 1st change **************

### 6.22.4	Input data collection by NWDAF for AI/ML positioning ML model training or ML model performance monitoring

The NWDAF containing MTLF may subscribe to input data (i.e. location measurement related data) from LMF for ML model training or ML model performance monitoring for LMF-based AI/ML Positioning.

Figure 6.22.4-1: Procedure of input data collection from LMF

1.	NWDAF containing MTLF determines to train a ML model for LMF-based AI/ML Positioning based on the request from LMF or internal trigger, or the NWDAF containing MTLF determines to perform ML model performance monitoring for LMF-based AI/ML Positioning.

2.	The NWDAF invokes an Nnrf_NFDiscovery_Request service operation to an NRF to discover an LMF, the service operation includes an AoI and the support for Nlmf_DataExposure service as discovery parameters. The NWDAF may include whether it wants to collect the input data  and ground truth data, the NWDAF may also request input data and ground truth data for PRUs, then will include the PRU existence indication for discovering the LMF(s) associated with PRUs (the PRU association procedures are defined in clause 6.17). The NRF selects one or more LMFs based on the AoI, the support of the Nlmf_DataExposure service, and the PRU existence indication (if available) in the request and sends an Nnrf_NFDiscovery_Request Response which includes the profiles of the selected LMFs to the NWDAF.

Editor's note:	The input data used for LMF-based AI/ML Positioning will be decided by RAN WGs.

3.	The NWDAF subscribes to or cancels subscription to input data and to ground truth from LMF by invoking Nlmf_DataExposure_Subscribe / Nlmf_DataExposure_UnSubscribe service operation. The NWDAF includes an AoI and a notification target address to request the input data from LMF. The NWDAF may also include the positioning case and a time window for data collection.

4.	The LMF performs the procedure for data collection  in clause 6.22.2, or  in clause 6.22.3 depending on the positioning case requested in step 3.

5.	The LMF sends the collected data samples to the NWDAF by invoking Nlmf_DataExposure_Notify service operation, then the NWDAF trains the ML model or performs ML model performance monitoring based on the input data and the ground truth data. The NWDAF unsubscribes to data collection from LMF when the time window for data collection expires.

************** End of changes **************