3GPP TSG-WG SA2 Meeting #167	S2-2501554

# Athens, Greece, 17th Feb – 21st Feb, 2025		(revision of S2-240xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6080 | rev | - | Current version: | 19.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on CAG information provisioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_Femto |  |  |  |  |  | Date: |  |  | 2025-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | CAG owner and authorized administrator are introduced in 5G Femto. A CAG owner or an authorized administrator can control subscribers allowed to access CAG cells via external parameter provisioning procedure. However, this procedure is triggered by AF. A clarification is needed for the association between the AF and the CAG owner or authorized administrator. There is an editorial error in the annex, the NF Femto should be changed into NR Femto. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify the CAG owner or authorized administrator may provide CAG ID and UE ID to the AF in order to trigger the external parameter provisioning procedure. - Editorial change for terminology. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - It is unclear how the CAG owner or the authorized administrator using the CAG provisioning procedure. - The terminology is wrong. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.50.3, Annex V.0 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.50.3	CAG Information Provisioning

In 5G, a CAG owner or an authorized administrator can control/provision subscribers allowed to access CAG cells via the AF using the provisioning of CAG Information as defined in clause 4.15.6.2 and 4.15.6.3h of TS 23.502 [3].

In this Release of the specification, CAG Information Provisioning is only supported for non-roaming (i.e. only applicable to HPLMN).

# * * * * Second change * * * *

V.0	General

This annex provides information on how the NR Femto can be deployed and used by means of functionalities defined in clause 5.50 and existing functionalities.

# * * * * End of changes * * * *