3GPP TSG-SA WG2 Meeting #167	S2-2501556

Athens, Greece, 17-21 Feburary 2025

Agenda item:	19.4.2

Source:	Huawei

Title:	Discussion on Energy Saving Indicator

WI/Release:	EnergySys/Rel-19

Document for:	Discussion and Decision

# 1	Discussion

At SA2#167 it was agreed to introduce in the 5GS an Energy Saving Indicator that is stored in the UDM as AM subscription data and can be provided to the NG-RAN and to the PCF.

In this discussion paper we propose that:

- The change of the name of the Energy Saving Indicator

- The introduction that AMF send the Energy Saving Indicator under the control of PCF

## 1.1 Name of the Energy Saving Indicator

The Energy Saving Indicator is an indicator that is send to the gNB related to the specific UE and not as a generic indicator that applies to all UEs. The current name gives the impression that is generic, from the point of view of people participating to the discussion the meaning is crystal clear, but when we move out of Energy-people the risk of misunderstanding is not negligible. Therefore, we propose the change of name to “UE Energy Saving Indicator” (UESI) to clear indicate that is an indicator per UE. Furthermore, it needs to be clarified the definition as follows:

“UE Energy Saving Indicator”: this parameter indicates that operations aiming to save energy are allowed for the UE.

## 1.2 Energy Saving Indicator under the control of PCF

In the current specification, there is no principle described for the handling of UEs with this indicator, in particular, when and how often RAN energy saving shall apply. Without such principle, the AMF forwards the Energy Saving indicator for all these UEs to NG-RAN and NG-RAN may often apply energy saving actions for these UEs. This can easily lead to situations where some UEs are consistently impacted and thus unequally treated compared to other UEs.

Figure 1.2-1: Example when UE are experiencing unequal treatment due to UESI.

We proposed to manage the control under EIF, but this is no more possible due to lack of interface between EIF and AMF. The issue of an unconditional and permanent distribution in the network and especially towards the RAN nodes. It is quite likely that there are end users which are often connected to RAN nodes in energy saving mode and thus will experience a certain degradation of service while other end users may rarely be connected to such RAN nodes. In consequence, some end users are quite unhappy with their contract (containing the Energy Saving indicator) and will change the contract or even the operator. And the RAN node will then have less UEs available for reducing the energy consumption when necessary. So, instead of an unconditional and permanent forwarding, the PCF is the second good position (we prefer EIF) to take QoS decision involving both the PCC rules and whether to send the Energy Saving Indication. In fact, the PCF may decide to send both or to send only the indicator or change the PCC rules for a given UE in order for example to avoid or to allow to apply double traffic restrictions the first or to control the traffic restriction to a given level setting a suitable QoS that will be not degraded too much by action taken by gNB.

Figure 1.2-2: Example when PCF takes decision whether to send the UESI.

The PCF performs the control sending an indication to the AMF via AM Policy control which enable the AMF to send the Energy Saving indicator via N2 to the gNB where the UE is connected to or not.

## 1.3 Single value vs multiple value

We would like to resume our comments provided during the e-meeting why we do not see needed at this stage to define more values:

- Different value interpreted as vendor specific in case of multi-vendor network requires a great effort for operator to provide a set of value that are interpreted consistently by different vendors.

- Different values assumes that different scenarios and actions are possible for different UEs and gNBs, but it is still questionable the benefit that can be obtained and the difference with using a single value that can be in any case interpreted differently.

- The Energy Saving Indicator as a such is pending to the RAN answer on whether is considered and whether is used, therefore to add more complexity for something that is still uncertain is not for which the benefit cannot evaluated, it is not a good idea at this stage of the work, which is assumed to be concluded.

# 2	Conclusions

It is proposed that the Proposal here below are agreed.

Proposal#1: We propose the change of name to “UE Energy Saving Indicator”.

Proposal#2: The document proposes a conditional forwarding of the Energy Saving indicator of a UE based on PCF control toward AMF to indicate whether the indicator shall be sent or not to gNB. The PCF send the control information within the AM Policy.