3GPP TSG-WG SA2 Meeting #167	S2-2501585

Athens, Greece, February 17 - 21, 2025	(revision of S2-250xxxx)

Source:	Ericsson

Title:	KI#1#3 Conclusion Update of Aggregation

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract: This pCR discusses and updates KI#1 and KI#3 conclusions to resolve the EN about Aggregation.

1. Introduction

In SA2#166 and SA2#166AHE, regarding the aggregation of the Ambient IoT service operation results, there were no consensus, and thus the EN (How the aggregation can be done is FFS) hasn’t been resolved.

In General parts of interim conclusion on KI#1:

d.	The AIoTF aggregates the service operation results (including the removal of the duplicated devices records) from BS Readers and UE Readers and sends to AF.

……

Editor's note:	For RRC based solution of topology 2, whether the aggregation can be performed by the RAN is FFS and coordination with RAN WGs is needed.

Editor's note:	How the aggregation can be done is FFS.

In principles on the procedure to support AIoT services of conclusion on KI#3:

5).	The Reader executes the inventory request, reporting AIoT specific NAS message responses from the AIoT Device to the AIOTF, including its Reader ID and correlation identifier from the AIOTF. The Reader may aggregate results from multiple AIoT Devices in the responding messages. The AIOTF can determine which request the results are for using the correlation identifier.

……

9).	Provide the results of the operation to the NEF. Results from multiple AIoT Devices may be included/aggregated in the service response(s).

According to the conclusions, the following observations can be found:

[Observation-1] The reader may perform aggregation.

[Observation-2] The AIOTF performs aggregation.

[Observation-3] The aggregation is to aggregate results from multiple sources, including the removal of duplicated records.

In TR 38.769, the following figure depicts the logical system architecture for topology 1:

And in TR 38,769, it is also agreed that one AIoT RAN node may support one or more readers:

In Topology 1, an A-IoT RAN node may serve one or more readers.

[Observation-4] Besides the AIOTF and the reader, the AIoT RAN node is another function which may perform aggregation. It is up to RAN3 to determine whether the AIoT RAN node aggregate the results from readers or not.

In our view, regarding how the aggregation is performed, it can be implementation specific on whether to enable the aggregation, and how the aggregation is performed. The further aggregation assistance information (e.g., aggregation period or completion time) would be helpful for the coordination of aggregations among the AIOTF, the AIoT RAN and the readers. But they are not absolutely needed.

[Proposal-1] Resolve the EN and clarify how the aggregation is performed is based on implementation.

However, it is important to understand whether the Inventory Response and Command Response can be aggregated in each function, considering the inventory and command use cases.

Inventory:

In inventory use case, the “inventory-only” procedure in TR 38.769 is executed. As there are no following procedures to be triggered, in this case, the Inventory Responses from the devices can be aggregated by the reader, the AIoT RAN node and the AIOTF.

Command:

In command use case, the “inventory and command” procedure in TR 38.769 is executed, considering the “command-only” is not supported in Rel-19.

- In inventory round, we assume that the reader has limited possibility to aggregate device responses, while it is possible that the AIoT RAN node aggregates the results from multiple readers. However, for topology 1, this concerns RAN internal functionality, so this is less of concern from SA2 point of view. And in the AIOTF, as the Inventory Response is not forwarded to the AF, the aggregation is not applicable.

- In command round, the reader may or may not aggregate Command Response depend on its implementation, while the AIoT RAN node may aggregate the results from multiple readers. And the AIOTF can perform the aggregation.

[Proposal-2] Clarify the aggregation mechanism as described above.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-13 v1.3.0:

* * * Start of Change * * * *

### 8.1.1	General

Key issue #1 includes the following aspects:

-	System architecture identified along with the solutions for KI#2 and KI#3.

Key issue#2 aspect on "Ambient IoT Device subscription management" and key issue#3 aspect on "Ambient IoT service exposure" is considered in this clause.

In this Release, normative work will take place for Topology 1 and no normative work will take place for Topology 2.

The following aspects common for Topology 1 and Topology 2 are concluded as principles for normative work:

1.	A new core network function is introduced to support Ambient IoT (e.g. AIOTF) service for both the topology 1 and topology 2. The AIoTF performs the following functionality.

a.	The AIOTF registers itself in the NRF with its NF profile, this is to enable the discovery of AIOTF instances e.g. by an NEF. The NF profile at least includes the AIOTF ID/address, NF type, information used to allow the NEF to discover AIOT instances, e.g. based on the target area information in AF request. Other details of the NF profile, if needed, will be completed in normative phase.

b.	For topology 1, the AIOTF selects AIOT RAN nodes and optionally a list of the BS readers. For topology 2, the AIOTF selects the UE readers (e.g. candidate or final UE readers) and provides the selected UE Reader list to the RAN.

NOTE 1:	Providing the UE Reader list to the RAN only applies to the RRC-based option.

For topology 1, NGAP is terminated at an AIOT RAN node. An AIOT RAN node that supports one or more BS readers, may report its supported BS reader information e.g. BS reader IDs and their corresponding service areas. The OAM may configure the AIOTF with BS Reader information including e.g. their corresponding service area when the BS Readers are integrated in the network.

NOTE 2:	Whether RAN3 will specify BS reader information (e.g. BS reader ID, service area) reporting to AIOTF over NGAP depends on RAN3 decision and if so SA WG2 may include this feature in SA WG2 specifications.

NOTE 3:	If the request from the AIOTF to the AIOT RAN does not include a BS reader list, then AIOT RAN can use all available BS readers.

NOTE 4:	The BS reader’s service area needs to coordinate with RAN WG(s).

c.	The AIOTF receives an AIoT service request from the AF and triggers the BS/UE Readers to perform AIoT service operations towards the AIoT Devices(s).

d.	The AIOTF aggregates the service operation results (including the removal of the duplicated devices records) from BS Readers (via A-IoT RANs) and UE Readers and sends to AF. How the aggregation is performed is based on implementation. For inventory, the AIOTF aggregates the service operation results from Inventory Reports from one or more A-IoT RAN nodes. For command, the AIOTF aggregates the service operation results from Command Responses from one or more A-IoT RAN nodes. The aggregation in the AIOTF is not applicable for the Inventory Reports in inventory round when “inventory and command” applies.

NOTE X:	It is up to RAN3 to decide whether the A-IoT RAN node aggregates the service operation results from multiple readers or not.

e.	The AIOTF may provide the following assistance information to AIoT RAN/UE Reader:

-	AIoT service type (e.g. Inventory, Command);

-	approximate number of AIoT devices based on AF request;

-	approximate D2R message size based on AF request.

NOTE 5:	If there are multiple Readers selected for the AIoT Service, the AIOTF may provide the approximate number of AIoT devices to each Reader based on implementation.

NOTE 6:	The approximate D2R message size considering the overhead of AIoT Device NAS layer will be determined later in cooperation with CT WG1 and SA WG3.

NOTE 7:	Further assistance information can be added during the normative phase and in cooperation with other WGs if necessary.

Editor's note:	For RRC based solution of topology 2, whether the aggregation can be performed by the RAN is FFS and coordination with RAN WGs is needed.

f.	When the AIOTF sends an operation request to a UE Reader or BS Reader (via AIOT RAN), a response and one or more reports with the results of the AIoT service operation is returned to the AIOTF with the results of the AIoT service operation, and the AIOTF needs to correlate the results to a given operation request. The AMF (if used to route the requests) additionally provides an AIOTF identifier with the request from the AIOTF which is returned with the response(s) related to the request, so the AMF can be routed back the requesting AIOTF.

2.	A Permanent AIoT Device ID is stored in the AIoT Device and the network or a Credential Holder's AAA server. The AIOTF checks whether the AIoT Device ID from AIoT Device has a subscription and retrieves.

Editor's note:	Whether and how the AIoT Device ID privacy protection and ID authentication is done will be concluded by SA WG3.

3.	The AIoT Device does not distinguish whether the connectivity topology is Topology 1 or Topology 2, nor the transport used by the AIoT Reader.

NOTE 8:	The AIoT device is also agnostic to the potential different architectures if more than one architecture is concluded for both the topology 1 and topology 2.

4.	AIoT Device NAS protocol is supported between the AIoT Device and the AIOTF. The AIoT Device NAS layer supports Inventory Response and Command (e.g. Read and Write) Request and Response.

5.	The AIOTF may store and manage the AIoT device related information (also known as device context information) locally that includes e.g. the AIOT device permanent ID, the last known reader information of the AIoT device. The last known reader information can be used to support the AIOTF to select the serving reader to forward the message towards the specific AIoT device(s).

NOTE 9:	The storage of security related information at the AIOTF is up to the coordination with SA WG3 later.

* * * Next Change * * * *

### 8.3.4	Principles on the procedures to support AIoT services

To support the services provided by 5GC and the NEF exposure of those AIoT services, the following procedures are supported:

-	Inventory Procedure.

-	Command Procedure, to e.g. transfer AF AIoT Data to/from AIoT Device(s) as AIoT specific NAS messages.

There are requests that are used from the AIOTF towards the Reader and responses from the Reader to the AIOTF. The routing of the request and response messages and their encoding depends on the topology and transport to the Reader (see KI#1).

All the procedures follow have the following steps:

1.	The AF makes a service request to the NEF, including parameters as defined in clause 8.3.5.

2.	The NEF determines an AIOTF for the requested operation, and invokes a new service operation on the AIOTF.

3.	For the requested operation the AIOTF:

1).	Performs initial reader selection by either:

-	The AF provide information to identify readers to include in the initial reader selection network. The information can identify multiple or an individual Reader.

NOTE 1:	Which readers are identified by the information from the AF is up to the network deployment, configuration or implementation.

-	If a single UE Reader ID is provided by the AF via the NEF for the operation, then that is used as the selected Reader.

-	If reader selection information or UE Reader ID is not provided, then how the AIOTF determines which readers to use is based on implementation. The AIOTF may be e.g. preconfigured with which readers to use, or take the requested target AIoT Devices last known location into account, etc.

If no readers can be selected then the request is rejected.

2).	Determines AIoT Device Identification information  based on the information from the AF, to be included in the paging message on the AIoT radio interface to find the AIoT Devices. AIoT Devices compare the AIoT Device Identification information  with their own AIoT Device Identifier (part of or full AIoT Device Identifier) to determine whether respond to the paging message.

Editor's note:	Whether and how the A-IoT Device Identification information will be security protected will be concluded by SA WG3.

3).	Determines Reader Assistance information required for the operation used to the Reader, taking into account assistance information from the AF.

4).	Constructs a request for an Inventory operation using the determined A-IoT Device Identification information page the AIoT Devices, and a correlation identifier for the AIOTF to correlate the inventory responses to the request. The Inventory request is routed to the Readers determined by the initial reader selection.

See clause 8.1 for how to provide the request to a Reader.

5).	The Reader executes the inventory request, reporting AIoT specific NAS message responses from the AIoT Device to the AIOTF, including its Reader ID and correlation identifier from the AIOTF. The Reader may aggregate results from multiple AIoT Devices in the responding messages, if the operation is not a command operation. The AIOTF can determine which request the results are for using the correlation identifier.

6).	The AIOTF may, depending on the information within the AIoT Device identifier, obtain subscription-like information from either:

-	the serving network performing the operation (either as identified by a AIoT Device Identifier or operator policy to check it been provided with information for a specific AIoT Device), or

-	another network as identified by a AIoT Device Identifier, or

-	A third party as identified by a AIoT Device Identifier.

7).	Checks if the AIoT Device is subscribed.

NOTE 2:	Whether and how AIOT Device Identifiers are verified depends on SA WG3.

8).	If the operation is a command operation, the AIOT generates a request, including an AIoT specific NAS message for the command, along with any additional information required by the Reader to execute the command, and a correlation identifier to determine the destination of command responses. The request is then routed to the Reader. The Reader executes the command, passing the AIoTF specific NAS message to the AIoT Device and collecting any AIoT specific NAS responses. The AIoT specific NAS responses are then routed back to the AIOTF.

NOTE 3:	Whether and how security protection is applied to the AIoT specific NAS message send to the AIoT Device and the response AIoT specific NAS message from the AIoT Device depends on SA WG3.

9).	Provide the results of the operation to the NEF. Results from multiple AIoT Devices may be included/aggregated in the service response(s).

4.	Provide the results of the operation from the NEF to the AF.

* * * End of Change * * * *