# 3GPP TSG-SA WG2 Meeting #167	S2-2501593

Athens, Greece, 17 - 21 February 2025	(Revision of S2-250xxxx)

Title:	[Draft] Reply LS on emergency service and eDRX

Response to:	S2-2500044/R2-2411041

Release:	Rel-17

# Work Item:	     NR_newRAT-Core, NR_redcap-Core

Source:	SA2

To:	RAN2, CT1

Cc:	RAN3

Contact Person:	Qian Chen

E-mail Address:	qian (dot) xb (dot) chen (at) ericsson (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	TS 23.501 CR 6084 (S2-2501587), TS 23.502 CR 5338(S2-2501590)

1. Overall Description:

SA2 thanks RAN2 for the LS (S2-2500044/R2-2411041) on emergency services and eDRX.

The LS highlights the issue as stated below:

# 1 Overall description

In RAN2#128, companies discussed a scenario where a UE is configured with a RAN eDRX cycle in RRC_INACTIVE state when the UE has a PDU session associated with emergency services. RAN2 notice the below description in TS 23.501 which states that an Idle eDRX cycle should not be used by the UE when it has a PDU session associated with emergency services. However, it is unclear whether it is possible for such UE to be released to RRC_INACTIVE state with a RAN eDRX cycle and if it is so how UE behavior may be.

#### 5.31.7.2 Extended Discontinuous Reception (DRX) for CM-IDLE and CM-CONNECTED with RRC-INACTIVE ##### 5.31.7.2.1 Overview ##### // skipped irrelevant part When the UE has PDU Session associated with emergency services, the UE and AMF follow regular discontinuous reception as defined in clause 5.4.5 and shall not use the extended idle mode DRX. Extended idle mode DRX parameters may be negotiated while the UE has PDU Session associated with emergency services. When the PDU Session associated with emergency services is released, the UE and AMF shall reuse the negotiated extended idle mode DRX parameters in the last Registration Update procedure. // skipped irrelevant part

Based on current specification, a RAN eDRX cycle can be configured by a RAN node when releasing the UE to RRC_INACTIVE state only if the UE is configured with an Idle eDRX cycle by CN.

RAN2 assumes that RAN node should be aware whether a UE has a PDU session associated with emergency services so that it can avoid releasing such UE to RRC_INACTIVE state with a RAN eDRX cycle, or even if it is so, there should be means for the RAN node and the UE to not use the configured RAN eDRX cycle. Regarding how RAN node can be aware of such PDU session, RAN2 understands that CN can already indicate this (e.g. via ARP value) but there is no requirement for CN to do this, so, there is case that RAN node may not be aware of this.

RAN2 would like SA2 and CT1 to consider the scenario described above, provide feedback regarding how the identified issue can be addressed and whether any standard solution is needed?

# 2 Actions

To SA2, CT1 group

ACTION: 	RAN2 would like SA2 and CT1 to consider the scenario described above, provide feedback regarding how the identified issue can be addressed and whether any standard solution is needed?

SA2 discussed the scenarios RAN2 named in the LS and would like to provide the following information:

- TS 23.501 has specified the following that each PLMN/operator reserves a specific ARP value for emergency PDU session and provide to 5G-AN:

#### 5.16.4.3        Mobility Restrictions and Access Restrictions for Emergency Services

When the (R)AN resources for Emergency Services are established, the ARP value for Emergency Services indicates the usage for Emergency Services to the 5G-AN.

#### 5.16.4.7        PCC for Emergency Services

Dynamic PCC is used for UEs establishing emergency service and shall be used to manage IMS emergency sessions when an operator allows IMS emergency sessions. When establishing Emergency Services with a SMF, the PCF provides the SMF with the QoS parameters, including an ARP value reserved for the Emergency Services to prioritize the QoS Flows when performing admission control, as defined in TS 23.503 [45].

#### 5.16.4.9        Handling of PDU Sessions for Emergency Services

The ARP reserved for emergency service shall only be assigned to QoS Flows associated with an emergency PDU Session.

- TS 23.502 has also specified the following:

#### 6.1.3.10        IMS emergency session support

The PCC Rule Authorization function selects QoS parameters that allow prioritization of IMS Emergency sessions. If an IMS Emergency session is prioritized the QoS parameters in the PCC Rule shall contain an ARP value that is reserved for intra-operator use of IMS Emergency services. The PCF does not perform subscription check; instead it utilizes the locally configured operator policies to make authorization and policy decisions.

SA2 considers that RAN is enabled to decide if eDRX can be applied when release a UE with emergency PDU session to RRC_INACTIVE state, taking into the consideration that CN provides RAN with both ARP and proper eDRX values for the UE context.

SA2 also indicates that a UE with emergency PDU Session stays in RRC_CONNECTED state with the possibility of inactivity timer running for emergency PDU session in CN. After inactivity timer expires, the emergency PDU sessions will be released (e.g. there is no scenario that paging will be triggered when UE has emergency PDU session). SA2 has agreed the attached CRs to clarify this.

2. Actions:

To RAN2, CT1

ACTION: 	SA2 kindly asks RAN2 and CT1 to take above information into consideration and update the corresponding specifications, if needed.

3. Date of Next SA2 Meeting:

SA2#168   				 07 – 11 April, 2025			        	     Goteborg, SE

SA2#169				 19 – 23 May, 2025				Fukuoka, Japan