3GPP TSG-WG2 Meeting #167	S2-2501607

# 17 - 21 February 2025, Athens, Greece

Source:	Ericsson

Title:	Analysis on the number of LCS-UPP connections

Agenda item:	9.6.2

Document for:	DISCUSSION

# Introduction

In SA2#165 SA2 meeting CR has been postponed defining the number of LCS-UPP connections per UE. Now this discussion paper analyses the situation and proposes the possible way forward.

# Background

LCS-UPP has been defined in Rel-18 to use user plane connection between UE and LMF to transfer LPP and supplementary services instead of using control plane. Based on stage 3 work and specification it is possible to maintain only one LCS-UPP, but it was never stage 2 intention to limit number of connections to one. The problem with one LCS-UPP connection per UE is that if AMF needs to select an LMF and if we want to leverage the user plane connection then AMF needs to select that LMF which has LCS-UPP based on UE context. In this case other selection criteria like slice information, tracking area, etc. cannot be used if those selection criteria are mutually exclusive. If operator wants to deploy LMFs for different positioning purposes and uses cases, then allowing only one LCS-UPP per UE limits the possibilities of user plane positioning solution as well as selection of dedicated LMF(s) for other services when deployed together.

Observation #1: Based on the existing solutions if an operator wants to leverage user plane connection for positioning AMF is forced to select LMF which has LCS-UPP based on UE context.

# Discussion

Based on CT4 TS 29.518 AMF can have only one LcsUPContext for the UE:

Table 6.1.6.2.84-1: Definition of type LcsUpContext

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| upConnectionStatus | UpConnectionStatus | O | 0..1 | UP Connection Status |
| servingLMFIdentification | LMFIdentification | O | 0..1 | Serving LMF ID |

Observation #2: AMF can store only one LcsUpContext.

If AMF selects that LMF which does not have LCS-UPP - but based on UE context there is an LMF which has LCS-UPP connection to the UE – then the selected LMF may initiate LCS-UPP towards the UE which shall be rejected by the UE since UE has already LCS-UPP with other LMF. This can cause extra delay in the positioning service.

Workaround could work if one LMF is registered in NRF with LCS-UPP capability, but operators might deploy multiple LMFs for different positioning requests, and there is no reason to limit only 1 LMF can have the LCS-UPP connection to the UE.

Observation #3: Selected LMF which capable of LCS-UPP is not aware that UE has already LCS-UPP connection with other LMF then it may try initiate LCS-UPP which can cause delay in positioning service.

Proposal: Define UE can have one LCS-UPP connection towards each LMF and can support multiple LCS-UPP connections towards different LMFs.  AMF supports multiple LCS-UPP connections for single UE.

Following stage 3 impacts would be expected if multiple LCS-UPP connections are supported from UEs, one only to each LMFs.

- AMF shall store more LcsUPContext per UE. AMF could consider more LMFs during LMF selection if user plane selection criteria is priority.

- For the user plane connections, the UE would have the information of the LMF LCS-UP address, which is included in the USER PLANE CONNECTION ESTABLISHMENT COMMAND message, it shall be used for the UE to route the corresponding LCS messages.

# 4. Proposals

Proposal: Define UE can have one LCS-UPP connection towards each LMF and can support multiple LCS-UPP connections towards different LMFs.  AMF supports multiple LCS-UPP connections for single UE.