3GPP TSG-SA2 Meeting #167	S2-2501612

Athens, Greece, February 17 – 21, 2025

Source:	Ericsson

Title:	Handling of UE access to satellite which has no UE context

Document for:	Discussion

Agenda Item:	19.1.2

Work Item / Release:	5GSAT_Ph3-ARC / Rel-19

Abstract of the contribution: Discusses a possible solution for UE attempting to access satellites where the NW is not expected to have the UE context.

# 1.	Introduction

In SA2#166-adhoc-e meeting, a SoH was held indicating that most companies preferred that the S&F monitoring list should be assistance information for the UE, i.e. the UE could handle it based on implementation. Views were also expressed to the point that adding normative UE behaviour on top of this assistance information does not align with the SoH result.

The paper S2-2501339 was agreed with the following editor’s note:

Editor’s Note: It is for further consideration whether specific UE behaviour or clarification needs to be added to prevent the UE from accessing satellites where the NW is not expected to keep a UE context.

The reason for the EN is that there is still a potential issue for a deployment where the UE context is only uploaded on a subset of MME-onboard(s) associated with MME-ground. In such case, if a UE makes an attempt to access the satellites which do not hold the UE context, the UE may be detached from the network and would need to re-attach.

To address the potential issue mentioned above, there were some options discussed last meeting. For example:

- alt 1:	Using TAU instead of (CP) Service Request to access a satellite not on the monitoring list. This would indicate UE’s S&F capability to the MME onboard the satellite. This can be perceived as going against the SoH result, since new UE behavior is standardized with regards to the monitoring list, which is supposed to be assistance information, and the handling is up to UE implementation. Another potential issue is that satellites without UE context are not aware whether there is any UE context in other satellites. For example, if the network detaches the UE it would inform the MME-onboards that have UE context about this, so that they can properly detach the UE (e.g. send a Detach message or reject a UE request with cause code #10 (implicitly detached)). However, satellites without UE context will not be informed about this and will continue to provide the S&F cause code to the UE, preventing the UE from initiating a new attach.

- alt 2:	Adding UE’s S&F capability in (CP) Service Request, so that the MME can be informed about UE’s S&F capability and reject UE with S&F cause. But according to 24.301 clause 4.4.4.3, an MME shall not process such NAS message, since it cannot integrity verify the massage due to no security/UE context on the MME for this UE. Also, opening up for performing capability negotiation via the (CP)SR procedures implies further system impact not investigated.

- alt 3:	All satellites are synchronized with a minimum context for every attached S&F capable UE associated with the ground MME (e.g. GUTI and NAS key). This may work in some deployments. But it is also true that in terms of IoT devices, a minimum context for every attached S&F capable UE may still consume lots of resources due to the high volume of the total device number.

Another option, proposed in this document, is that the MME-onboard without UE context sends a reject with a legacy cause code (e.g. #9) but also includes additional information that can be interpreted by an S&F capable UE. Such additional information is ignored by a non-S&F capable UE (e.g. a legacy UE). This is further elaborated below.

# 2.	Proposal

When MME receives the UE request message targeting the MME itself (e.g. according to its S-TMSI, GUTI) and the MME does not hold the UE context, the existing behaviour is that MME rejects the UE. This will cause the UE to become detached from the network.

This paper proposes one alternative approach to solve the above issue: the MME can be configured to provide extra information in the reject message with legacy reject cause to indicate to the S&F-capable UE that it can remain in its EMM state.

The benefit of the proposal is that this is an MME configuration that can be set based on operator policy and deployment and does not depend on MME parsing NAS messages that cannot be integrity verified. If in a deployment, only a subset of MME-onboard(s) are uploaded with the UE context, to prevent the UE from detaching when the UE’s request ends up in an MME-onboard without UE context, the UE will be rejected using legacy message and legacy cause (e.g. cc#9), but with this additional S&F indication, the UE can remain in its EMM state. Such additional information is only handled by UE enabled for S&F, and ignored by other UEs.

A potential problem with any solution using S&F cause code is that the UE may get stuck in an endless loop where its TAU is repeatedly rejected with an S&F cause (e.g. if the network has detached the UE and no MME-onboard has any UE context). To avoid getting stuck in waiting, it can be up to UE implementation to decide how many times to follow this indication consecutively and whether to take into account the satellite monitoring list as an input (e.g. whether the reject message comes from satellite on the list or not) to follow the indication. It can be noted that this is a problem also with the existing support in 23.401 for rejecting a TAU with S&F cause, and thus a corresponding UE behaviour will anyway be needed.