3GPP TSG-WG2 Meeting #167	S2-2501646

# 17 - 21 February, 2025, Athens, Greece				                               (revision of S2-2501345)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1333 | rev | 3 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolve some ENs in the vertical federated learning inference procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO, [Nokia, Ericsson, Samsung] |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-02-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Trusted AF as the VFL server case is added based on the NWDAF as the server case since both the case have many overlap regarding the service operation and the VFL logic. Resolving the following EN by adding the sentenc to let server can internally store information about the features considered when deriving the analytics results together with a timestamp to identify the analytics results to enable to subsequently trace the origin of analytics results. Editor's note: It is FFS how the origin of analytics results can be traced and explained if not all clients participate and results are not satisfying. The use case for the VFL trigger is simplified to only two case. One is the NWDAF which has the AnLF and MTLF can be the VFL server. Consumer NF can discover such NWDAF based on the analytics ID and Analytics request/subscribe service. The other case is AF is the server, the consumer NF interact with the AF server via AnLF. AnLF send the Naf_Inference_subscribe/request to the AF to tigger the VFL server AF to perform the training and also perform inference. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the trusted AF as the VFL server case. Add the description on how to trigger the VFL inference procedure. The VFL training procedure can also be triggerd based on the analytics request from the consumer before the VFL server initiate the VFL inference. Remove the following ENs: Editor's note: The service name between the AnLF and untrusted AF as VFL server is FFS. Editor's note: It is FFS how the origin of analytics results can be traced and explained if not all clients participate and results are not satisfying. Editor's note: Parameters in the triggering of inference and Analytics is FFS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The VFL inference procedure is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2H.2.4.1, 6.2H.2.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * *Start of Changes * * *

6.2H.2.4.1	Inference procedure for vertical federated learning when NWDAF or Trusted AF is acting as VFL server

Figure 6.2H.2.4.2-1: Inference procedure for vertical federated learning when NWDAF or Trusted AF is acting as VFL server

The inference procedure when trusted AF is acting as VFL server may be triggered by a request or subscription from a 5GC consumer NF or internal service logic of the AF acting as VFL server. If triggered by internal service logic of the AF acting as VFL server, the steps 0, 1,7 and 8 are skipped.

0.	The analytics consumer NF sends an Analytics request/subscribe (Analytics ID, Target of Analytics Reporting= e.g. UE IDs and optionally Analytics Reporting Information=Analytics target period and Analytics Filter) to NWDAF containing AnLF by invoking a Nnwdaf_AnalyticsInfo_Request or a Nnwdaf_AnalyticsSubscription_Subscribe.

1.	If the NWDAF containing AnLF can be the VFL server to generate the VFL inference results for the requested analytics ID, then step 1 is skipped.

If the NWDAF containing AnLF can not generate the analytics output, the NWDAF containing AnLF determines the VFL Server AF for the requested analytics.

If the VFL server is trusted AF, the NWDAF containing AnLF sends a request to trusted AF VFL server using Naf_Inference_subscribe/request including Analytics ID, Target of Analytics Reporting = e.g. UE IDs and optionally Analytics Reporting Information=Analytics target period and Analytics Filter.

2.	Based on the information received in the step 0 or 1, VFL server decides to initiate the VFL inference procedure with the VFL clients. Before the VFL server initiate the VFL inference procedure, the VFL server may initiate the VFL training procedure as described in the clause 6.2H.2.3 based on the information received in the step 0 or 1 or VFL server local configuration. VFL Server selects clients(s) using information stored in the VFL training process. The server may select some or no clients, e.g. depending on the accuracy of the VFL model, the contribution to the training result and the current status of the VFL clients.

When no VFL Clients are selected, the VFL server may generates the VFL inference results based only on its local trained ML model associated with the determined VFL correlation ID, skipping the steps 2 - 6 (and 7 if step 1 was also skipped).

NOTE 1:	If the server does not select VFL clients for all features, it can internally store information about the features considered when deriving the analytics results together with a timestamp to identify the analytics results, to enable to subsequently trace the origin of analytics results and to interpret possible feedback on analytics accuracy.

VFL server NWDAF or trusted AF sends a VFL Inference request/subscription to the VFL clients including the Target of VFL inference = e.g. UE IDs, VFL correlation ID to indicate the VFL client which previously well-trained VFL local model associated with this ID will be used and optionally VFL inference filter.

Editor's note:	It is FFS whether additional parameters are needed to send from the VFL server to VFL client, e.g. parameters used in training phase and parameters from Analytics request.

2a.	For each NWDAF VFL client, the VFL Server NWDAF or trusted AF sends an Nnwdaf_VFLInference_Subscribe or Nnwdaf_VFLInference_Request to the VFL client.

2b.	For each trusted AF VFL client, the VFL Server NWDAF sends an Naf_VFLInference_Subscribe or Naf_VFLInference_Request to the VFL client.

2c.	For each untrusted AF VFL client, the VFL Server NWDAF sends an Nnef_VFLInference_Subscribe or Nnef_VFLInference_Request to the NEF serving the AF.

2d.	For each untrusted AF VFL client, the NEF converts any internal identifiers to external identifiers and sends an Naf_VFLInference_Subscribe or Naf_VFLInference_Request to the untrusted AF VFL client.

NOTE 2:	For the trusted AF is acting as VFL server, the VFL client can only be the NWDAF.

3.	Each VFL Client collects its local data by using the current mechanism if the VFL Client does not have local data available already.

4.	Based on the VFL correlation ID, each VFL Client determines the VFL local model to generate the intermediate local inference results.

Editor's note:	Whether VFL client may also provide local intermediate inference results to other VFL client is FFS.

5.	VFL Client sends the client intermediate results to the VFL server.

The intermediate results, which are sent from the VFL Client to the VFL Server during the VFL inference process, are the information for the VFL Server to combine and generate the VFL inference results.

Editor's note:	It is FFS additional parameters are needed to send from the VFL client to VFL server.

If the VFL server used an inference subscription in step 2, step 5 may be repeated.

5a.	Each NWDAF VFL client sends an Nnwdaf_VFLInference_Notify or Nnwdaf_VFLInference_Request response to the VFL Server NWDAF or trusted AF.

5b.	Each trusted AF VFL client sends a Naf_VFLInference_Notify or Naf_VFLInference_Request response to the VFL Server NWDAF.

5c.	Each untrusted AF VFL client sends a Naf_VFLInference_Notify or Naf_VFLInference_Request response to the NEF.

5d.	For each untrusted AF VFL client, the NEF converts any external to internal identifiers and sends an Nnef_VFLInference_Notify or Nnef_VFLInference_Request response to the NWDAF VFL server.

6.	The VFL server may collect its local data and generate the intermediate local inference results. When the VFL Server selected VFL clients to participate in the VFL Inference process, it combines all the intermediate results to generate the VFL inference results based on the VFL correlation ID. The VFL server takes into account the participation of each VFL client during the ML training process and the importance of the intermediate results when generates the combined inference output.

7.	Depending on request, the NWDAF or trusted AF VFL server sends Nnwdaf_AnalyticsInfo_Response or Nnwdaf_AnalyticsSubscription_Notify or Naf_Inference_Response/Notify to the consumer (i.e. NWDAF containing AnLF) including the VFL inference results.

8.	The NWDAF containing AnLF provides the analytics output to the analytics consumer NF based on the VFL inference results by means of either Nnwdaf_AnalyticsInfo_Response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 0.

* * *Next Change * * *

6.2H.2.4.2	Inference procedure for vertical federated learning when untrusted AF is acting as VFL server

Figure 6.2H.2.4.2-1: Inference procedure for vertical federated learning when untrusted AF is acting as VFL server

The inference procedure when untrusted AF is acting as VFL server may be triggered by a request or subscription from a 5GC consumer NF or internal service logic of the AF acting as VFL server. If triggered by internal service logic of the AF acting as VFL server, the steps 0, 1,7 and 8 are skipped.

0.	Same as step 0 in clause 6.2H.2.4.1.

1.	Same as step 1 in clause 6.2H.2.4.1, to NEF using Nnef_Inference_subscribe/request. NEF forwards the subscription request to AF using Naf_Inference_subscribe/request.

Editor's note:	When the AnLF determine the VFL server AF is FFS. For example, during the training phase.

2.	Same as step 2 in clause 6.2H.2.4.1, to NEF using Nnef_VFLInference_subscribe/request. An untrusted AF includes the external NWDAF ID and sends the request to the NEF.

NEF converts any received external identifiers to internal identifiers and forwards the subscription request to NWDAF using Nnwdaf_VFLInference_subscribe/request.

Editor's note:	It is FFS additional parameters are needed to send from the VFL server to VFL client.

3.	Same as step 3 in clause 6.2H.2.4.1.

4.	Same as step 4 in clause 6.2H.2.4.1.

Editor's note:	Whether VFL client may also provide local intermediate inference results to other VFL client is FFS.

5.	Same as step 5 in clause 6.2H.2.4.1, to NEF using Nnwdaf_VFLInference_Notify/Request Response.

NEF converts any received internal identifiers to external identifiers and forwards the subscription notify to the untrusted AF using Nnef_VFLInference_Notify/Request Response.

Editor's note:	It is FFS additional parameters are needed to send from the VFL client to VFL server.

If the VFL server used an inference subscription in step 2, step 5 may be repeated.

6.	Same as step 6 in clause 6.2H.2.4.1.

7.	Same as step 7 in clause 6.2H.2.4.1, to NEF using Naf_Inference_Notify/Request Response.

NEF converts any received internal identifiers to external identifiers and forwards the subscription Notify to NWDAF using Nnef_Inference_Notify/Request Response.

8.	Same as step 8 in clause 6.2H.2.4.1.

* * *End of Changes * * *