3GPP TSG-WG2 Meeting #167	S2-2501678

# Athens, Greece, 17 – 21 February 2025								   		 (revision of S2-2501143)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1370 | rev | 2 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#2 – update to registration and discovery procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [InterDigital, Nokia, KDDI, OPPO, LGE, Ericsson, Samsung], vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Solving multiple Editor’s Notes - Editor´s note: It is FFS, whether to keep the case that trusted AF as VFL server along with the clause of NWDAF as server or with the clause of untrusted AF as server. The trusted AF, as VFL server, can communicate directly with any NF and should be handled in the same way as an NF. Registration and discovery procedure with NWDAF as VFL server are updated to include trusted AF case also. - Editor´s note: The following is FFS: a deployment scenario that can also be agreed between operator and application service provider is when untrusted AF is the VFL server and only one NWDAF will be the VFL client. In such case the VFL Server can be configured to go directly to preparation procedure in clause 6.2H.2.2. When an untrusted AF as VFL server performs an VFL operation, NEF needs to discover any NWDAF as VFL client supporting the corresponding Analytic ID. And when NEF is configured with NWDAF’s information supporting VFL for an analytic ID, it may respond the discovery result based on configuration to the untrusted AF. But even the above scenario, step 4 and step 9 between AF(VFL server) and NEF needs to be performed. NOTE is added to clarify it. - Editor's note: Whether a discovery of an AF acting as VFL server is required and how it can be done is FFS. When AF is 3rd party entity, its capability is registered to NEF via OAM configuration and NF consumer (e.g., NWDAF with AnLF) may discover AF as VFL server via NRF. - Editor's note: It is FFS, whether NEF should perform VFL NWDAF selection (or shortlisting) or it should only discover NWDAF client candidates without any further selection. The location information of an NF instance (TS23.501 clause 6.3.1.2) can be used to select appropriate NWDAF(s) as VFL clients for a requested VFL operation. However, since the location information of an NF instance is highly sensitive and could disclose the operator’s deployment details, the NEF should be involved in selecting suitable NWDAF candidates based on this location information, without sharing it with 3rd party AFs. It is clarified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR addresses a number of Editor’s notes pointing at incomplete procedures that need to be completed for the correct execution of VFL discovery.. |
| Consequences if not approved: |  | VFL discovery will not be implemented correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2, 5.5, 6.2H.2.1.1, 6.2H.2.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | In rev2: - Distinct parameters clearly for VFL registration and VFL discovery correspondingly; - Move the untrused AF as VFL server’s registration operation to a correct step for procedure of 6.2H.2.1.2. - Other minor changes. |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

5.2	NWDAF Discovery and Selection

The NWDAF service consumer selects an NWDAF that supports requested analytics information and required analytics capabilities and/or requested ML Model Information by using the NWDAF discovery principles defined in clause 6.3.13 of TS 23.501 [2].

Different deployments may require different discovery and selection parameters. Different ways to perform discovery and selection mechanisms depend on different types of analytics/data (NF related analytics/data and UE related analytics/data). NF related refers to analytics/data that do not require a SUPI nor group of SUPIs (e.g. NF load analytics). UE related refers to analytics/data that requires SUPI or group of SUPIs (e.g. UE mobility analytics).

In order to discover an NWDAF containing AnLF using the NRF:

-	If the analytics is related to NF(s) and the NWDAF service consumer (other than an NWDAF) cannot provide an Area of Interest for the requested data analytics, the NWDAF service consumer may select an NWDAF with large serving area from the candidate NWDAFs from discovery response. Alternatively, in case the consumer receives NWDAF(s) with aggregation capability, the consumer preferably selects an NWDAF with aggregation capability with large serving area.

NOTE 1:	If the selected NWDAF cannot provide the requested data analytics, e.g. due to the NF(s) to be contacted being out of serving area of the NWDAF, the selected NWDAF might reject the analytics request/subscription or it might query the NRF with the service area of the NF to be contacted to determine another target NWDAF.

-	If the analytics is related to UE(s) and the NWDAF service consumer (other than an NWDAF) cannot provide an Area of Interest for the requested data analytics, the NWDAF service consumer may select an NWDAF with large serving area from the candidate NWDAFs from discovery response. Alternatively, in case the consumer receives NWDAF(s) with aggregation capability, the consumer preferably selects an NWDAF with aggregation capability with large serving area.

NOTE 2:	If a selected NWDAF cannot provide analytics for the requested UE(s) (e.g. the NWDAF serves a different serving area), the selected NWDAF might reject the analytics request/subscription or it might determine the AMF serving the UE as specified in clause 6.2.2.1, request UE location information from the AMF and query the NRF with the tracking area where the UE is located to discover another target NWDAF serving the area where the UE(s) is located.

-	If the analytics are related to UE(s) and if NWDAF instances indicate weights for TAIs in their NF profile (see clause 6.3.13 of TS 23.501 [2]), the NWDAF service consumer may use the weights for TAIs to decide which NWDAF to select.

-	If the NWDAF service consumer needs to discover an NWDAF containing an AnLF with analytics accuracy checking capability, the consumer may query NRF providing also the analytics accuracy checking capability in the discovery request.

-	If the NWDAF service consumer needs to discover an NWDAF containing AnLF which can use the Model provided by the specific NWDAF containing MTLF (e.g., in case of analytics context transfer), the consumer should discover the NWDAF(s) whose vendor ID is in the ML Model Interoperability indicator of the NWDAF containing MTLF.

If the NWDAF service consumer needs to discover an NWDAF that is able to collect data from particular data sources identified by their NF Set IDs or NF types or to collect data from particular NWDAF Serving Area, the consumer may query NRF providing the NF Set IDs or NF types or Area of Interest in the discovery request.

NOTE 3:	The NF Set ID or NF Type of a data source serving a particular UE, can be determined as indicated in Table 5A.2-1.

In order to discover an NWDAF that has registered in UDM for a given UE:

-	NWDAF service consumers or other NWDAFs interested in UE related data or analytics, if supported, may make a query to UDM to discover an NWDAF instance that is already serving the given UE.

If an NWDAF service consumer needs to discover NWDAFs with data collection exposure capability, the NWDAF service consumer may discover via NRF the NWDAF(s) that provide the Nnwdaf_DataManagement service and their associated NF type of data sources or their associated NF Set ID of data sources or NWDAF Serving Area information as defined in clause 6.3.13 of TS 23.501 [2].

In order to discover an NWDAF containing MTLF via NRF:

-	When one or more trained ML Models are available for one or more Analytics ID(s) the NWDAF containing MTLF shall include the Analytics ID(s) that is(are) supported per service in the registration towards NRF. The NWDAF containing MTLF may wait to register in NRF the above services until at least one trained model is available. The NWDAF containing MTLF may provide to the NRF a list of Analytics IDs corresponding to the trained ML Models and possibly the ML Model Filter Information for the trained ML Model per Analytics ID(s), if available. In this Release of the specification, only the S-NSSAI(s) and Area(s) of Interest from the ML Model Filter Information for the trained ML Model per Analytics ID(s) may be registered into the NRF during the NWDAF containing MTLF registration. If supporting model training for the LMF-based AI/ML Positioning, the NWDAF containing MTLF provides its NF profile to the NRF during registration with an indication of supporting model training for LMF-based AI/ML Positioning and positioning case information (e.g. UE assisted LMF-based AI/ML Positioning case, NG RAN assisted LMF-based AI/ML Positioning case, or both). If the NWDAF containing MTLF supports ML Model interoperability, the NWDAF containing MTLF includes, in the registration to the NRF, an ML Model Interoperability indicator for each Analytics ID.

NOTE 4:	How to implement the indication of supporting model training for LMF-based AI/ML Positioning in a backward compatible and extensible manner is up to stage 3.

Editor's note:	Whether the AI/ML positioning model needs to be trained per case (i.e. case 2b, case 3b) is FFS, this needs coordination with RAN WGs.

-	The ML Model Interoperability indicator comprises a list of NWDAF providers (vendors) that are allowed to retrieve ML Models from this NWDAF containing MTLF. It also indicates that the NWDAF containing MTLF supports the interoperable ML Models requested by the NWDAFs from the vendors in the list.

NOTE 5:	The S-NSSAI(s) and Area(s) of Interest from the ML Model Filter Information are within the indicated S-NSSAI and NWDAF Serving Area information in the NF profile of the NWDAF containing MTLF, respectively.

-	During the discovery of NWDAF containing MTLF, a consumer (e.g. an NWDAF containing AnLF, an NWDAF containing MTLF as FL server or FL client) may include in the request the target NF type (i.e. NWDAF), the Analytics ID(s), the S-NSSAI(s), Area(s) of Interest of the Trained ML Model required and Vendor ID. If the consumer is an LMF, it may include in the request an indication that supporting of ML Model training for LMF-based AI/ML Positioning is required. The NRF returns one or more candidate instances of NWDAF containing MTLF to the NF consumer and each candidate instance of NWDAF containing MTLF includes the Analytics ID(s), possibly the ML Model Filter Information for the available trained ML Models and ML Model Interoperability indicator, if available.

-	If the NWDAF service consumer needs to discover an NWDAF containing an MTLF with ML Model accuracy checking capability, the consumer may query NRF also providing the ML Model accuracy checking capability in the discovery request.

In order to discover an NWDAF containing MTLF with Horizontal Federated Learning (HFL) capability via NRF, in addition to the procedures described above for discovering NWDAF containing MTLF:

-	An NWDAF containing MTLF supporting FL as a server shall additionally include FL capability type (i.e. FL server) and may include Time interval supporting FL as FL capability information during the registration in NRF.

-	An NWDAF containing MTLF supporting FL as a client shall additionally include FL capability type (i.e. FL client) and may include Time interval supporting FL as FL capability information during the registration in NRF, and it may also include, NF type(s) and NWDAF Serving Area information and/or NF set ID(s) of the data source(s) where data can be collected as input for local model training.

NOTE 6:	An NWDAF containing MTLF may indicate to support both FL server and FL client in the FL capability for specific Analytics ID. The FL capability type only applies for HFL to maintain backward compatibility.

-	During the discovery of NWDAF containing MTLF as FL server, a consumer (e.g. a NWDAF containing MTLF) may include in the request the FL capability type as FL server and may include Time Period of Interest and ML Model Filter information for the trained ML Model(s) per Analytics ID(s), if available. The NRF returns one or more NF profiles of candidate instances of NWDAF satisfying the query parameters.

-	During the discovery of NWDAF containing MTLF as FL client, a consumer (e.g. an FL server) may include in the request FL capability type as FL client and may include Time Period of Interest, a list of NF type(s) and/or NF set ID(s) of the data source(s). The NRF returns one or more NF profiles of candidate instances of NWDAF satisfying the query parameters.

NOTE 7:	The service consumer to discover an NWDAF containing MTLF with FL capability is limited to NWDAF containing MTLF in this Release.

A PCF may learn which NWDAFs being used by AMF, SMF and UPF for a specific UE, via signalling described in clause 4.16 of TS 23.502 [3]. This enables a PCF to select the same NWDAF instance that is already being used for a specific UE.

In the roaming architecture, the NWDAF with roaming exchange capability (RE-NWDAF) to request analytics or input data is discovered via the NRF. A consumer in the same PLMN as the RE-NWDAF discovers the RE-NWDAF(s) by querying for NWDAF(s) where the roaming exchange capability is indicated in its (their) NF profile. A consumer in a peer PLMN (i.e. RE-NWDAF) discovers the RE-NWDAF(s) by querying for NWDAF(s) in the target PLMN that is (are) supporting the specific services defined for roaming. A RE-NWDAF discovers the RE-NWDAF(s) in a different PLMN (i.e. HPLMN or VPLMN) using the procedure defined in clause 4.17.5 (if delegated discovery is not used) or clause 4.17.10 (if delegated discovery is used) of TS 23.502 [3], where the detailed parameters are determined based on the analytics request or subscription from the consumer 5GC NF, operator policy, user consent and/or local configuration.

In order to support VFL training and inference, the NWDAF supporting VFL shall include its VFL capability information per supported Analytics ID during registering to NRF. The VFL capability information includes VFL capability type (i.e. VFL server or VFL client or both) and Time interval supporting VFL if available. When the VFL capability type indicates VFL client, NWDAF also includes its VFL interoperability indicator(s) per Analytics ID, optionally with supported feature ID(s) and it may also include NWDAF Serving Area information, NF type(s) and/or NF set ID(s) of the data source(s), where data can be collected as input for local model training during registration to NRF. When NFs (i.e. NWDAF and/or AF) perform VFL together, only NFs sharing same VFL interoperability indicator can understand feature ID(s) and other VFL configurations from each other. Feature ID(s) represent the feature information supported by the NWDAF as VFL client per Analytics ID.

To discover NWDAF supporting VFL from NRF, the consumer should consider the VFL capability information. To discover NWDAF as VFL client from via NRF, the consumer should also consider VFL interoperability indicator(s) and optionally consider Area of interest, target NF type(s) and/or target NF set ID(s) of the data source(s) and/or target feature ID(s). The detailed procedure and parameters of NWDAF registration and discovery for VFL are as defined in clause 6.2H.2.1.

NOTE 8:	There are no standardized feature ID in this release, and only feature IDs related to static/preconfigured feature information can be registered to NRF.

* * *Next Change * * *

5.5	AF Discovery and Selection for VFL

The AF discovery and selection is defined in clause 6.3.25 of TS 23.501 [2]. In addition to support VFL training and inference, the following factors may be considered for AF discovery and selection:

-	VFL capability information per supported AnalyticsID, which includes:

-	VFL capability type (i.e. VFL client, and/or optionally VFL server)

-	optional Time interval supporting VFL.

-	VFL interoperability indicator(s) of AF as VFL client per supported Analytics ID: indicating that certain NFs (i.e. NWDAF and/or AF) that share the same VFL interoperability indicator, can perform VFL together.

-	[Optional] supported feature ID(s): representing the feature information supported by the AF as VFL client.

A trusted AF registers the above factors in NRF. For an untrusted AF the NEF is configured via OAM to register the factors into NRF.

For the discovery of a trusted AF or untrusted AF, the consumer may select an AF instance considering the above factors. The detailed procedure and parameters of AF registration and discovery for VFL are as defined in clause 6.2H.2.1.

* * * Next Change * * *

6.2H.2.1	Registration and Discovery procedure for Vertical Federated Learning

6.2H.2.1.1	Registration and Discovery procedure for Vertical Federated Learning when NWDAF or trusted AF is acting as the VFL server

Figure 6.2H.2.1.1-1: Registration and Discovery procedure for Vertical Federated Learning when NWDAF or trusted AF is acting as VFL server

Steps 1 to 3 are the NWDAF and AF Registration procedures when the VFL server is NWDAF or a trusted AF.

1a.	VFL Server NWDAF/trusted AF registers to NRF with its NF profile, which includes NF Type (i.e. NWDAF type or AF type), Analytics ID(s), service area if available, VFL capability information per analytics ID and VFL interoperability indicator(s) and optional supported feature ID(s), and other parameters as described in clause 5.2.

1b.	NWDAF as VFL client registers to NRF with its NF profile, which includes NF Type (i.e. NWDAF type), Analytics ID(s), service area if available, VFL capability information per analytics ID and VFL interoperability indicator(s) and optional supported feature ID(s), and other parameters as defined in clause 5.2.

1c.	When untrusted AF as VFL client, it shall register to the NEF via OAM configuration: Analyics ID(s) and its VFL capability information per supported analytics ID and VFL interoperability indicator(s) and optional supported feature ID(s), and other parameters, as defined in clause 5.5. Then NEF updates NEF profile to NRF including associated AF ID and AF's parameters described in clause 5.5.

1d.	When trusted AF as VFL client, it registers to NRF with its NF profile, which includes NF Type (i.e. AF type), analytics ID(s), service area if available, VFL capability information per analytics ID and VFL interoperability indicator and optional supported feature IDs, and other parameters, as defined in clause 5.5.

2.	The NRF receives the registrations from VFL server and VFL client(s), and stores their NF profile.

3.	The NRF sends registration response to VFL server and VFL client(s).

Steps 4 to 6 are the NWDAF,AF and NEF Discovery procedures when the VFL server is NWDAF or trusted AF.

4-6.	The VFL server determines that the ML Model requires VFL based on e.g. operator policy, Analytics ID, VFL interoperability indicator(s) and Service Area.

NOTE:	Step 4 in Figure 6.2H.2.1.1-1 may be triggered at the VFL Server by itself or a request from a consumer.

If an NWDAF can not perform as VFL Server, it first discovers and selects another VFL Server from NRF by invoking the Nnrf_NFDiscovery_Request service operation. The following criteria might be used: Analytics ID, VFL capability type as VFL server, Time Period of Interest and optional Service Area.

Once the VFL Server is determined, the VFL Server discovers other NWDAF(s) and/or AF(s) as VFL Client from NRF by invoking the Nnrf_NFDiscovery_Request service operation. The following criteria might be used: NF type(s) (i.e. NWDAF type, AF type, or NEF type), Analytics ID, VFL capability type (i.e.VFL client), VFL interoperability indicator(s), Time Period of Interest, optional feature ID(s) and optional Service Area. The AF(s) discovered by the VFL Server may be trusted and/or untrusted, the NF type may be AF type when the AF(s) as VFL Client are trusted, and the NF type may be NEF type when the AF(s) as VFL Client are untrusted, and the NF type may contain both AF type and NEF type if both trusted AF and untrusted AF are involved as VFL clients.

When an AF is acting as a VFL server, only NWDAF(s) are discovered as VFL clients.

6.2H.2.1.2	Registration and Discovery procedure for Vertical Federated Learning when untrusted AF is acting as the VFL server

The procedure below shows registration and discovery for VFL training and inference when the untrusted AF is the VFL server. There can be multiple NWDAFs as VFL clients.

NOTE 1:	For a deployment scenario that untrusted AF is the VFL server and only one NWDAF will be an VFL client, it is assumed that VFL client information for an Analytic ID is configured in NEF and step 6 and 7 may be skipped in Figure 6.2H.2.1.2-1.

Figure 6.2H.2.1.2-1: Registration and Discovery procedure for Vertical Federated Learning when an untrusted AF is acting as VFL server and NWDAF(s) are the VFL clients

Steps 1 to 3 are the NWDAF and AF Registration procedures when the VFL server is untrusted AF.

1.	Same as the step 1b, in clause 6.2H.2.1.1, NWDAF as VFL client registers to NRF with its NF profile, it may include those analytics IDs on which it supports to do VFL with AF as VFL server.

NOTE 1:	The AF can use non-standardized values of the Analytics ID. The non-standardized values can be used by the AF as the VFL server to initiate the VFL training or VFL inference and these values need to be supported by NWDAFs acting as VFL clients. It is the operator´s responsibility to guarantee that non-standardized Analytics ID values within a PLMN are unique. Based on operator local policies, the AF may register its capability as VFL server for an Analytics ID to NEF via OAM configuration, this is used to make it possible for NWDAF to trigger the AF as VFL server to train a model by initating VFL training.

2-3.	Same as the steps 2-3, in clause 6.2H.2.1.1.

Steps 4-10 are the NWDAF Discovery procedures when the VFL server is an untrusted AF.

4.	Untrusted AF acting as the VFL server determines that VFL operations are required and the NWDAF(s) as VFL client(s) are required. The AF sends a Nnef_NFDiscovery_Request for the VFL client(s) to the NEF and for each client provides selection criteria: Analytics ID, required NF type (i.e. NWDAF type), VFL capability type (i.e. VFL client), VFL interoperability indicator(s), Time Period of Interest, optional required feature ID(s), and optional Service Area.

NOTE:	Step 4 in Figure 6.2H.2.1.2-1 may be triggered at the VFL Server by itself or when requested from a consumer.

5.	The NEF checks based on configured policies whether the AF is entitled to request single or multiple VFL client(s) for the Analytics ID.

6-7.	The NEF discovers VFL client(s) (i.e. NWDAF(s)) on behalf of the AF from the NRF by invoking the Nnrf_NFDiscovery_Request using the selection criteria provided by the AF as defined in step 4.

8.	The NEF selects NWDAF(s), e.g., by using the NWDAF(s) NF profile and parameters including load/priority/capacity that are capable of acting as VFL client(s) and matching the received selection criteria such as location information. The NEF anonymizes NWDAF instances ID(s) and assigns external NWDAF ID(s) for each selected NWDAF instance as VFL client. The NEF stores the external NWDAF ID(s) together with information how to reach the NWDAFs.

NOTE 2:	The external NWDAF ID assigned by NEF is temporary and can be released if needed.

9.	The NEF sends Nnef_NFDiscovery_Request response only including the external NWDAF ID(s) as selected in step 8 and information how they relate to corresponding selection criteria provided by the AF, VFL interoperability indicator(s), Time Period of Interest, optional supported feature ID(s) and optional Service Area to the untrusted AF.

10.	The AF stores the received external NWDAF ID(s) and uses it subsequent interactions with the NEF to indicate the target VFL client.

NOTE 3:	If the VFL client NWDAFs are statically preconfigured to the NEF and untrusted AF and not released, the discovery procedure can be skipped.

The AF may indicate to the NEF that the AF will no longer use the VFL process identified by VFL Correlation ID (external NWDAF IDs are no longer required). If the NEF receives this indication, it shall remove the stored external NWDAF ID(s) associated with the VFL correlation ID allocated to the AF. Then, the NEF responds to the AF.

* * * End of Changes * * *