3GPP TSG-WG SA2 Meeting # 167	S2-2501878

# 17 – 21 February, 2025, Athens, Greece

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1591 | rev | - | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on DC App Info mismatches in ADC procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC |  |  |  |  |  | Date: |  |  | 2025-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to clause AC.5 of this specification, the DC application profile or DC control policy associated with a DC application binding information is maintained in DCSF, and the DC application binding information is provided to the UE via Bootstrap DC when the DC application is downloaded to the UE. [Clause AC.5 in TS 23.228 V18.8.0] The binding information is maintained by the HPLMN or retrieved from DC application provider when a DC Application is uploaded to DCSF. DCSF may be configured with a DC application profile associated with the binding information, which indicates the DC control policy (e.g. whether the Application DC establishment follows the P2P, P2A/A2P or P2A2P procedure). The UE receives the binding information of a DC application via the Bootstrap DC, e.g. when the DC application is downloaded to the UE. The DC application profile maintenance or DC control policy configuration will be conducted by HPLMN operator or the DC application provider, and it is dependent to UE’s DC application downloading. Therefore, the UE may initiate an Application DC establishment procedure with the outdated DC application binding information, which mismatches with the DC application profile or DC control policy available at DCSF. For example, the UE may request ADC setup for a DC application that is no longer allowed to the UE (or old version of DC App), or the UE may request a type of ADC (e.g. P2P, P2A, P2A2P) that is no longer allowed to the UE. Hence, it is proposed to clarify that the DCSF can decide not to proceed the requsted ADC establishement due to DC application binding information mismatches between the DCSF and the UE, and send a rejection with an appripriate cause to the UE, so that the UE can decide to obtain the latest DC application binding information from the network and reattempt the ADC establishment procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding descriptions about the DCSF’s actions when the DC application binding information provided in the ADC establishment request by the UE and the available information at DCSF do not match. Adding descriptions about the UE’s action when the ADC establishment request is rejected due to DC application binding information mismatches. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent behaviors between networks and UEs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.7.2.1, AC.7.2.2, AC.7.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************** 1st change **************

AC.7.2.1	Person to Person (P2P) Application Data Channel Setup

Figure AC.7.2.1-1 depicts a signalling flow diagram for establishing an application data channel in a person-to-person use case. In this scenario, the MF is not used to anchor the Application data channel.

In the call flow the UEs have already established an IMS audio session, and the originating UE is updating the IMS audio/video session to an IMS data channel session.

Figure AC.7.2.1-1: Person-to-Person Application Data Channel set up Signalling Procedure

The steps in the call flow are as follows:

0.	IMS session and bootstrap data channel have been established. Selected data channel application(s) have been downloaded to UE#1 and possibly UE#2.

1.	UE#1 sends the SIP reINVITE request with an updated SDP to IMS AS, through originating network P-CSCF and S-CSCF. The updated SDP contains the bootstrap data channel information, as well as the requested application data channel and the associated DC application binding information, according to TS 26.114 [76].

NOTE 1:	The SIP re-INVITE can be initiated either from UE#1 or UE#2.

2.	The IMS AS validates user subscription data to determine whether the media change request event should be notified to the DCSF.

3.	The IMS AS notifies the DCSF, via Nimsas_SessionEventControl_Notify (MediaChangeRequest Event, Calling ID, Called ID, Session ID, Event Direction, Media Info List) of the media change request event.

4.	After receiving the session event notification, the DCSF determines the policy about how to process the application data channel establishment request based on the related parameters (i.e. associated DC application binding information) in the notification and/or DCSF service specific policy.

If the DCSF determines not to proceed with the requested application data channel establishment due to mismatches between the DC application binding information sent from the UE and the DC application profile or the DC control policy available at the DCSF, then the DCSF may reject the application data channel establishment request with an appropriate cause.  In this case, the IMS AS forwards the rejection and cause via the modified SDP answer for ADC in step 14.

NOTE X:	It is assumed that the DC application profile or the DC control policy associated with a DC application binding information can be changed at the DCSF, independently of whether the DC application binding information has already been provided to the UE.

5.	The DCSF determines that the added application data channel media descriptor of the SDP offer takes UE#2 as target endpoint and does not require anchoring on the local MF If MF needs to anchor application data channel, DCSF would have used the Nimsas_MediaControl service operation to instruct IMS AS to allocate data channel media resources of the MF.

6.	DCSF responds to the notification received in step 3.

7-8.	IMS AS sends the re-INVITE to the originating S-CSCF and then to the terminating network side and UE#2.

9-11.	UE#2 and terminating network returns a 200 OK response with SDP answer for application data channel to originating network. Based on the received DC application binding information in the SDP offer of the re-INVITE, UE#2 may need to download the corresponding DC Application signalled in the SDP offer, if not done already and associate it with the requested application DC.

NOTE 2:	The UE at the terminating side is capable to determine if to use the DC application based on the received DC application binding information.

12.	IMS AS notifies the DCSF of the successful data channel modification.

13.	DCSF responds to the notification received in step 12.

14-15.	The IMS AS sends 200 OK response to the originating S-CSCF and P-CSCF.

16.	The originating network P-CSCF executes QoS procedure for application data channel media based on the SDP answer information from the 200 OK response.

17.	P-CSCF returns the 200 OK response to UE#1.

If the UE receives a rejection and the cause of application data channel establishment due to DC application binding information mismatched, as in step 4, the UE may decide to request the data channel application or the data channel application list through an existing bootstrap data channel to obtain the latest DC application binding information, and reattempt the ADC establishement.

18.	UE#1 sends ACK to the terminating network.

19.	The application data channel between UE#1 and UE#2 is established. In this example, it is not anchored in MF.

************** 2nd change **************

AC.7.2.2	Person-to-Application (P2A) Application Data Channel Setup

Figure AC.7.2.2-1 depicts a signalling flow diagram for establishing an Application Data Channel in a person to application use case.

Figure AC.7.2.2-1: Person-to-Application (P2A) Application Data Channel set up Signalling Procedure

The steps in the call flow are as follows:

0-3.	Steps 0-3 of clause AC.7.2.1 applies.

NOTE 1:	The SIP re-INVITE can be initiated either from UE#1 or UE#2.

4.	After receiving the session event notification, the DCSF determines the policy about how to process the application data channel establishment request based on the related parameters (i.e. associated DC application binding information) in the notification and/or DCSF service specific policy.

If the DCSF determines not to proceed with the requested application data channel establishment due to mismatches between the DC application binding information sent from the UE and the DC application profile or the DC control policy available at the DCSF, then the DCSF may reject the application data channel establishment request with an appropriate cause.  In this case, the IMS AS forwards the rejection and cause via the SDP answer for ADC in step 22.

NOTE Y:	It is assumed that the DC application profile or the DC control policy associated with a DC application binding information can be changed at the DCSF, independently of whether the DC application binding information has already been provided to the UE.

5.	The DCSF determines that the added Application Data Channel media of the offer takes DC Application Server as target endpoint and requires to anchor in the MF.

6.	The DCSF invokes Nimsas_MediaControl service operation to instruct IMS AS to terminate the media flow of the originating UE to MF. The instruction also includes information to be consumed by the MF that the data channel media shall be relayed via the MDC2 interface.

7.	The IMS AS invokes Nmf_MRM_Create(List of Media Termination Descriptors) service operation to instruct MF on application data channel establishment and data channel media resource reservation based on the DC media information received from DCSF.

8.	The IMS AS notifies the MediaControl instruction control response to DCSF.

9.	The DCSF stores the media resource information and sends a P2A application data channel establishment request (including the MDC2 media resource received from MF) to the DC Application Server via DC3/DC4.

10.	DC Application Server accepts the P2A application data channel establishment request, returning an MDC2 reserved media resource as answer and is prepared for UE#1 traffic through MDC2.

NOTE 2:	Details on how DCSF communicates with the DC Application Server is out of scope of this Release.

11.	DCSF requests IMS AS to update the MF resource with MDC2 media endpoint information of DC Application Server.

12.	IMS AS updates the MF resource.

13.	IMS AS notifies the MediaControl instruction control response to DCSF.

14.	DCSF replies to the notification received in step 13.

15-16.	IMS AS sends the re-INVITE to remote network side and UE#2, via the originating S-CSCF, which does not include application data channel request in the SDP for the application data channel.

17-19:	UE#2 and terminating network returns a 200 OK response with SDP answer for audio/video.

20.	IMS AS notifies the DCSF about the successful result of the MediaChangeRequest event.

21.	DCSF replies to the notification.

22-23.	The IMS AS includes SDP answer for application data channels to UE#1 in 200 OK response and sends 200 OK response to S-CSCF and P-CSCF.

24.	The originating network P-CSCF executes QoS procedure for application data channel media based on the SDP answer information from the 200 OK response.

25.	CSCF returns the 200 OK response to UE#1.

If the UE receives a rejection and the cause of application data channel establishment due to DC application binding information mismatched, as in step 4, the UE may decide to request the data channel application or the data channel application list through an existing bootstrap data channel to obtain the latest DC application binding information, and reattempt the ADC establishement.

26.	UE#1 send ACK to the terminating network.

27.	The application data channel between UE#1 and DC Application Server is established via MF. MF forwards data channel traffic between UE#1 and DC Application Server based on MDC2 media point information received in step 9 and 12.

************** 3rd change **************

AC.7.2.3	Person-to-Application and Application-to-Person (P2A2P) Procedure

This procedure enables originating and terminating UE to establish application data channels for the same application to communicate with the same Data Channel Application Server.

The P2A2P procedure requires the establishment of application data channels from UE#1 to MF and from UE#2 to MF. The two application data channels are associated with the same application. This enables UE#1 and UE#2 to interact with the same DC Application Server simultaneously and the DC Application Server to correlate the data exchanged with both UEs. The P2A procedures as described in clause AC.7.2 are used to establish one application data channel between UE#1 and MF and, for A2P scenario, one application data channel between UE#2 and MF. In case of two involved UEs, this enables independent communication between UE#1 and UE#2 with the DC Application Server.

Figure AC.7.2.3-1: Symmetric Application Data Channel Establishment

0.	IMS session and bootstrap data channels are established. Selected data channel applications are downloaded to UE#1 and possibly UE#2.

1.	UE#1 sends SIP re-INVITE request with an updated SDP to IMS AS. The updated SDP contains the bootstrap data channel, the application data channel information and associated DC application binding information.

NOTE 1:	The SIP re-INVITE can be initiated either from UE#1 or UE#2.

2.	IMS AS validates the data channel media description information and/or user subscription data to determine whether the DCSF needs to be notified.

3.	IMS AS selects and notifies the DCSF about the call event and data channel establishment request.

4.	The DCSF determines how to process the application data channel establishment request based on the parameters (i.e. associated DC application binding information) in the notification from IMS AS and/or DCSF service specific policy.

If the DCSF determines not to proceed with the requested application data channel establishment due to mismatches between the DC application binding information sent from the UE and the DC application profile or the DC control policy available at the DCSF, then the DCSF may reject the application data channel establishment request with an appropriate cause.  In this case, the IMS AS forwards the rejection and cause via the SDP answer for ADC in step 20.

NOTE Z:	It is assumed that the DC application profile or the DC control policy associated with a DC application binding information can be changed at the DCSF, independently of whether the DC application binding information has already been provided to the UE.

5.	DCSF determines that the added application data channel media in the SDP offer requires the DC Application Server is the endpoint for both originating and terminating UE and that the application DC must be anchored at the MF. DCSF communicates with the DC Application Server for DC resource control. Once the application data channel is established, the DC Application Server will send/receive traffic to/from UE#1 through the MDC2 interface.

NOTE 2:	Details on how the DCSF communicates with the DC Application Server are out of scope of this release.

6.	DCSF invokes Nimsas_MediaControl service to send data channel control request to IMS AS, including information how to relay data channel media via the MDC2 interface.

7.	IMS AS reserves data channel media resources at the MF via DC2 based on the DC media information received from DCSF. These requests by the IMS AS, are used to create two associated resources (one for the originating UE, and one for the terminating UE) in the MF.

8.	IMS AS notifies the DCSF about MediaControl instruction control response.

9.	DCSF communicates with the DC Application Server for DC resource control and provides information on data channel media resources reserved at the MF.

10.	The DCSF stores the media resource information and replies to the Nimsas notification request.

11-12.	IMS AS sends re-INVITE which include the SDP offer from MF for the application data channel to the originating S-CSCF and then to the remote network and UE#2.

13-16.	UE#2 and terminating network return 200 OK with SDP answer for audio/video and for the application data channel. The terminating network P-CSCF executes QoS procedure for application data channel media based on the SDP in the 200 OK. Based on the received DC Application binding information in the re-INVITE, UE#2 may need to download the corresponding DC Application signalled in the SDP offer, if not done already and associate it with the requested application DC.

NOTE 3:	The UE at the terminating side is capable to determine if to use the DC application based on the received DC application binding information.

17.	IMS AS notifies the DCSF about the successful result of the MediaChangeRequest event.

18.	DCSF updates media resources in the MF via IMS AS.

19.	DCSF replies to the notification received in step 17.

20-21.	The IMS AS includes SDP answer for application data channel to UE#1 in 200 OK and sends 200 OK to S-CSCF and P-CSCF.

22.	The originating network P-CSCF executes QoS procedure for application data channel media based on the SDP in the 200 OK.

23.	P-CSCF returns the 200 OK to UE#1.

If the UE receives a rejection and the cause of application data channel establishment due to DC application binding information mismatched, as in step 4, the UE may decide to request the data channel application or the data channel application list through an existing bootstrap data channel to obtain the latest DC application binding information, and reattempt the ADC establishement.

24.	UE#1 sends ACK to the terminating network.

25.	The application data channel between UE#1 and DC Application Server is established via MF. MF forwards data channel traffic between UE#1 and DC Application Server via MDC2.

26.	The application data channel between UE#2 and DC Application Server is established via MF. MF forwards data channel traffic between UE#2 and DC Application Server via MDC2.

************ End of changes ************