3GPP SA WG2 Meeting #167	S2-2501900

# Athens, 17 – 21 February 2025

Agenda item:	19.50

Source:	Nokia, ZTE

Title:	Emergency Services and Inactive eDRX

WID/SID:	NR_newRAT-Core, NR_redcap-Core

Document for:	Discussion and Decision

# 1	Introduction

At SA2#166AH-e, SA2 received the LS in [1] from RAN2 reporting an issue when eDRX feature is combined with RRC_INACTIVE feature.

The LS states:

In RAN2#128, companies discussed a scenario where a UE is configured with a RAN eDRX cycle in RRC_INACTIVE state when the UE has a PDU session associated with emergency services. RAN2 notice the below description in TS 23.501 which states that an Idle eDRX cycle should not be used by the UE when it has a PDU session associated with emergency services. However, it is unclear whether it is possible for such UE to be released to RRC_INACTIVE state with a RAN eDRX cycle and if it is so how UE behavior may be.

Based on current specification, a RAN eDRX cycle can be configured by a RAN node when releasing the UE to RRC_INACTIVE state only if the UE is configured with an Idle eDRX cycle by CN.

RAN2 assumes that RAN node should be aware whether a UE has a PDU session associated with emergency services so that it can avoid releasing such UE to RRC_INACTIVE state with a RAN eDRX cycle, or even if it is so, there should be means for the RAN node and the UE to not use the configured RAN eDRX cycle. Regarding how RAN node can be aware of such PDU session, RAN2 understands that CN can already indicate this (e.g. via ARP value) but there is no requirement for CN to do this, so, there is case that RAN node may not be aware of this.

RAN2 would like SA2 and CT1 to consider the scenario described above, provide feedback regarding how the identified issue can be addressed and whether any standard solution is needed?

# 2	Actions

To SA2, CT1 group

ACTION: 	RAN2 would like SA2 and CT1 to consider the scenario described above, provide feedback regarding how the identified issue can be addressed and whether any standard solution is needed?

The action is directed to SA2, and the technical discussion took place during SA2#166AH-e, but no consensus was reached. Therefore, this discussion paper has been created.

# 2	Discussion

- Requirements

The main question is whether it is acceptable to permanently disable extended eDRX for voice-centric UEs that may need to receive callbacks within a reasonable time, given that extended eDRX can last up to three hours.

Permanently disabling eDRX would lead to faster battery drain. While this may be acceptable for smartphones—which are less likely to use extended eDRX—it may not be suitable for other types of device.

Certain RedCap devices capable of handling emergency calls may be configured to use eDRX. Examples include:

- Senior Medical Alert Systems – Simplified wearable devices, such as those worn on a neck strap or in the form of a key ring, that enable periodic geographical position reporting during normal operation with long update intervals. In emergencies, they can automatically or manually establish immediate emergency calls.

- Emergency Buttons – Simplified cellular devices permanently attached to building infrastructure or public transport vehicles (including autonomous vehicles). These devices are equipped with a ‘red button’ used to initiate emergency calls. Automatic activation of emergency calls and activation by ‘red button’ are mentioned in 3GPP TS 22.101 V19.0.0 Clause 10.1.

Such devices may utilize eDRX to support delay-tolerant MT communication for remote management and status monitoring while ensuring long battery life.

Given these requirements, solutions must ensure that extended eDRX is only temporarily disabled when a PS callback is expected, rather than being permanently disabled.

The duration of this temporary eDRX disablement is recognized as a matter of regulatory policy.

- Scenarios

There are two cases:

Case 1: emergency PDU session activated

As long as the UE is in RRC_CONNECTED state and the emergency PDU session is active, the gNB has the PDU session context, including the ARP of QCI 5, which indicates emergency traffic. Therefore, the gNB should refrain from transitioning the UE to the eDRX RRC_INACTIVE state.

However, while this behavior may seem common sense for the gNB, the caveat is that there is currently no explicit specification in the 3GPP standards that mandates the gNB to act this way.

Another issue is that even if the gNB is configured to disable inactive eDRX while the emergency PDU session is active, this solution assumes that the UE remains RRC-connected during the callback period. However, this period can be quite long, depending on regulations. For example, RFC 7090 Clause 1 mentions a typical duration of 30 minutes. See the extract below:

Case 2: emergency session deactivated

For example, the following scenario could happen:

- The UE has both an emergency PDU session and another PDU session (PDU session 1).

- After the user originates an emergency call, the emergency PDU session is deactivated while the emergency PDU session is ongoing at least until a PSAP callback occurs.

- In the meantime, the inactivity timer for PDU session 1 expires. Since the gNB is only aware of PDU session 1 at this point (it no longer retains context for the emergency PDU session), it treats the UE as inactive and transitions it to RRC_INACTIVE with eDRX enabled.

- When the PSAP callback arrives from the 5GC, the UE experiences a delay due to inactive eDRX.

This scenario could happen because of TS 23.501 section 5.6.8 which does not forbid SMF to deactivate emergency PDU sessions. Even if SA2 specification change, there can be SMFs which behave as such.

- Solutions

Solution 1: SMF-only solution

The SMF behavior could be modified to never deactivate an emergency PDU session. However, this does not necessarily mean that the emergency PDU session would be reactivated immediately when the UE transitions to RRC_CONNECTED, bringing us back to Case 1.

For example, the following scenario could happen:

- The UE has both an emergency PDU session and another PDU session (PDU session 1).

- After the user originates an emergency call, the emergency PDU session is not deactivated while the emergency PDU session is ongoing at least until a PSAP callback occurs.

- In the meantime, the UE goes idle (e.g., the gNB releases the UE due to a lost connection), causing the gNB to lose all session context.

- The UE reconnects and sends a service request for PDU session 1 (e.g., an internet connection). Since the gNB is only aware of PDU session 1, it does not recognize the emergency PDU session.

- Due to PDU session 1 becoming inactive, the gNB transitions the UE to RRC_INACTIVE with eDRX enabled.

- When the PSAP callback arrives from the 5GC, the UE experiences a delay due to inactive eDRX.

Observation 1: SMF-only solution does not solve the problem.

Solution 2: SMF + AMF solution (always-on like solution)

Currently, the AMF is aware whether there is an ongoing emergency PDU session: this can be seen from the note 7 in TS 23.502 clause 4.2.2.2:

NOTE 7:	The new AMF can determine if a PDU Session is used for emergency service by checking whether the DNN matches the emergency DNN.

This information is stored in UE context because we can see that it is transferred later on to the SMF during mobility in TS 23.502 clause 4.9.1.3.2 step 3:

3.	[Conditional] S-AMF to T-AMF: Namf_Communication_CreateUEContext Request (N2 Information (Target ID, Source to Target transparent container, SM N2 information list, PDU Session IDs), UE context information (SUPI, Service area restriction, Allowed NSSAI for each Access Type and Partially Allowed NSSAI if available, Tracing Requirements, LTE M Indication, the list of PDU Session IDs along with the corresponding SMF information and the corresponding S-NSSAI(s), PCF ID(s), DNN, UE Radio Capability ID and UE Radio Capability Information, N2 Notify URI). If the subscription information includes Tracing Requirements, the old AMF provides the target AMF with Tracing Requirements.

Observation 2: AMF is aware when an ongoing PDU session is an emergency PDU session

In order to complement the above SMF-only solution, we could have a change in AMF to behave like always-on for this emergency PDU session. This is to say that whenever the UE gets RRC connected and AMF receives a service request, if the AMF has knowledge of an emergency PDU session (see observation 2), the AMF asks SMF to reactivate it.

The required change for AMF would be to modify TS 23.502 clause 4.2.3.2 (UE triggered service request) because currently AMF asks SMF to reactivate PDU sessions only based on UE trigger as can be seen at step 4:

The Nsmf_PDUSession_UpdateSMContext Request is invoked:

-	If the UE identifies List Of PDU Sessions To Be Activated in the Service Request message;

Observation 3: AMF+SMF solution can work to fall back to “activated” case 1. However, its effectiveness would still depend on the gNB's behavior, just as in Case 1.

Solution 3: AMF-only solution (disable inactive eDRX in the gNB)

As shown before, the AMF is aware when there is an ongoing emergency PDU session.

The AMF may not always be aware of whether the SMF has deactivated the emergency PDU session. However, a simple rule could be that the AMF disables inactive eDRX mode in the gNB as soon as or as long as there is an ongoing emergency PDU session, regardless of whether it is deactivated or not.

No additional signaling is needed for this because the AMF can already control inactive eDRX in the gNB by sending a UE Context Setup/Modification Request, which either includes or omits the NR eDRX Information IE.

Observation 4: AMF-only solution does not require new signaling.

Another advantage of the AMF-only solution is that it does not rely on gNB implementation, since the gNB cannot transition the UE to inactive eDRX if the NR eDRX Information IE is not present.

Observation 5: no impact or dependency foreseen on gNB for the AMF-only solution.

# 3	Conclusion and Proposals

It has identified three possible solutions and has made the following observations:

Observation 1: SMF-only solution does not solve the problem.

Observation 2: AMF is aware when an ongoing PDU session is an emergency PDU session

Observation 3: AMF+SMF solution can work to fall back to “activated” case 1. However, its effectiveness would still depend on the gNB's behavior, just as in Case 1.

Observation 4: AMF-only solution does not require new signaling not any coordination.

Observation 5: no impact or dependency foreseen on gNB for the AMF-only solution.

Therefore, it is proposed to answer the received LS and agree with the CRs based on the solution 3:

Proposal 1: decide to support the Solution 3: AMF-only solution (disable inactive eDRX in the gNB)

Proposal 2: agree the reply LS in [2] to RAN2.

# 4	References

- S2-2501367/R2-2411041, LS on Emergency Services and eDRX

- S2-2501899, Reply LS on emergency services and eDRX