SA WG2 Meeting #167	S2-2501947

17 - 21 February, 2025, Athens, Greece	(revision of S2-25xxxxx)

Source:	ZTE

Title:	pCR to TS 23.xyz: Procedure for Inventory

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT / Rel-19

Abstract of the contribution: The paper provides high level procedure for inventory service.

1. Discussion

The paper provides procedure for AIoT inventory service.

2. Proposal

It is proposed to capture the following changes into TS 23.xyz.

* * * * Start of Changes * * * *

6.2	Procedure for Inventory

Figure 6.2-1 illustrates an inventory procedure for AIoT Devices.

Figure 6.2-1: Procedure for Inventory Service

The steps in the AIoT Device inventory procedure are as follows:

1.	The AF initiates inventory request to the NEF. The inventory request includes the service area information or device(s) information.

2.	The NEF discovers the AIOTF information from the NRF based on the service area information received in the request.

NOTE 1:	It is assumed AIOTF already registers its information and coverage area into NRF.

3.	The NEF sends inventory request to the AIOTF. If the request from the AF includes device information, the NEF may translate it to Device ID.

NOTE 2:	The request could be sent to multiple AIOTFs which are discovered in step.2 based on the area information.

4.	The AIOTF interacts with the ADM to authorize the service request form the AF; if the authorization is successful the AIOTF further discovers destination AIoT RAN Reader based on area information or device information received in inventory request.

The AIOTF may also finds the destination AIoT RAN Reader based on the last serving reader information of the AIoT Device stored in the ADM.

5.	The AIOTF sends inventory response to the AF through the NEF to indicate the inventory service request is accepted.

6.	The AIOTF sends the inventory request to the AIoT RAN Reader(s).

7.	The AIoT RAN Reader sends a paging message including the identifiers of the destination AIoT Devices; and the AIoT Devices matching the identifiers performs random access procedure with the AIoT RAN Reader.

8.	The AIoT Device sends back the inventory result including its Device ID to the AIOTF via AIoT Device NAS message which is transferred by the AIoT RAN Reader.

9.	The AIOTF stores the AIoT Device status including the last serving reader information in the ADM based on the received inventory result.

The AIOTF may also store the AIoT Device status if it supports. When there are multiple AIOTFs deployed in the network, the AIOTF may share the AIoT Device status.

10.	The AIOTF reports the inventory result to the NEF.

11.	The NEF transfers the inventory result to the AF.

NOTE 3:	For the indirect interface architecture, in step.4 the AIOTF also needs to discover the AMF serving the AIoT RAN Reader; and in step.6 the AIOTF sends the inventory request to the AIoT RAN Reader via the AMF.

* * * * End of Changes * * * *