3GPP TSG-WG SA2 Meeting #167	S2-2501973

Athens, GR, 17th Feb – 21st Feb, 2025	(revision of S2-2500931)

Source:	Huawei, HiSilicon

Title:	pCR to TS 23.xyz: Network Functions Services

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT / Rel-19

Abstract: add the services for AMF, NEF, AIOTF, ADM in respect to Ambient IoT.

1. Discussion

Follow the conclusion in clause 8.3 and 8.1.2.3 of TR 23700-13, this proposal introduces the support of Ambient IoT services for AMF, NEF, AIOTF, ADM. This includes:

1) clause 8.3.3: NEF exposure:

NEF supports to expose Ambient IoT service towards the AF.

-	New NEF services (e.g. Nnef_AIoT) to support the AIoT services including Service Operations of Inventory, Read, Write, and Permanent Disable will be defined.

2) clause 8.3.5: Information provided by the AF

The AF provides one or some of the following information:

-	Information about AIoT services:

-	Service type indicating e.g. inventory or command (e.g. read or write).

-	Information to be used for AIoT reader selection:

-	At least one of the UE reader ID or the target area information.

-	Optionally, Information about the target AIoT device(s):

-	At least one of the AIoT device ID(s) or the filtering information which can be used to associate with multiple AIoT devices.

-	Optionally, Information to be used for resource allocation that is detailed in TR 38.769 [8]:

-	Approximate number of AIoT devices.

-	Approximate D2R message size.

3) clause 8.1.2.3: AIOT RAN and the AIOTF communicate indirectly via an AMF

When AIOT RAN and the AIoTF communicate indirectly via an AMF:

-	The AIoTF connects with AIOT RAN via an AMF. NGAP over the N2 reference point between the AIOT RAN and AMF supports Ambient IoT services including delivery of inventory/command messages.

-	NGAP between AIOT RAN and the AMF is enhanced to support Ambient IoT Services.

-	The AMF shall be enhanced to support:

-	On the Nz reference point, the AMF supports Services which are used by an AIoTF for Ambient IoT Operations e.g. to send AIoT requests towards AIOT RAN and to receive AIoT responses from the AIOT RAN.

-	On the N2 reference point, the AMF supports sending AIOT information to the AIOT RAN (e.g. operation requests) and receiving responses from the AIOT RAN.

-	The AMF routes the AIoT messages between the AIOT RAN (over N2 reference point) and the AIOTF (over Nz reference point).

4) Clause 8.2.2:	Subscription Management

The Ambient IoT Device ID is used by the AIOTF together with local configuration, 3rd party related context to locate the entity storing the AIoT device profile data of an Ambient IoT Device. The AIoT device profile data includes the AIoT Device ID. When the AIoT device profile data is managed by the 5GC, it is stored in the ADM (Ambient IoT Data Management), possibly together with UDR, which exclusively supports AIoT devices.

2. Text Proposal

It is proposed to capture the following changes vs. TS 23.xyz.

# * * * * First change (all new texts) * * * *

7.1	General

In the context of Ambient IoT specification, an NF service is offering a capability to authorised upstream and downstream NF consumers, thus to enable the network to offer the end-to-end Ambient IoT services from system perspective.

The following clauses provide for each involved NF the NF services it exposes through its service-based interfaces.

7.2	AIOTF service

The following table shows the Naiotf_AIoT Service and Service Operations.

Table 7.2-1: NF services provided by AIOTF

| NF service name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Naiotf_AIoT | Inventory | Request/Response | NEF |
|  | Read | Request/Response | NEF |
|  | Write | Request/Response | NEF |
|  | PermanentDisable | Request/Response | NEF |
|  | Notify | Subscribe/Notify | NEF |

7.2.1	Naiotf_AIoT_Inventory service operation

Service operation name: Naiotf_AIoT_Inventory

Description: the NF consumer requests to perform an inventory device operation for an AIoT device or a group of AIoT devices.

Inputs, Required:

1) Information to identify the requested AF:

-	AF ID.

2) Information to be used for AIoT reader selection:

-	At least the target area information.

Inputs, Optional:

1) Information about the target AIoT device(s):

-	At least one of the AIoT device ID(s) or the filtering information which can be used to associate with multiple AIoT devices.

2) Information to be used for resource allocation:

-	Approximate number of Ambient IoT devices.

Outputs, Required: Result indication (Success or Failure), Failure Cause in case of Failure, Correlation ID.

7.2.2	Naiotf_AIoT_Read service operation

Service operation name: Naiotf_AIoT_Read

Description: the NF consumer requests to perform a read device operation for an AIoT device or a group of AIoT devices.

Inputs, Required:

1) Information to identify the requested AF:

-	AF ID.

2) Information to be used for AIoT reader selection:

-	At least the target area information.

3) Read command specific parameters:

-	AIoT Data specific for the Read Command to send to the target AIoT device(s).

-	Response AIoT data size.

Inputs, Optional:

1) Information about the target AIoT device(s):

-	At least one of the AIoT device ID(s) or the filtering information which can be used to associate with multiple AIoT devices.

2) Information to be used for resource allocation:

-	Approximate number of Ambient IoT devices.

Outputs, Required: Result indication (Success or Failure), Failure Cause in case of Failure, Correlation ID.

7.2.3	Naiotf_AIoT_Write service operation

Service operation name: Naiotf_AIoT_Write

Description: the NF consumer requests to perform a write device operation for an AIoT device or a group of AIoT devices.

Inputs, Required:

1) Information to identify the requested AF:

-	AF ID.

2) Information to be used for AIoT reader selection:

-	At least the target area information.

3) Write command specific parameters:

-	AIoT Data specific for Write Command to send to the target AIoT device(s).

Inputs, Optional:

1) Information about the target AIoT device(s):

-	At least one of the AIoT device ID(s) or the filtering information which can be used to associate with multiple AIoT devices.

2) Information to be used for resource allocation:

-	Approximate number of Ambient IoT devices.

Outputs, Required: Result indication (Success or Failure), Failure Cause in case of Failure, Correlation ID.

7.2.4	Naiotf_AIoT_PermanentDisable service operation

Service operation name: Naiotf_AIoT_PermanentDisable

Description: the NF consumer requests to perform a permanent disable device operation for an AIoT device or a group of AIoT devices.

Inputs, Required:

1) Information to identify the requested AF:

-	AF ID.

2) Information to be used for AIoT reader selection:

-	At least the target area information.

3) Permanent disable command specific parameters:

Inputs, Optional:

1) Information about the target AIoT device(s):

-	At least one of the AIoT device ID(s) or the filtering information which can be used to associate with multiple AIoT devices.

2) Information to be used for resource allocation:

-	Approximate number of Ambient IoT devices.

Outputs, Required: Result indication (Success or Failure), Failure Cause in case of Failure, Correlation ID.

7.2.5	Naiotf_AIoT_Notify service operation

Service operation name: Naiotf_AmbientIoT_Notify

Description: This service operation allows the consumer to receive the status or results of the requested service operation. If the NF consumer invokes the Naiotf_AIoT_Inventory, or Naiotf_AIoT_Read or Naiotf_AIoT_Write or Naiotf_AIoT_PermanentDisable service operation, the NF consumer implicitly subscribes to receive the status or results of the requested service operation

Inputs, Required: Correlation ID, Device Operation Execution Result indication (Success or Failure), List of Ambient IoT Device ID, Failure Cause in case of Failure. In case of Naiotf_AIoT_Read, AIoT Data read from each target Ambient IoT Device.

Outputs, Required: Operation execution result indication.

7.3	NEF services

The following table shows the Nnef_AIoT Service and Service Operations, in addition to the NEF services offered as defined in TS 23.502 clause 5.2.6.

Table 7.3-1: NF services provided by NEF

| NF service name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnef_AIoT | Inventory | Request/Response | AF |
|  | Read | Request/Response | AF |
|  | Write | Request/Response | AF |
|  | PermanentDisable | Request/Response | AF |
|  | Notify | Subscribe/Notify | AF |

7.3.1	Nnef_AIoT_Inventory service operation

Service operation name: Nnef_AIoT_Inventory

Description: the NF consumer requests to perform an inventory device operation for an AIoT device or a group of AIoT devices.

Inputs, Required:

1) Information to identify the requested AF:

-	AF ID.

2) Information to be used for AIoT reader selection:

-	At least the target area information.

Inputs, Optional:

1) Information about the target AIoT device(s):

-	At least one of the AIoT device ID(s) or the filtering information which can be used to associate with multiple AIoT devices.

2) Information to be used for resource allocation:

-	Approximate number of Ambient IoT devices.

Outputs, Required: Result indication (Success or Failure), Failure Cause in case of Failure, Transaction ID.

7.3.2	Nnef_AIoT_Read service operation

Service operation name: Nnef_AIoT_Read

Description: the NF consumer requests to perform a read device operation for an AIoT device or a group of AIoT devices.

Inputs, Required:

1) Information to identify the requested AF:

-	AF ID.

2) Information to be used for AIoT reader selection:

-	At least the target area information.

3) Read command specific parameters:

-	AIoT Data specific for the Read Command to send to the target AIoT device(s).

-	Response AIoT data size.

Inputs, Optional:

1) Information about the target AIoT device(s):

-	At least one of the AIoT device ID(s) or the filtering information which can be used to associate with multiple AIoT devices.

2) Information to be used for resource allocation:

-	Approximate number of Ambient IoT devices.

Outputs, Required: Result indication (Success or Failure), Failure Cause in case of Failure, Transaction ID.

7.3.3	Nnef_AIoT_Write service operation

Service operation name: Nnef_AIoT_Write

Description: the NF consumer requests to perform a write device operation for an AIoT device or a group of AIoT devices.

Inputs, Required:

1) Information to identify the requested AF:

-	AF ID.

2) Information to be used for AIoT reader selection:

-	At least the target area information.

3) Write command specific parameters:

-	AIoT Data specific for Write Command to send to the target AIoT device(s).

Inputs, Optional:

1) Information about the target AIoT device(s):

-	At least one of the AIoT device ID(s) or the filtering information which can be used to associate with multiple AIoT devices.

2) Information to be used for resource allocation:

-	Approximate number of Ambient IoT devices.

Outputs, Required: Result indication (Success or Failure), Failure Cause in case of Failure, Transaction ID.

7.3.4	Nnef_AIoT_PermanentDisable service operation

Service operation name: Nnef_AIoT_PermanentDisable

Description: the NF consumer requests to perform a permanent disable device operation for an AIoT device or a group of AIoT devices.

Inputs, Required:

1) Information to identify the requested AF:

-	AF ID.

2) Information to be used for AIoT reader selection:

-	At least the target area information.

3) Permanent disable command specific parameters:

Inputs, Optional:

1) Information about the target AIoT device(s):

-	At least one of the AIoT device ID(s) or the filtering information which can be used to associate with multiple AIoT devices.

2) Information to be used for resource allocation:

-	Approximate number of Ambient IoT devices.

Outputs, Required: Result indication (Success or Failure), Failure Cause in case of Failure, Transaction ID.

7.3.5	Nnef_AIoT_Notify service operation

Service operation name: Nnef_AmbientIoT_Notify

Description: This service operation allows the consumer to receive the status or results of the requested service operation. If the NF consumer invokes the Nnef_AIoT_Inventory, or Nnef_AIoT_Read or Nnef_AIoT_Write or Nnef_AIoT_PermanentDisable service operation, the NF consumer implicitly subscribes to receive the status or results of the requested service operation.

Inputs, Required: Transaction ID, Device Operation Execution Result indication (Success or Failure), List of Ambient IoT Device ID, Failure Cause in case of Failure. In case of Naiotf_AIoT_Read, AIoT Data read from each target Ambient IoT Device.

Outputs, Required: Operation execution result indication.

7.4	AMF services

The following table shows the Namf_AIoT Service and Service Operations. This Namf_AIoT AMF service is used when AIOT RAN and the AIoTF communicate indirectly via an AMF.

Table 7.4-1: NF services provided by AMF

| NF service name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Namf_AIoT | MessageDelivery | Request/Response | AIOTF |
|  | Notify | Subscribe/Notify | AIOTF |

7.4.1	Namf_AIoT_MessageDelivery service operation

Service operation name: Namf_AIoT_MessageDelivery

Description: the NF consumer requests delivery of inventory/command messages towards AIOT RAN or AIoT devices.

Inputs, Required:

1) 	AIoT NGAP Message Type ("Inventory" or "Command").

2) 	AIoT messages to deliver:

-	NGAP INVENTORY REQUEST Message Container.

-	or, NGAP COMMAND REQUEST Message Container.

3) 	AIOTF Identifier, this is to allow the AMF to route back the requesting AIOTF with the response(s) to the delivered request.

4) 	AIOT RAN ID

Outputs, Required: NGAP INVENTORY RESPONSE Container or NGAP COMMAND RESPONSE Container, Result indication (Success or Failure), Failure Cause in case of Failure.

7.4.2	Namf_AIoT_Notify service operation

Service operation name: Namf_AIoT_Notify

Description: the NF consumer requests to receive inventory/command messages from AIOT RAN or AIoT devices. If the NF consumer invokes the Namf_AIoT_MessageDelivery, the NF consumer implicitly subscribes to receive the inventory/command messages from AIOT RAN or AIoT devices.

Inputs, Required:

1) AIoT messages to receive:

-	NGAP INVENTORY Report Message Container.

Outputs, Required: Result indication (Success or Failure), Failure Cause in case of Failure.

7.5	ADM services

The following table shows the Nadm_DM Service and Service Operations.

Table 7.5-1: NF services provided by AIOTF

| NF service name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nadm_DM | Query | Request/Response | AIOTF, NEF |
|  | Update | Request/Response | AIOTF |

7.5.1	Nadm_DM_Query service operation

Service operation name: Nadm_DM_Query.

Description: NF service consumer requests the AIoT device profile data or AF authorization data from the ADM.

Inputs, Required: AIoT Device Permanent ID, or AF ID.

Outputs, Required: Requested AIoT data.

7.5.2	Nadm_DM_Update service operation

Service operation name: Nadm_DM_Update.

Description: NF service consumer intends to update AIoT device profile data or AF authorization data in the ADM.

Inputs, Required: AIoT Device Permanent ID, updated AIoT data

Outputs, Required: Result indication (Success or Failure), Failure Cause in case of Failure.

# * * * * End of changes * * * *