3GPP TSG-WG SA2 Meeting #167	S2-2501978

Athens, GR, 17th Feb – 21st Feb, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	pCR to TS 23.xyz: Updates to Architecture clause

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT / Rel-19

Abstract: Update of clause 4 to include overall architectures, add ADM, and provide descriptions of the NFs along with terminology alignment.

1. Introduction/Discussion

An update of clause 4 architecture model and concepts to:

-	Align terminology (e.g., A-IOT RAN to AIoT RAN, Ambient IoT Device to AIoT Device etc.)

-	Add contents to clause 4.2.1, to provide an overview of which architectures are included (and allowing expansion for future architectures).

-	Add complete service-based and reference architectures for AIoT RAN Readers topology, including ADM etc.

-	Identify which parts of the overall topology 1 architecture apply to direct and indirect path.

-	Add ADM based reference points and service-based interfaces.

-	Descriptions for the NFs in the system, based on the discussion in SA2#166ahe and introduce the ADM storage architecture.

-	Align the protocol stacks to use similar terms, such as Nb/N33 stack, and label reference points with consistent naming to the architecture clause.

2. Text Proposal

It is proposed to capture the following changes vs. TS 23.xyz-101.

# * * * * First change * * * *

1	Scope

The present document specifies architectural enhancements to the 5G system to support Ambient power-enabled Internet of Things, complying to the requirements in TS 22.369 [1] applicable to the AIoT Device types, traffic types, use cases and connectivity topologies defined in TS 38.300 [5].

# * * * * Second change * * * *

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

AIoT Device: An Ambient IoT device is an IoT device powered by energy harvesting, with limited energy storage capability.

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

ADM	AIoT Data Management

AIoT	Ambient IoT

AIOTF	Ambient IoT Function

4	Architecture model and concepts

4.1	General concept

Ambient IoT is a service that can be provided by the 5GS system to support Ambient power-enabled IoT devices that are powered by energy harvesting, being either battery-less or with limited energy storage capability (e.g. using a capacitor) and the energy is provided through the harvesting of radio waves, light, motion, heat, or any other suitable power source.

The 5GS System architecture for Ambient IoT include the following functions and procedures for:

-	AIoT Device identification;

-	AIoT Device inventory;

-	Providing to, and obtaining from, an AIoT Device application data.

4.2	Architecture

4.2.1	General

The 5GS System architecture for Ambient IoT includes core network functions, AIoT Readers and AIoT Devices. The AIoT architecture supports different AIoT Reader architectures, which allow different deployment options with different AIoT Readers to be used by the AIOTF, for the following cases:

-	AIoT RAN, which includes either a direct path between AIoT RAN and the AIOTF or an indirect path between AIoT RAN and the AIOTF via an AMF.

The architecture for Network Exposure Function, using reference point representation, defined in TS 23.501 [x] clause 4.2.3 is applicable for Ambient IoT, with a southbound interface from the NEF to AIOTF.

4.2.2	Architecture for AIoT RAN Readers

4.2.2.1	General

An AIoT RAN may support one or more AIoT Readers which provide communication services towards AIoT Devices. 5GS system architecture for Ambient IoT supports one of the following paths to access an AIoT RAN:

-	Direct Path: AIOTF communicates with AIoT RAN via a direct interface.

-	Indirect Path via AMF: AIoT RAN and the AIOTF communicate indirectly via an AMF.

Figure 4.2.2.1-1 depicts the complete non-roaming architecture showing the overall architecture for AIoT RAN readers, including both the Indirect Path and Direct Path.

Figure 4.2.2.1-1: Non-roaming AIoT System Architecture (AIoT RAN Readers)

Figure 4.2.2.1-2 depicts the complete non-roaming AIoT architecture, using the reference point representation, showing the overall architecture for AIoT RAN readers, including both the Indirect Path and Direct Path.

Figure 4.2.2.1-2: Non-roaming AIoT System Architecture (AIoT RAN Readers) in reference point representation

NOTE 1:	For the sake of clarity of the point-to-point diagrams, the AF and NRF have not been depicted. However, all depicted Network Functions can interact with the UDR, NEF and NRF as necessary.

NOTE 2:	The ADM uses AIoT Device profile data and AF authentication data that may be stored in UDR (refer to clause 4.5.8).

NOTE 3:	For clarity, the UDR and its connections with other NFs, e.g. ADM, are not depicted in the point-to-point and service-based architecture diagrams. For more information on the ADM data storage architectures refer to clause 4.5.8.

The architectures in the following clauses showing parts of the overall AIoT architecture specific to each path between the AIOTF and AIoT RAN:

-	Direct Path: In the Direct Path architecture the AIOTF uses Nx to access AIoT RAN, and is described in clause 4.2.2.2.

-	Indirect Path via AMF: In the Indirect Path architecture the AIOTF uses an AMF which uses N2 to access AIoT RAN, and is described in clause 4.2.2.3.

NOTE x: A deployment that only uses, e.g., the Direct Path only does not need to deploy the NFs, reference points and service-based interfaces associated with the Indirect Path, and vice-versa.

4.2.2.2	Direct Path interface between AIOTF and AIOT RAN

In the Direct Path architecture, the AIOTF uses Nx to communicate directly with AIoT RAN.

Figure 4.2.2.2-1 depicts the AIoT architecture, using the service-based interfaces, showing only the parts of the AIoT architecture for an AIOTF accessing AIoT RAN via the Direct Path. The remaining parts of the AIoT architecture shown in Figure 4.2.2.1-1 remain unchanged.

Figure 4.2.2.2-1: AIoT RAN – AIOT Direct Path Architecture

Figure 4.2.2.2-2 depicts the AIoT architecture, using the reference point representation, showing only the parts of the AIoT architecture for an AIOTF accessing AIoT RAN via the Direct Path. The remaining parts of the AIoT architecture shown in Figure 4.2.2.1-2 remain unchanged.

Figure 4.2.2.2-2: AIoT RAN – AIOT Direct Path Architecture in reference point representation

4.2.2.3	Indirect Path interface via AMF between AIOTF and AIOT RAN

In the Indirect Path, the AIOTF uses an AMF via the Namf service-based interface/Na reference which accesses AIoT RAN via N2.

Figure 4.2.2.3-1 depicts the AIoT architecture, using the service-based interfaces showing only the parts of the AIoT architecture for an AIOTF accessing AIoT RAN via the Indirect Path. The remaining parts of the AIoT Architecture shown in Figure 4.2.2.1-1 remain unchanged.

Figure 4.2.2.3-1: AIoT RAN – AIOT Indirect Path Architecture

Figure 4.2.2.3-2 depicts the AIoT architecture, using the reference point representation showing only the parts of the AIoT architecture for an AIOTF accessing AIoT RAN via the Indirect Path. The remaining parts of the AIoT architecture shown in Figure 4.2.2.1-2 remain unchanged.

Figure 4.2.2.3-2: AIoT RAN – AIOT Indirect Path Architecture in reference point representation

4.3	Reference points

The AIoT Architecture contains the following reference points:

Ny:	Reference point between the AIoT Device and the AIOTF.

Nx:	Reference point between the AIoT RAN and the AIOTF.

The following reference points show the interactions that exist between the NF services in the NFs. These reference points are realized by corresponding NF service-based interfaces and by specifying the identified consumer and producer NF service as well as their interaction in order to realize a particular system procedure.

Na:	Reference point between the AIOTF and the AMF.

Nb:	Reference point between the AIOTF and the NEF.

Nc:	Reference point between the AIOTF and the NRF.

Nd:	Reference point between the AIOTF and the ADM.

Ne:	Reference point between the ADM and the UDR.

Nf:	Reference point between the ADM and the NEF.

In addition to the relevant reference points defined in TS 23.501 [3], in the case of Ambient IoT, these reference points are as follows:

N2:	Reference point between the AIoT RAN and the AMF.

N33:	Reference point between NEF and AF.

4.4	Service-based interfaces

Naiotf:	Service-based interface exhibited by the AIOTF.

Nadm:	Service-based interface exhibited by the ADM.

In addition to the relevant services defined in TS 23.501 [3] for the following service-based interfaces, in the case of Ambient IoT, the services can be provided by corresponding NF are as follows:

Namf:	Service-based interface exhibited by AMF.

Nnef:	Service-based interface exhibited by NEF.

Naf:	Service-based interface exhibited by AF.

Nnrf:	Service-based interface exhibited by NRF.

4.5	Functional Entities

4.5.1	AIoT Device

An AIoT Device can store a range of information and supports the following functions:

-	Inventory procedures, including reporting its identity,

-	Command procedures, to read and write application data to and from the device, and disable functionality,

-	AIoT NAS protocol, used between the AIoT Device and the AIOTF.

4.5.2	AIoT Readers

4.5.2.1	General

An AIoT Reader is functionality that provides communication services towards AIoT Devices, in response to requests from the network.

The following AIoT Readers are supported:

-	AIoT RAN, as described in clause 4.5.2.2.

4.5.2.2	AIoT RAN

AIoT RAN supports NGAP to either AIOTF over Nx or to an AMF over N2, supporting the functionality as defined in TS 38.300.

4.5.3	AIOTF

The AIOTF supports the following functions:

-	Termination of the AIoT NAS protocol with the AIoT Device(s).

-	Receives and authorises Ambient IoT service requests, and triggers AIoT Readers to perform operations towards AIoT Devices.

-	AIoT RAN or AIoT RAN Reader selection for AIoT service operations.

-	Provide assistant information to AIoT Readers to enable them to fulfil requests.

-	Retrieving AIoT Device profile data from the ADM.

- 	AIoT Device context management.

-	Determining the aggregation control information based on operator policy and the requirements provided by the AF.

4.5.4	NEF

In addition to the functions defined in TS 23.501 [x], the NEF performs the following functions:

-	Providing an interface to AFs for AIoT services.

-	Interacting with AF, ADM and AIOTF.

-	Selection of AIOTF for AIoT services including Inventory, Command.

4.5.5	AF

The AF performs the following functions to support AIoT services:

-	Interacting with NEF/AIOTF for AIoT related service exposure.

-	Providing an aggregation indication along with specific requirements for the aggregation.

AFs and NFs may access AIoT services from an AIOTF in the same trust domain (e.g. in the same PLMN) using the Naiotf interface. External AFs may access AIoT services from an NEF using Nnef interface.

4.5.6	NRF

In addition to the functions defined in TS 23.501 [x], the NRF performs the following functions:

-	Support of new NF types AIOTF and their corresponding NF profiles.

-	Support of AIOTF discovery based on parameters such as target area information.

-	Support of AMF discovery based on parameters such as target area information.

4.5.7	AMF

In addition to the functions defined in TS 23.501 [x], the AMF performs the following functions:

-	The AMF contains functionality responsible for routing messages to and from AIoT RAN and AIOTF.

-	The AMF is accessible to the AIOTF via the Namf interface and accesses AIoT RAN via the N2 reference point.

-	Provide transport for an AIoT RAN Node’s serving area information between AIoT RAN and AIOTF.

If the AMF is deployed to only support AIoT, the other functionally defined in TS 23.501 [x] is optional to support.

4.5.8	ADM

The AIoT Data Management, ADM, is a NF that contains AIoT Device information and authorisation data for external AFs to validate the external AFs request for AIoT services. The ADM is accessible from an AIOTF or NEF via the Nadm interface.

As depicted in Figure 5.5.8-1, the 5GS System architecture for AIoT allows the ADM, in addition to NFs as described in TS 23.501 [x] clause 4.2.5, to store data in the UDR and has the following additional function:

-	UDR accessed by the ADM belongs to the same PLMN where the ADM is located.

Figure 4.5.8-1: ADM Data Storage Architecture

NOTE 1:	A separate instance of the UDR can be deployed, storing AIoT Device profile data and AF authorisation data, and providing service to the ADM, as described in TS 23.501 [x] clause 4.2.5.

4.6	Protocol Stacks

4.6.1	General

This clause specifies the protocol stacks between entities used for AIoT. The protocol stacks include the following:

-	Protocol stacks between AIoT Device and AFs.

4.6.2	Protocol Stack between AIoT Device and AF

4.6.2.1	General

The general protocol stack between an AIoT Device, AIoT RAN, AIOTF and AF is shown in Figure 4.6.2.1-1.

Legend:

-	AIoT NAS: The NAS protocol between AIoT Device and AIOTF.

-	AIoT AS: It is between the AIoT Device and the AIoT RAN is specified in TS 38.300 [5].

-	AIoT Data: It is the application data exchanged between the AIoT Device and AF.

Figure 4.6.1-1: Protocol Stack Between AIoT Device and AF

The AIoT RAN may communicate to AIOTF via different paths, see clause 4.2.2.1. The AIoT RAN protocol stack remains the same and is agnostic to the difference paths. The protocol stacks and routing of information between an AIoT RAN and the AIOTF is defined in other clauses depending upon the architecture used:

-	Direct Path: When the AIoT RAN communicates with AIOTF directly, the protocol stack is specified in clause 4.6.2.2.

-	Indirect Path: When the AIoT RAN communicates with AIOTF indirectly via an AMF, the protocol stack is specified in clause 4.6.2.3.

4.6.2.2	Protocol Stack between AF and AIoT Device for AIoT RAN Direct Path

Legend:

-	AIoT Reader Control: It is between the AIOTF and AIoT RAN.

Figure 4.6.2.2-1: Protocol Stack between AF and AIoT Device for Direct Path option

4.6.2.3	Protocol Stack between AF and AIoT Device for AIoT RAN Indirect Path

Figure 4.6.3.1-2: Protocol Stack between AF and AIoT Device for Indirect Path via AMF option

In this Protocol Stack, AIoT NAS messages are transferred via the AMF transparently.

Editor's note:	Whether AIoT Reader Control is transported by NGAP or is part of the NGAP protocol will be updated based on RAN WG3 decision.

Editor's note:	The reference to AIoT AS Layer protocol will be updated based on RAN WG1 or RAN WG2 decision.

# * * * * Third change * * * *

5.7.2	AIoT Device Permanent Identifier

# * * * * End of changes * * * *