3GPP TSG-SA2 Meeting #167	S2-2502081

17-21 Feb 2025, Athens, Greece

Source:	Xiaomi

Title:	Discussion paper for Rel-20 5G-A ISAC

Document for:	Information

Agenda Item:	30.8

Work Item / Release:	5G-A/Rel-20

Abstract of the contribution: this paper introduces Xiaomi’s view and way forward for Rel-20 5G-A ISAC for the potential controversial issues in the SID, also by considering the feedbacks from ISAC NWM discussion;

1 Introduction

Following WTs have been discussed during ISAC NWM discussion, which is in the endorsed SID (SP-231754):

- WT-0: Architecture and function enhancements to support sensing.

- WT-1: Service authorization and revocation. This includes investigating mechanisms for authorization and revocation sensing service requests from the service consumer (e.g. 3rd party application, 5GC NF).

- WT-2: Discovery and selection of sensing entities (e.g. gNB, 5GC NF) based on service requirements triggered by the service request and capability of the sensing entities.

- WT-3: Sensing data and the associated information collection and transport mechanisms for result

calculation.

- WT-4: Service invocation, including providing sensing associated information, and result (including sensing result and contextual information) exposure mechanisms, for one time, periodic or event based reporting.

- WT-5: Configuration parameters/policy provisioning to gNB for the support of ISAC services.

NOTE 1: Privacy protection and other security aspects will be tasked to SA3, and the related impact to architecture enhancement will be based on SA3 conclusion.

NOTE 2: Charging aspects will be tasked to SA5, and the related impact to architecture enhancement will be based on SA5 conclusion.

NOTE 3: Implications to RAN or RAN dependent aspects will be coordinated with RAN WGs.

2 Xiaomi views and way forwards on following controversial issues

- gNB based sensing, or UE based sensing, or both be in the scope of Rel-20 5G-A ISAC?

Based on the endorsed RAN Chair’s summary in last RAN#106 (RP-243292), although this is not the final agreement, it seems no RAN plan to include UE based sensing, and also the majority SA2 companies prefer to study gNB based sensing in Rel-20 5G-A ISAC. So Xiaomi believes it is a good and feasible option to study gNB based sensing in Rel-20.

Meantime by considering the RAN status and the limited time for Rel-20 5G-A, Xiaomi thinks it is necessary to further reduce the scope to gNB monostatic sensing mode, so that SA2 can have sufficient time to complete the Rel-20 5G-A ISAC study and normative work. But the final decision needs further coordination with RAN.

Proposal 1: gNB monostatic sensing mode only in scope of Rel-20 5G-A ISAC. Further coordination with RAN is needed to decide the final scope.

- TU estimation for study and normative work

Based on the suggestion and the further reduced scope (i.e. gNB monostatic sensing mode) above, Xiaomi believes it is feasible to complete both study and normative work of Rel-20 5G-A ISAC within 14 TUs (7+7 TU for study and normative work)

- For WT0, architecture aspect, 2TUs is used to design the ISAC architecture for gNB monostatic sensing mode, and also considering other sensing modes.

- For WT1, WT2, WT3, WT4, during TR study, these solutions are probably covering multiple WTs among WT1/2/3/4. So Xiaomi thinks total 4 TUs are sufficient for WT1/2/3/4. Suggest 1TU for each WT.

- For WT5, 1 TU is also sufficient.

Surely, further alignment with RAN decision is needed.

Proposal 2: Total 14 TUs for study work (7TUs) and normative work (7 TUs). Further coordination with RAN is needed to adjust the final TUs allocation.

- Relationship between 5G-A ISAC and 6G ISAC

For Xiaomi’s view, the duplicated work between 5G-A ISAC and 6G ISAC should be also avoided. From business deployment point of view, it is feasible option to finish the study gNB monostatic sensing mode within Rel-20 5G-A.

For 6G ISAC, we think during the 6G architecture design, it may natively support all 6 sensing modes.

Proposal 3: Duplicated work between 5G-A ISAC and 6G ISAC should be avoided. For 5G-A, we propose to reuse existing architecture as much as possible to minimize the impact to 5GC. For 6G, we may consider the ISAC native architecture to support all the sensing modes.

3, Proposal

Based on the analysis above, Xiaomi would like to provide following proposals as way forward to be considered during Rel-20 5G-A ISAC SID discussion:

Proposal 1: gNB monostatic sensing mode only in scope of Rel-20 5G-A ISAC. Further coordination with RAN is needed to decide the final scope.

Proposal 2: Total 14 TUs for study work (7TUs) and normative work (7 TUs). Further coordination with RAN is needed to adjust the final TUs allocation.

Proposal 3: Duplicated ISAC work between 5G-A and 6G should be avoided. For 5G-A, we propose to reuse existing architecture as much as possible to minimize the impact to 5GC. For 6G, we may consider the ISAC native architecture to support all the sensing modes.