3GPP TSG-WG SA2 Meeting #167	S2-2502083

Athens, GR, 17th February – 21th February, 2025	(revision of S2-250xxxx)

Source:	NTT DOCOMO, Deutsche Telekom, Philips International B.V., Ericsson, KPN N.V.

Title:	Discussion on assistance information to support command procedure

Document for:	Discussion

Agenda Item:	19.14.2

Work Item / Release:	Architecture support of Ambient power-enabled Internet of Things (AmbientIoT) / Rel-19

Abstract: This paper discusses the need for assistance data to support command procedure

# 1. Introduction

This paper discusses a potential issue when there is a time delay between the AIOT NAS Inventory response and the delivery of a AIOT NAS Command request.

# 2. Discussion

## 2.1	Ambient IoT service operation

Figure 2.1-1 Ambient IoT service operation

### 2.1.1	AIOTF AIOT RAN during inventory

Figure 2.1-1 illustrates an Application Function requesting a Command service operation. In order to perform the command, the AIOTF first triggers an inventory request to find the candidate Ambient IoT device.

The AIOTF forwards the inventory request to the AIOT RAN that executes an Ambient IoT paging to trigger candidate Ambient IoT device(s) to initiate Random access.

In step 5, the candidate Ambient IoT device provides an Inventory response with the Ambient IoT device ID to the AIOT RAN, AIOT RAN forwards the Inventory response to the AIOTF.

Observation 1: If the inventory has a subsequent command, the AIOT RAN should have some kind of NGAP context. If it is only an inventory without subsequent command, the AIOT RAN should not create some kind of NGAP context.

Proposal 1: The AIOTF indicates to AIOT RAN, as part of assistance data, whether or not subsequent communication is expected so that the AIOT RAN can avoid creating unnecessary NGAP kind contexts for Ambient IoT devices only performing an inventory request. The AIOT RAN can also know there is a use for an NGAP like context, easing the delivery of the subsequent command.

### 2.1.2 AIOTF AIOT RAN during command

Figure 2.1.2-1 Ambient IoT command operation

Figure 2.1.2-1 illustrates an example of the call flow for performing a command, first inventory is performed, then a command is sent from the core to the AIOT RAN.

Depending on delay between Inventory response in Step 4 and the time to deliver the AIOT NAS Command request in Step 5 to the AIOT RAN, the connection to the Ambient IoT device may or may not be available. In a more favourable scenario, the connection is still there and the command can be easily delivered. The less favourable option would be a requirement to perform another paging/inventory to establish a connection with the device then deliver the command. The connection may be closed by either the Ambient IoT device or the AIOT RAN after the inventory. Meaning each command operation may require two inventory requests.

Observation 2: It would be useful for both the AIOT RAN and Ambient IoT device to be aware there is a command pending after an inventory request to keep the connection established until the command has been delivered to avoid duplicate inventory procedures.

Observation 3: It is unclear if Step 6 can be executed in Alternative 2 as the AIOTF has not provided any inventory request, the AIOT RAN has no identifier to target.

# 3. Conclusion and proposal(s)

Based on the observations in clause 2, it is proposed to include information in the paging message that indicates an inventory is to be followed by a subsequent command and in the signalling between AIOTF and AIOT RAN include information indicating there is a subsequent command pending after the inventory as proposed in S2-2502084.