3GPP TSG-SA WG2#167	S2-2502107

Athens, Greece, 17-21 February 2025	(was S2-250xxxx)

Source:	Qualcomm Incorporated

Title:	KI#2: Clarifications for Ambient IoT Device ID

Document for:	Discussion/Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT

Abstract of the contribution: The contribution adds clarifications for the Ambient IoT Device ID.

# 1.	Text proposal

It is proposed to agree the following changes to TR 23.700-13:

# >>>>BEGINNING OF CHANGES<<<<

### 8.2.1	Identifier and Identification Management

The following aspects and principles are considered and agreed for the interim conclusion on Identifier and Identification Management.

The following principles are agreed for Identifier and Identification Management:

-	An Ambient IoT Device is configured with a permanent Ambient IoT Device Identifier which can be assigned by an operator or by a third party. The Identifier is used to identify Ambient IoT Device and locate the corresponding authentication server.

NOTE 1:	How to configure Ambient IoT Device with the permanent Ambient IoT Device Identifier is out of 3GPP scope.

The permanent Ambient IoT Device Identifier includes the following information:

1)	Part1information:

-	The ID type, including

-	information indicating whether the network identifier is included or not.

-	information indicating whether information used to identify a 3rd party is included or not.

-	the Part2 type indicating EPC or unstructured Part2 information.

-	A network identifier (i.e. MCC+MNC and/or NID), when the ID includes the network identifier.

-	Information used to identify a 3rd party when the ID includes the information used to identify a 3rd party.

2)	Part2 information:

-	The variable-length information (EPC as defined in clause 14 of [10] or  unstructured information (different from EPC)) to distinguish different Ambient IoT Devices within the scope identified by the Part1 information.

NOTE 3:	The format of the unstructured information is beyond the scope of 3GPP. The coding for the above information is left to stage 3.

Figure 8.2.1-1: The example of Operator allocated ID

For operator allocated Part 1 information, the network identifier is mandatory and can be used to index the authentication server or not. If it is not used to index the authentication server, the ID may further include the information used to identify a 3rd party. The third party may be the credential holder or not. If it is not the credential holder, the network should be provided with third party related context including the information used to locate the authentication server.

Figure 8.2.1-2: The example third party allocated ID

For third party allocated Part 1 information, the network identifier is not needed. The third party may be the credential holder or not. If it is not the credential holder, the network should be provided with third party related context including the information used to locate the authentication server.

NOTE 4:	Within each type of ID, the length of Part 1 is fixed (i.e. the length of ID type, the Network Identifier (if present) and the Information used to identify a third party (if present) are fixed) and the length of Part2 is dynamic. The detail is left to stage 3.

With the above information, the AIoT device ID is globally unique.

Editor's note:	Whether the temporary ID in the AIoT NAS layer is required for the privacy protection is FFS and is pending SA WG3 decision.

# >>>>END OF CHANGES<<<<