3GPP TSG-SA2 Meeting #167	S2-2502131

# Athens, Greece, 17th-21st Feb 2025	                    (Rev. S2-250xxxx)

Source:	Google

Title:	KI#4-Way Forward Proposal for identification of encrypted multiplexed media flows

Document for:	Discussion

Agenda Item:	19.3.2

Work Item / Release:	XRM_Ph2 / Rel-19

Abstract: This paper provides way forward proposal for identification of encrypted multiplexed media flows

# Discussion

There were intensive discussions on whether identification of encrypted traffic with multiplexed media flows can be supported in this release.

The concept of multiplexing datagrams and using identifiers for differentiation purposes has been described in IETF RFC 9221. For KI#2 and KI#4, several solutions in the TR 23.700-70 addressed the scenario and solutions pointing to the similar approach to use metadata (media related information) with an identifier over N6 as described in IETF RFC 9221, e.g. Track ID (as in Solution#9), stream info (as in Solution #14) Connection or Stream ID (as in Solution #15), QSC-ID (as in Solution #27), sub-flow ID (as in Solution #29) or Obfuscated codes (as in Solution #12).

Accordingly, based on KI#4 conclusion, the additional Packet Filter and QoS requirements for media flows are expected to be added in normative work for supporting multiplexed media flows in the "AF session with required QoS" procedure from the AF to the NEF/PCF.

Observation#1: the solutions to resolve encrypted multiplexed media flows have been intensively discussed in both KI#2 and KI#4. These solutions proposed to use metadata (media related information) with an identifier over N6, which is in line with IETF RFC 9221 that describes concept of multiplexing datagrams and using identifiers for differentiation purposes. In addition, the corresponding enhancement with additional Packet filter and QoS requirements for AF session with required QoS procedure are aligned with TR conclusion for KI#4.

Furthermore, there was a discussion in S2-2500409r1 about using the MPQUIC framework for identification of multiplexed media flows. A major concern raised was the congestion control in Subflow identifier-based approaches using a single transport layer flow (IP 5-tuple) for QUIC connections. Although sufficient information about the MPQUIC approach is currently lacking in the TR, based on IETF draft MPQUIC extension, it presents a potential enhancement for future releases. In the followings, we provide our views on two approaches: using the MPQUIC framework and Subflow identifier-based approaches for identification of multiplexing media flows:

|  | Sub-Flow Identifier-Based Approaches | MP-QUIC Framework (Multiple 5-Tuples) |  |
| --- | --- | --- | --- |
| QoS Differentiation | Achieved via sub-flow IDs embedded in metadata, enabling mapping to QoS flows without requiring separate 5-tuples. | Achieved by assigning each media stream to a separate IP 5-tuple, allowing network-level QoS mapping. | Both approaches achieve QoS differentiation effectively. MP-QUIC uses multiple paths, while sub-flow IDs rely on metadata within a single 5-tuple. |
| Congestion Control | Shared congestion control across sub-flows within the same transport connection may lead to bandwidth competition among sub-flows, potentially leading to unfair distribution of resources, which may not adapt well to varying conditions on individual paths. | Per-path congestion control ensures independent handling of different media streams. However, managing separate congestion control states for each path requires more resources and complex logic. | MP-QUIC's per-path congestion control is better for isolating streams but adds complexity. Sub-flow IDs face challenges in managing QoS conflicts under shared congestion control. |
| Scalability | Scales better: No need for additional paths or 5-tuples; relies on media related information (metadata) for differentiation. | Limited: Managing multiple 5-tuples for many media streams can increase signalling overhead and resource usage. | Sub-flow IDs scale better as they avoid the overhead of managing multiple paths and 5-tuples, making them more suitable for XR applications with many streams. |
| Implementation Complexity | Moderate: Requires UPF to parse sub-flow IDs from metadata but avoids multipath complexities. | High: Requires MP-QUIC support, multiple 5-tuples, and enhanced UPF functionality for path management. High implementation complexity due to multipath management and path validation. | MP-QUIC introduces higher complexity due to multipath management. Sub-flow IDs are simpler but require enhanced UPF functionality for supporting media related information over N6. |
| Load Balancer Compatibility | Compatible: Single connection simplifies load balancing as all traffic is routed to the same server instance. | Compatible: Multiple 5-tuples are steered to the same server instance using QUIC-LB. | Both approaches are compatible with load balancers, but sub-flow IDs simplify routing further by maintaining a single connection context. |
| Network Dependency | Relies on UPF's ability to identify sub-flow IDs from media related information over N6 but avoids dependency on multiple 5-tuple management. Easier integration into existing protocols and network infrastructure. | Requires network support for managing and differentiating multiple 5-tuples. | Sub-flow IDs reduce network dependency by avoiding reliance on multiple paths or 5-tuple management, making them easier to deploy in existing networks without significant changes. |
| Flexibility for Applications | High: Applications only need to include sub-flow IDs in metadata; simpler integration with existing protocols. | Moderate: Applications must manage path creation and assignment of streams to specific 5-tuples. | Sub-flow IDs are more flexible and easier for application developers compared to MP-QUIC paths, which require explicit path management logic at the application layer. |
| Standardization Alignment | Aligned with IETF standards RFC 9221 by using identifier for multiplexed traffic purposes, and metadata-based approaches for encrypted traffic differentiation as descripted in clause 5.37.9.1 of TS23.501. | Aligned with IETF MP-QUIC standards but requires enhancements for multipath proxying in the UPF. | Both approaches align with IETF standards, but sub-flow IDs rely on simpler mechanisms and integrated well in clause 5.37.9.1 of TS23.501 as compared to MP-QUIC's more complex multipath extensions. |
| Potential Enhancements for R20 XRM_Ph3 study | Enabling more granular control over the transport network resources allocated for each sub-flow, e.g. ﻿different transport level DSCP marking values can be used for a sub-flow based on priority of each sub-flow provided by application. | For encrypted multiplexed media flows handling, it needs to investigate how to proper integrate MP-QUIC proxy functionality in UPF for both UL and DL direction, enable UE support for MPQUIC, and integrate support of On-Path N6 Connection. |  |

Observation#2: For simplicity and scalability, the sub-flow identifier-based approaches is generally better and well fit into the concluded features of clause 5.37.9 for supporting media related information over N6 and features of clause 5.37.11 for Traffic identification and differentiated QoS for multiplexed media flows.

Observation#3: The MP-QUIC framework could be a strong candidate to support encrypted multiplexed media flows in both UL and DL for 5G-A XRM_Ph3 study.

Proposal#1: It is proposed to proceed with normative work in R19 based on Subflow identifier based approach to identify Multiplexed media flow for end-to-end encrypted traffic based on the support of media related information over N6 defined in clause 5.37.9.1 of TS23.501.

- The format of the Subflow identifier is left for stage 3.

- In this release, only DL direction is supported for identification of encrypted multiplexed media flows.

Proposal#2: for XRM_Ph3 study, it is proposed to continue enhancement of identification for encrypted multiplexed media flows for both UL and DL.