3GPP TSG-WG SA2 Meeting #167	S2-2502143

Athens, Greece, 17th – 21st Feb, 2025	(revision of S2-25xxxxx)

Title:	Reply LS to "OSAppID usage by AppToken use case"

Response to:	LS from GSMA TSGNS: OSAppID usage by AppToken use case (S2-2500077)

Release:	Rel-19

Work Item:	TEI19

Source:	SA2

To:	GSMA TSGNS

Cc:	SA3

Contact Person:

Name:	Krisztian Kiss

E-mail:	krisztian@apple.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

SA2 would like to thank GSMA TSGNS for the incoming LS. TSGNS asked SA2 to confirm the feasibility of the mechanism described in the LS. Additionally, SA2 can provide the following answers to the questions raised by GSMA:

- Do the existing 3GPP specifications allow the network to use application identities that are generated by an application backend to populate URSP in step 2?

Answers from SA2:

Based on the OSAppId definition in TS 23.503 (“An identifier associated with a given application and uniquely identifying the application within the UE for a given operating system”) it is unclear how the application backend could guarantee the uniqueness of the identifier within the UE for the given operating system without e.g. standardizing the format of the identifier or making sure the application backend has access to some kind of centralized database on OSAppIds.

SA2 believes such uniqueness can only be guaranteed if values for the OSAppId are reserved and managed by the operating system of the UE.

- Would 3GPP SA2 prefer an alternate mechanism to preserve end user privacy?

Answers from SA2:

It is unclear to SA2 how end user privacy is achieved by the proposed mechanism. The usage of App Token achieves anonymization of the OSAppId, however such anonymization looks redundant. Since the Operator Entitlement Server belongs to the MNO and is aware of the mapping between the App token and the OSAppId, the MNO has clear visibility on the applications installed on the UE, hence no user privacy is guaranteed.

Considering the MNO also learns information about when the user launches the application on the UE, end user privacy is considered even degraded compared to when no App Token mechanism is used.

- Consider the architectural implications of the proposed mechanism and provide feedback.

Answers from SA2:

SA2 thinks the described mechanism creates additional signaling without achieving user privacy, hence, the described flow is not feasible in its current state.

SA2 would also like to point out that the proposed mechanism is dependent on third party applications being able to pass the App Token to the OS in the UE which adds additional burden for third party application developers.

2. Actions:

To GSMA TSGNS group.

ACTION: 	SA2 kindly asks GSMA TSGNS to take SA2’s responses into account.

3. Date of Next SA2 Meetings:

SA WG2#168	07 – 11, April 2025	Goteborg, Sweden

SA WG2#169	19 – 23, May 2025	Fukuoka, Japan