3GPP TSG-WG SA2 Meeting #167	S2-2502321

Athens, Greece, February 17 - 21, 2025	(revision of S2-2501586)

Source:	Ericsson

Title:	KI#2 Conclusion Update of Subscription Data for AF Authorization

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract: This pCR discusses and updates KI2 conclusions for subscription data for AF authorization.

1. Introduction

In KI#2, regarding the AF authorization, the conclusions are reached as below:

Subscription data for 3rd party AF is required:

-	It is used by the AIOTF together with SLA and operator policy at NEF to perform authorization of the AF request targeting for Ambient IoT service. This includes: 1) checking whether the AF is authorized for the requested service operation; 2) checking whether the AF is authorized for the requested target area; 3) checking whether the AF is authorized for the requested target AIoT device(s).

-	it is stored at UDM within 5G network.

NOTE 5:	Items within the subscription data for 3rd party AF will be determined during normative phase.

First of all, UDM stores subscription data for regular UEs identified by SUPI. This can include information for the authorization of AF requests for each individual UE/SUPI. It has been concluded that ADM will be the entity responsible of managing subscription data related to AIoT devices. AIoT devices use an identifier other than SUPI. Therefore, any subscription data related to a given AIoT device, including information for the authorization of AF requests for the individual AIoT device, shall be managed by the ADM and not by the UDM.

It is understood that authorization for 1) and 2) above is executed at the NEF as the prime contact point with the AF. It is also understood that the NEF does not require subscription data of the individual AIoT device to execute this level of authorization and thus related information is not required to be part of the subscription data for AIoT devices stored in ADM/UDR. NEF can execute authorization 1) and 2) based on SLAs and operator policy locally configured in NEF.

Thus, the only authorization info for 3rd party AF requests that makes sense to be considered within the subscription data of AIoT devices stored in ADM/UDR is the one related to 3) above. It is assumed that the NEF fetches this subscription data for authorizing 3rd party AF requests to a target AIoT device.

Therefore, NEF can execute authorization of 3rd party AF requests as in 1), 2) and 3) above. This is, AIOTF should need not to be involved in the authorization of 3rd party AF requests.

[Proposal-1] Update the conclusion for the subscription data for AF authorization as described above.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-13 v1.3.0:

* * * Start of Change * * * *

### 8.2.2	Subscription Management

The AIoT device may or may not have operator’s subscription data in the network. If the AIoT device doesn’t have the operator’s subscription data in the network, the corresponding related data (e.g. Device ID or credentials) is stored in the AAA server external to the network.

Subscription data for an Ambient IoT Device is required in case the Ambient IoT Device is managed by the network:

-	It is used by the AIOTF to check whether the device is subscribed.

NOTE 1:	Checking whether the device is subscribed needs coordination with SA WG3 study outcomes on validation of the ambient IoT Device identifier.

-	It is dedicated subscription data for Ambient IoT Devices, i.e. different with UE subscription data.

-	It contains at least a permanent Ambient IoT Device ID, etc.

NOTE 2:	SA WG2 will align security related materials in subscription data with SA WG3 during the normative phase.

The Ambient IoT Device ID is used by the AIOTF together with local configuration, 3rd party related context to locate the entity storing the AIoT device profile data of an Ambient IoT Device. The AIoT device profile data includes the AIoT Device ID. When the AIoT device profile data is managed by the 5GC, it is stored in the ADM (Ambient IoT Data Management), possibly together with UDR, which exclusively supports AIoT devices.

NOTE 3:	Whether the AIoT device profile data can include credentials for AIoT device will be determined by SA WG3.

NOTE 4:	Storage of AIoT device profile data (i.e. AIoT device subscription like data) in the network does not impact the UDM.

Authorization data for 3rd party AF is required:

-	It is used by the AIOTF together with SLA and operator policy at NEF to perform authorization of the AF request targeting for Ambient IoT service. This includes: 1) checking whether the AF is authorized for the requested service operation; 2) checking whether the AF is authorized for the requested target area; 3) checking whether the AF is authorized for the requested target AIoT device(s), including individual devices and a group of devices.

-	it is stored at ADM within 5G network or configured in the NEF.

NOTE 5:	Items within the subscription data for 3rd party AF will be determined during normative phase.

NOTE 6:	Based on operator policy and local configuration, the NEF performs AIoT specific AF authorization. The AIOTF performs AIoT specific AF authorization based on the authorization data stored in the ADM.

UE Subscription data is enhanced to include an indication that whether the UE is allowed to operate as a UE reader.

* * * End of Change * * * *