3GPP TSG-WG SA2 Meeting #167	S2-2502324

Athens, GR, 17 – 21 Feb, 2025	Revision of S2-2501706

Source:	China Mobile, Spreadtrum, ZTE, CATT

Title:	pCR to 23.xyz: Reader selection for Ambient IoT service request

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT/ Rel-19

Abstract: This paper proposes the BS reader selection methods.

# 1. Introduction/Discussion

This contribution is proposed to add reader selection functionality in 5.3 in TS 23.xyz.

# 2. Text Proposal

It is proposed to approve the following changes.

# * * * * FIRST CHANGE* * * *

# High level functionality and features

## 5.3	Reader selection

The AF may provide NEF with the expected target area information in the Ambient IoT service request. NEF can convert the expected target area information to internal location information. NEF can query NRF to obtain serving AIOTF(s) based on internal location information. Then, NEF forwards the Ambient IoT service request, including the internal location information, to the AIOTF.

Editor’s note: How the NEF converts target area information to internal location information and the format of the internal location information is FFS.

The AIOTF may have BS reader information from AIoT RAN by the NGAP or OAM configuration.

Editor’s note: BS reader information needs to be coordinated with the RAN WG(s). Details are pending RAN WG feedback.

Editor’s note: The AIOTF configuration of BS reader information over NGAP needs to coordinate with RAN WG(s).

NOTE 1: The BS reader information is not exposed to the AF.

The AIOTF selects AIOT RAN node(s) and, optionally, a list of the BS readers. The AIOTF selects the AIoT RAN node(s) based on the internal location information.

Editor’s note: How the AIOTF chooses the BS reader(s) is FFS.

If the AIOTF doesn’t include a BS reader list in the Ambient IoT service request to the AIOT RAN, then the AIOT RAN needs to use all available BS readers.

If the Ambient IoT service is for one or multiple specific ambient IoT devices, the AIOTF may consider the last known serving BS reader(s) as targeted BS reader(s) directly.

# * * * * END OF CHANGE* * * *