3GPP TSG-WG SA2 Meeting #167	S2-2502339

17th – 21st February 2025, Athens, Greece	(revision of 25XXXXX)

Source:	Lenovo

Title:	pCR to TS 23.xyz: AF authorization to the Ambient IoT Services

Document for:	Approval

Agenda Item: 	19.14.2

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: This paper proposes to add the support of AF Authorization to Ambient IoT Services for 3rd party AF.

1	Discussion

This paper aims to capture the support of AF authorization to the Ambient IoT services, based on the latest agreed conclusion principles in the TR 23.700-13.

2. Text Proposal

It is proposed to capture the following changes to TS 23.xyz for Ambient IoT.

# * * * * First change * * * *

5.6	AF authorization to the Ambient IoT Services

The information/data needed to support the authorization of the AF for performing the Ambient IoT service is stored as the authorization data for 3rd party AF in the ADM within the 5GC network, or locally configured in the NEF.

Table 5.6-1 below describes items stored as AF authorization data for the AIoT service.

Table 5.6-1: AF Authorization Data for AIoT service

| AF Authorization Data | Status | Description |
| --- | --- | --- |
| AF ID | M | Identifier used to identify the AF. |
| Allowed area | M | Indicate the allowed area for the indicated AF to perform the AIoT services operations. |
| Allowed service operations | M | Indicated the allowed service operation (s) for the indicated AF, e.g., inventory, read, write, permanent disable. |
| Allowed target AIoT devices | O | Indicate the allowed AIoT Device(s) for the indicated AF. |

The authorization to the AF for the Ambient IoT services mainly includes two parts:

- NEF authorizes the Ambient IoT service requests from the AF based on the service layer agreement (SLA) between the 3rd party AF and the 5GS, the operator policy and local configuration.

- AIOTF authorizes the Ambient IoT service requests from the AF directly or via NEF, by checking and retrieving the AF authorization data stored in the ADM as described in above Table 5.6-1, using at least the AF ID.

.

# * * * * End of changes * * * *