3GPP TSG-WG SA2 Meeting #167	S2-2502370

Athens, GR, 17 – 21 Feb, 2025	Revision of S2-2502324, S2-2501706

Source:	China Mobile, Spreadtrum, ZTE, CATT, InterDigital

Title:	pCR to 23.xyz: Reader selection for Ambient IoT service request

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT/ Rel-19

Abstract: This paper proposes the BS reader selection methods.

# 1. Introduction/Discussion

This contribution is proposed to add reader selection functionality in 5.3 in TS 23.xyz.

# 2. Text Proposal

It is proposed to approve the following changes.

# * * * * FIRST CHANGE* * * *

# High level functionality and features

## 5.3	RAN Reader selection

The AIOTF selects AIOT RAN node(s) and, optionally, a list of the RAN readers. The AIOTF selects the AIoT RAN node(s) based on the internal area information.

Editor’s note: How the AIOTF chooses the RAN reader(s) is FFS.

Editor’s note: How AIOTF selects the AMF and the AIOT RAN in the indirect path is FFS.

The AF may provide the expected target area information in the Ambient IoT service request. NEF converts the target area information received from the AF to internal area information. NEF queries NRF to obtain serving AIOTF(s) based on internal area information. Then, NEF forwards the AIoT service request, including the internal area information, to the AIOTF.

Editor’s note: How the NEF converts target area information to internal area information and the format of the internal area information is FFS.

The AIOTF may have AIoT RAN information and the optional RAN reader information. For direct path, the AIOTF obtains the information from AIoT RAN by the NGAP or OAM configuration.

Editor’s note: AIoT RAN and RAN reader information needs to coordinate with the RAN WG(s). Details are pending RAN WG feedback.

Editor’s note: The AIOTF or AMF configuration of AIOT RAN and RAN reader information over NGAP needs to coordinate with RAN WG(s).

Editor’s note: It is FFS how AIOT RAN transfers and updates its information to the AIOTF in indirect path.

NOTE 1: RAN reader information is not exposed to the AF.

In case an AIoT service request includes AIoT device Identifier(s),

the AIOTF may consider the last known serving RAN reader(s) to determine targeted RAN reader(s) directly.

# * * * * END OF CHANGE* * * *